3GPP TSG-SA3 Meeting #112	S3-234064

# Goteborg, Sweden 14 - 18 August 2023												   revision of S3-23xxxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.503 | CR | draft-CR | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Detailed protection information of U2U relay discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-07-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Need a solution for the secetion between establishing PC5 security with or without network assistance |  |  |  |  |  |  |  |  |
| Summary of change: |  | A solution for the secetion between establishing PC5 security with or without network assistance |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete security specification for the UE-to-UE relay communication |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************ START OF CHANGES************

6.1.3.3	5G ProSe UE-to-UE Relay Discovery

6.1.3.3.1	General

The two sets of discovery security materials are used for UE-to-UE Relay discovery message protection. One (i.e., Direct Discovery security materials) is used for protecting direct discovery set that is an end-to-end data element between 5G ProSe End UEs and is not processed by the 5G ProSe UE-to-UE Relay. The other one (i.e., UE-to-UE Relay Discovery security materials) is used for protecting the UE-to-UE relay discovery messages. The UE-to-UE relay discovery messages include the protected direct discovery set.

Provisioning of the Direct Discovery security materials reuses the security material provisioning mechanism for Restricted 5G ProSe Direct Discovery as specified in clause 6.1.3.3.

Provisioning of the UE-to-UE Relay Discovery security materials reuses the security material provisioning mechanism for 5G ProSe UE-to-Network Relay discovery as specified in clause 6.1.3.3.

The protection is configurable based on the provisioned discovery security materials.

6.1.3.3.2	Security requirements for 5G ProSe UE-to-UE Relay Discovery

5G ProSe UE-to-UE Relay Discovery addresses the following security requirements:

-	The 5G System shall provide a means for confidentiality protection, integrity protection and replay protection of discovery messages for UE-to-UE Relay discovery.

-	The 5G System shall provide a means to mitigate trackability and linkability attacks of 5G ProSe End UEs during UE-to-UE Relay discovery procedure.

-	The 5G System shall provide a means to securely provision the security materials for UE-to-UE Relay discovery.

6.1.3.3.3	Security flows

6.1.3.3.3.1	Security procedure for 5G ProSe UE-to-UE Relay Discovery with Model A

The security procedure for 5G ProSe UE-to-UE Relay Discovery with Model A is described as follows.

Figure 6.1.3.3.3.1-1: Security procedure for 5G ProSe UE-to-UE Relay Discovery with Model A

NOTE 1: The protection of direct discovery set and Announcement message reuses the protection mechanism specified in clause 6.1.3.2.3 of the present document.

1a.	The monitoring 5G ProSe End UE and announcing 5G ProSe End UE are provisioned with the discovery security materials associated with a ProSe service based on the procedure specified in clause 6.1.3.2.2 of the present document.

1b.	The monitoring 5G ProSe End UE, announcing 5G ProSe End UE, and 5G ProSe UE-to-UE Relay are provisioned with discovery security materials associated with an RSC based on the procedure specified in clause 6.1.3.2.2 of the present document.

2.	The announcing 5G ProSe End UE shall protect the direct discovery set using the discovery security materials associated with the ProSe service as specified in clause 6.1.3.2.3 of the present document. The User Info ID of Monitoring 5G ProSe End UE shall be confidentiality protected if DUCK exists.

The 5G ProSe UE-to-UE Relay obtains the protected RSC and direct discovery set from the announcing 5G ProSe End UE in proximity (e.g., via a previous 5G ProSe UE-to-UE Relay Discovery or 5G ProSe UE-to-UE Relay Communication procedures) as specified in clause 6.3.2.4.2 of TS 23.304 [2]. When 5G ProSe UE-to-UE Relay Discovery is used to deliver the direct discovery set, the announcing 5G ProSe End UE shall include the protected direct discovery set in a discovery message that is protected using the discovery security materials associated with the RSC as specified in clause 6.1.3.2.3 of the present document.

When 5G ProSe UE-to-UE Relay Communication is used to deliver the direct discovery set, the announcing 5G ProSe End UE shall use the secure PC5 unicast link with the 5G ProSe UE-to-UE Relay to send the protected direct discovery set. The 5G ProSe UE-to-UE Relay shall store the protected direct discovery set along with its validity time.

Editor’s Note: how to ensure that valid protected direct discovery sets are announced by the U2U relay is FFS.

3.	When broadcasting the Announcement message, the 5G ProSe UE-to-UE Relay shall include the list of valid protected direct discovery sets in the Announcement message and protect the Announcement message using the discovery security materials associated with the RSC as specified in clause 6.1.3.2.3 of the present document. Then, the 5G ProSe UE-to-UE Relay sends the Announcement message.

4.	On receiving the Announcement message from the 5G ProSe UE-to-UE Relay, the monitoring 5G ProSe End UE shall decrypt and/or verify the received Announcement message using the discovery security materials associated with the RSC as specified in clause 6.1.3.2.3 of the present document. If the verification is successful, the monitoring 5G ProSe End UE shall extract the direct discovery set(s) from the Announcement message, and decrypt and/or verify the direct discovery set(s) using the discovery security materials associated with the ProSe service as specified in clause 6.1.3.2.3 of the present document.

6.1.3.3.3.2	Security procedure for 5G ProSe UE-to-UE Relay Discovery with Model B

The security procedure for 5G ProSe UE-to-UE Discovery with Model B is shown in Figure 6.1.3.3.3.2-1.

Figure 6.1.3.3.3.2-1: Security procedure for 5G ProSe UE-to-UE Relay Discovery with Model B

0.	The discoverer 5G ProSe End UE and discoveree 5G ProSe End UE are provisioned with the discovery security materials associated with a ProSe service based on the procedure defined in clause 6.1.3.2.2.2.

The discoverer 5G ProSe End UE, discoveree 5G ProSe End UE and 5G ProSe UE-to-UE Relay are provisioned with the discovery security materials associated with a RSC based on the procedures defined for UE-to-Network relay in clause 6.3.

1.	The discoverer 5G ProSe End UE shall protect a direct discovery set using the discovery security materials associated with the ProSe service as specified in clause 6.1.3.2.3. The direct discovery set shall include User Info ID of the discoverer 5G ProSe End UE and User Info ID of the discoveree 5G ProSe End UE. The User Info ID of the discoverer 5G ProSe End UE and User Info ID of the discoveree 5G ProSe End UE shall be confidentiality protected if DUCK exists. Then, the discoverer 5G ProSe End UE shall include the protected direct discovery set in the Solicitation message and protect the Solicitation message using the discovery security materials associated with the RSC as specified in clause 6.1.3.2.3. The solicitation message is sent to the 5G ProSe UE-to-UE Relay.

2.	On receiving the 5G ProSe UE-to-UE Relay Discovery Solicitation message from the discoverer 5G ProSe End UE, the 5G ProSe UE-to-UE Relay shall decrypt and/or verify the received UE-to-UE Relay Discovery Solicitation message using the discovery security materials associated with the RSC as specified in clause 6.1.3.2.3.

If the verification is successful, the 5G ProSe UE-to-UE Relay shall modify the UE-to-UE Relay Discovery Solicitation message to include User Info ID of the 5G ProSe UE-to-UE Relay.

The 5G ProSe UE-to-UE Relay Discovery Solicitation message is protected using the security materials associated with the RSC as specified in clause 6.1.3.2.3. The User Info ID of the 5G ProSe UE-to-UE Relay shall be confidentiality protected if DUCK exists.

Then, 5G ProSe UE-to-UE Relay sends the message to the discoveree 5G ProSe End UE.

3.	The discoveree 5G ProSe End UE shall decrypt and/or verify the received UE-to-UE Relay Discovery Solicitation message using the discovery security materials associated with the RSC as specified in clause 6.1.3.2.3.

If the verification is successful, the discoveree 5G ProSe End UE shall extract the protected direct discovery set from the message and decrypt and/or verify the direct discovery set using the discovery security materials associated with the ProSe service as specified in clause 6.1.3.2.3.

The discoveree 5G ProSe End UE shall protect a direct discovery set using the discovery security materials associated with the ProSe service as specified in clause 6.1.3.2.3. Then, the discoveree 5G ProSe End UE shall include the protected direct discovery set in the UE-to-UE Relay Discovery Response message and protect the UE-to-UE Relay Discovery Response message using the discovery security materials associated with the RSC as specified in clause 6.1.3.2.3. The User Info ID of the discoverer 5G ProSe End UE and User Info ID of the discoveree 5G ProSe End UE shall be confidentiality protected if DUCK exists. The discoveree 5G ProSe End UE replies to the 5G ProSe UE-to-UE Relay with the UE-to-UE Relay Discovery Response message.

4.	On receiving the UE-to-UE Relay Discovery Response message from the discoveree 5G ProSe End UE, the 5G ProSe UE-to-UE Relay shall decrypt and/or verify the received UE-to-UE Relay Discovery Response message using the discovery security materials associated with the RSC as specified in clause 6.1.3.2.3.

If the verification is successful, the 5G ProSe UE-to-UE Relay shall modify the UE-to-UE Relay Discovery Response message to include User Info ID of 5G ProSe UE-to-UE Relay.

The UE-to-UE Relay Discovery Response message is protected using the security materials associated with the RSC as specified in clause 6.1.3.2.3. The User Info ID of the discoveree 5G ProSe End UE and User Info ID of the 5G ProSe UE-to-UE Relay shall be confidentiality protected if DUCK exists. Then, 5G ProSe UE-to-UE Relay sends the UE-to-UE Relay Discovery Response message to the discoverer 5G ProSe End UE.

On receiving the UE-to-UE Relay Discovery Response message, the discoverer 5G ProSe End UE shall decrypt and/or verify the UE-to-UE Relay Discovery Response message using the discovery security materials associated with the RSC as specified in clause 6.1.3.2.3.

If the verification is successful, the discoverer 5G ProSe End UE shall extract the protected direct discovery set from the UE-to-UE Relay Discovery Response message and decrypt and/or verify the direct discovery set using the discovery security materials associated with the ProSe service as specified in clause 6.1.3.2.3.

************ END OF CHANGES************