3GPP TSG-SA3 Meeting #112 	S3-233585

# Goteborg, Sweden, 14 -18 August 2023

# Source:	Philips International B.V.

# Title:	Update 5G ProSe UE-to-UE Discovery Model A – validity time.

# Document for:	Approval

Agenda Item:	4.25

# 1	Decision/action requested

SA3 is kindly requested to approve the proposed text.

# 2	References

[1] 		S3-232007 ProSe Phase 2 draft CR

# 3	Rationale

This Tdoc addresses the following Editor’s Note:

Editor’s Note: how to ensure that valid protected direct discovery sets are announced by the U2U relay is FFS.

By indicating how the validity time is determined and that direct discovery sets are removed once the validity times expire.

4	Detailed proposal

SA3 is kindly requested to approve the below

************ START OF CHANGES************

##### 6.1.3.3.3	Security flows

###### 6.1.3.3.3.1	Security procedure for 5G ProSe UE-to-UE Relay Discovery with Model A

The security procedure for 5G ProSe UE-to-UE Relay Discovery with Model A is described as follows.

Figure 6.1.3.3.3.1-1: Security procedure for 5G ProSe UE-to-UE Relay Discovery with Model A

NOTE 1: The protection of direct discovery set and Announcement message reuses the protection mechanism specified in clause 6.1.3.2.3 of the present document.

1a.	The monitoring 5G ProSe End UE and announcing 5G ProSe End UE are provisioned with the discovery security materials associated with a ProSe service based on the procedure specified in clause 6.1.3.2.2 of the present document.

1b.	The monitoring 5G ProSe End UE, announcing 5G ProSe End UE, and 5G ProSe UE-to-UE Relay are provisioned with discovery security materials associated with an RSC based on the procedure specified in clause 6.1.3.2.2 of the present document.

Editor’s Note: How to provision the discovery security materials associated with a Prose service is FFS.

2.	The announcing 5G ProSe End UE shall protect the direct discovery set using the discovery security materials associated with the ProSe service as specified in clause 6.1.3.2.3 of the present document. The 5G ProSe UE-to-UE Relay obtains the protected RSC and direct discovery set from the announcing 5G ProSe End UE in proximity (e.g., via a previous 5G ProSe UE-to-UE Relay Discovery or 5G ProSe UE-to-UE Relay Communication procedures) as specified in clause 6.3.2.4.2 of TS 23.304 [2]. When 5G ProSe UE-to-UE Relay Discovery is used to deliver the direct discovery set, the announcing 5G ProSe End UE shall include the protected direct discovery set in a discovery message that is protected using the discovery security materials associated with the RSC as specified in clause 6.1.3.2.3 of the present document. When 5G ProSe UE-to-UE Relay Communication is used to deliver the direct discovery set, the announcing 5G ProSe End UE shall use the secure PC5 unicast link with the 5G ProSe UE-to-UE Relay to send the protected direct discovery set. The 5G ProSe UE-to-UE Relay shall store the protected direct discovery set along with its validity time. If the 5G ProSe UE-to-UE Relay Discovery is used to deliver the direct discovery set, the validity time is determined from the UTC-based counter associated to the received discovery message. When 5G ProSe UE-to-UE Relay Communication is used to deliver the direct discovery set, the validity time is determined from a timestamp associated to the protected message.

3.	When broadcasting the Announcement message, the 5G ProSe UE-to-UE Relay shall include the list of valid protected direct discovery sets in the Announcement message and protect the Announcement message using the discovery security materials associated with the RSC as specified in clause 6.1.3.2.3 of the present document. Then, the 5G ProSe UE-to-UE Relay sends the Announcement message. A protected discovery set shall be removed once its validity time has expired.

4.	On receiving the Announcement message from the 5G ProSe UE-to-UE Relay, the monitoring 5G ProSe End UE shall decrypt and/or verify the received Announcement message using the discovery security materials associated with the RSC as specified in clause 6.1.3.2.3 of the present document. If the verification is successful, the monitoring 5G ProSe End UE shall extract the direct discovery set(s) from the Announcement message, and decrypt and/or verify the direct discovery set(s) using the discovery security materials associated with the ProSe service as specified in clause 6.1.3.2.3 of the present document.

************ END OF CHANGES************