3GPP TSG-SA3 Meeting #112 	S3-233928

# Goteborg, Sweden, 14 - 18 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1757 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Authentication method selection and clause structure for non-3GPP access support in SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The procedures for non-3GPP access support in SNPN contain the following Editor's Notes in several places: 1) "Whether procedures for onboarding should be moved to a separate clause is FFS." 2) "How the authentication method is selected is ffs." |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) It is proposed to not move the procedures for onboarding to a separate clause, and simply remove the Editor's Note. 2) The procedures for non-3GPP access support in SNPN do not propose any changes in authentication method selection compared to other procedures in TS 33.501. Therefore, it is proposed to not make any statement on authentication method selection. The Editor's Note is removed and the statement on authentication method selection simplified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Remaining Editor's Notes in normative specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex I.10.2.1, Annex I.10.3.1, Annex I.10.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

### I.10.2.1	Untrusted non-3GPP access support in SNPN without CH

Procedures for untrusted non-3GPP access authentication are described in clause 7.2.1. For SNPN the procedures are modified as follows:

Steps 1-4 are performed as described in clause 7.2.1.

In step 5, the SUCI can be an onboarding SUCI.

Further in step 5, the SUCI can be of type anonymous SUCI if the construction of SUCI as described in clause 6.12 cannot be used and if the employed EAP method supports SUPI privacy.

Step 6 is performed as described in clause 7.2.1.

In step 7 of clause 7.2.1, in case the AUSF receives an onboarding indication, the AUSF shall perform steps 6-10 and 14-17 as described in Annex I.2.2.2.

In the selection of UE authentication method in step 7 of clause 7.2.1, 5G AKA, EAP-AKA’, or any other key-generating EAP authentication method apply. In case UDM receives an anonymous SUCI, the UDM chooses the authentication method as described in step 4 of Annex I.2.2.2.2.

In case the AUSF received an anonymous SUCI in step 7 (but no onboarding indication was received) the AUSF shall perform steps 11-13 of Annex I.2.2.2 after a successful authentication to inform the UDM of the actual SUPI. In case anonymous SUCI and onboarding indication was received in step 7, steps 11-13 of Annex I.2.2.2 can be skipped.

In step 8 of clause 7.2.1 in case an EAP method is used for primary authentication, the AMF shall encapsulate the EAP-Success received from AUSF within the SMC message.

Steps 9-16 are performed as described in clause 7.2.1.

*** NEXT CHANGE ***

### I.10.3.1	Trusted non-3GPP access support in SNPN without CH

Procedures for trusted non-3GPP access authentication are described in clause 7A.2.1. For SNPN the procedures are re-used with the following modifications:

Steps 0-4 are performed as described in clause 7A.2.1.

In step 5, the SUCI can be an onboarding SUCI.

Further in step 5, the SUCI can be of type anonymous SUCI if the construction of SUCI as described in clause 6.12 cannot be used and if the employed EAP method supports SUPI privacy.

Editor’s note: In case anonymous SUCI is used, the identifier used as UE Id in the AN parameters is FFS

Step 6-7 is performed as described in clause 7A.2.1.

In step 8 of clause 7A.2.1, in case the AUSF receives an onboarding indication, the AUSF shall perform steps 6-10 and 14-17 as described in Annex I.2.2.2.

In the selection of UE authentication method in step 8 of clause 7A.2.1, 5G AKA, EAP-AKA’, or any other key-generating EAP authentication method apply. In case UDM receives an anonymous SUCI, the UDM chooses the authentication method as described in step 4 of Annex I.2.2.2.2.

In case the AUSF received an anonymous SUCI in step 7 (but no onboarding indication was received) the AUSF shall perform steps 11-13 of Annex I.2.2.2 after a successful authentication to inform the UDM of the actual SUPI. In case anonymous SUCI and onboarding indication was received in step 7, steps 11-13 of Annex I.2.2.2 can be skipped.

Steps 9-12 are performed as described in clause 7A.2.1.

In step 13, in case anonymous SUCI was used in step 5, a temporary UE identifier shall be used in the IDi payload.

Editor’s note: The value of the temporary identifier is FFS.

Steps 14-19 are performed as described in clause 7A.2.1.

*** NEXT CHANGE ***

### I.10.4.1	Support for N5CW devices in SNPN without CH

Procedures for authentication for devices that do not support 5GC NAS over WLAN access are described in clause 7A.2.4. For SNPN the procedures are modified as follows:

Steps 0-1 are performed as described in clause 7A.2.4.

In step 2 of clause 7A.2.4 the SUCI can be of type anonymous SUCI if the construction of SUCI as described in clause 6.12 cannot be used and if the employed EAP method supports SUPI privacy.

Step 3-6 is performed as described in clause 7A.2.4.

In the selection of authentication method in step 7 of clause 7A.2.4, 5G AKA, EAP-AKA’, or any other key-generating EAP authentication method apply. In case UDM receives an anonymous SUCI, the UDM chooses the authentication method as described in step 4 of Annex I.2.2.2.2.

In step 8 of clause 7A.2.4, in case the AUSF received an anonymous SUCI in step 7, the AUSF shall perform steps 11-13 of Annex I.2.2.2 after a successful authentication to inform the UDM of the actual SUPI.

Steps 9-14 are performed as described in clause 7A.2.4.

*** END CHANGES ***