3GPP TSG-SA3 Meeting #112	S3-234058

# Goteborg, Sweden 14 - 18 August 2023												   revision of S3-23xxxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.503 | CR | draft-CR | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Selection between establishing PC5 security with or without network assistance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-07-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Need a solution for the secetion between establishing PC5 security with or without network assistance |  |  |  |  |  |  |  |  |
| Summary of change: |  | A solution for the secetion between establishing PC5 security with or without network assistance |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete security specification for the UE-to-UE relay communication |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************ START OF CHANGES************

6.6.3.3	Selection between mechanisms with or without network assistance

6.6.3.3.3.1	General

There are two types of security establishment mechanisms for 5G ProSe PC5 Communication for 5G ProSe Layer-3 UE-to-UE relay communications. One is with network assistance, and the other is without network assistance. For PC5 security procedure with network assistance, the procedure described in clause 6.6.3.1 shall be executed. For PC5 security procedure without network assistance, the procedure described in clause 6.6.3.2 shall be executed.

6.6.3.3.3.2	Selection in discovery procedure

The UE_1 sends a 5G ProSe UE-to-UE Relay Discovery Announcement message in 5G ProSe UE-to-UE Relay Discovery with Model A or a 5G ProSe UE-to-UE Relay Discovery Solicitation message in 5G ProSe UE-to-UE Relay Discovery with Model B. Both messages shall include the PC5 security establishment capabilities parameter that lists the PC5 security establishment mechanisms that UE_1 supports.

Upon receiving the 5G ProSe UE-to-UE Relay Discovery message sent by UE_1, the 5G ProSe UE-to-UE Relay shall check whether it support at least one of the mechanisms listed in the PC5 security establishment capabilities parameter. If more than one mechanism can be used, the 5G ProSe UE-to-UE Relay may select the one with the highest priority according to a preconfigured ordered list. If passing the check,  the 5G ProSe UE-to-UE Relay may send a new 5G ProSe UE-to-UE Relay Discovery Announcement message in 5G ProSe UE-to-UE Relay Discovery with Model A or a 5G ProSe UE-to-UE Relay Discovery Solicitation message in 5G ProSe UE-to-UE Relay Discovery with Model B. Both messages shall include the PC5 security establishment capabilities parameter that lists the PC5 security establishment mechanisms that the 5G ProSe UE-to-UE Relay supports.

Upon receiving the 5G ProSe UE-to-UE Relay Discovery Announcement/Solicitation message sent by the5G ProSe UE-to-UE Relay, the UE_2 shall check whether it support at least one of the mechanisms listed in the PC5 security establishment capabilities parameter. If more than one mechanism can be used, the 5G ProSe UE-to-UE Relay may select the one with the highest priority according to a preconfigured ordered list.

In 5G ProSe UE-to-UE Relay Discovery with Model B, UE_2 shall include PC5 security establishment capabilities parameter that contains a PC5 security establishment mechanism chosen by UE_2 in the 5G ProSe UE-to-UE Relay Discovery Response message.

In 5G ProSe UE-to-UE Relay Discovery with Model B, the 5G ProSe UE-to-UE Relay shall include PC5 security establishment capabilities parameter that contains a PC5 security establishment mechanism chosen by the 5G ProSe UE-to-UE Relay in the 5G ProSe UE-to-UE Relay Discovery Response message.

6.6.3.3.3.3	Selection in integrated discovery procedure

For 5G ProSe UE-to-UE Relay Communication with integrated Discovery, similar to discovery process, the UE_1 shall include the security establishment capabilities parameter that lists the PC5 security establishment mechanisms that UE_1 supports in the Direct Communication Request, and the UE_1 shall include the security establishment capabilities parameter that lists the PC5 security establishment mechanisms that 5G ProSe UE-to-UE Relay supports in the Direct Communication Request.

************ END OF CHANGES************