3GPP TSG-SA3 Meeting #117	S3-242785

Maastricht , NL,  19th - 23th Aug 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.738 | CR | 0003 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supplementary on Solution #6: Collection of security related log data of NFs via DCCF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BSI (DE) |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_eNA_SEC_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | OAM data collection is typically limited to specific network function domains, whereas DCCF can integrate multi-source data across domains. Compared to OAM, DCCF provides more flexible data collection, and unified data management, making it especially critical for anomaly detection in complex networks, such as lateral attack detection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a section on collecting security-related log data from NFs via DCCF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If this modification is not accepted, it will be difficult to obtain complete multi-source data from across domains, especially in the collection of security-related log data, which will directly affect the accuracy of anomaly detection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

6.6.2.x Collection of security related log data of NFs via DCCF

The procedure described in Figure 6.6.2.x-1 allows users to request NWDAF to analyze anomalous NF behavior using security logs obtained from DCCF.

Figure 6.6.2.x-1: NWDAF Uses Security Logs Obtained from DCCF to Provide NF Anomalous Behavior Analysis

- NF (such as AF) subscribes to anomaly detection requests by sending Nnwdaf_AnalyticsInfo_Subscribe to NWDAF. The request includes anomaly NF event identifiers, detection target identifiers, and other relevant information.

- Based on the anomalous NF event identifier in the request, NWDAF can subscribe to the DCCF service via Ndccf_DataManagement_Subscribe to obtain NF event information  from the NF to be detected that corresponds to the target identifier (e.g., information related to horizontal anomaly NF attack detection: number of service requests sent/number of successful requests sent, number of service requests received/number of requests successfully processed, service request sending/receiving frequency, resource usage such as CPU, memory, and disk usage).

- DCCF collects all the required information and security logs from the NF and provides them to NWDAF.

4a. The NWDAF subscribes to NRF to receive notification on changes, e.g., on the load and status of NF instances registered in NRF, via Nnrf_NFManagement_NFStatusSubscribe service operation for all targeted NF instances. NF instances are identified by their NF id.

4b. NRF notifies NWDAF of changes on the load and status of the requested NF instances via Nnrf_NFManagement_NFStatusNotify service operation.

5. NWDAF requests the corresponding anomaly analysis model to perform the required analysis based on the NF event information provided by DCCF and NRF

6. NWDAF provides the anomaly analysis results for the NF (including possible horizontal NF attack detections and threat alerts) via Nnwdaf_AnalyticsInfo_Response. Appropriate actions can be taken to prevent the affected NF from affecting other NFs. If necessary, NWDAF can also notify related NFs about the anomaly attack situation.

********** END OF CHANGE **********