# 3GPP TSG-SA3 Meeting #117 	S3-243536
August 19 – 23, 2024  Maastricht, NL                                                Revision of S3-242847

# Source: 	Interdigital

# Title: 	An Update for the Evaluation of Solution #21

# Document for: 	Approval

Agenda Item: 	5.7

# 1	Decision/action requested

Approve the pCR to TR 33.713

# 2	References

[1]	TS 33.501 Security architecture and procedures for 5G System

[2]	TR 33.700-29 Study on Security Aspects of 5G Satellite Access

# 3	Rationale

This contribution proposes an update for the evaluation of Solution #21: Remediation of unauthenticated (D)DOS in S&F.

# 4	Detailed proposal

SA3 is asked to approve the following pCR.

*** Start of 1st Change ***

### 6.21.3	Evaluation

This solution proposes a method for remediation of unauthenticated (D)DOS in S&F. This solution satisfies potential security requirements in KI#1. It applies to the “split MME” scenario and does not need a pre-established security association for “authenticity check,” or pre-authorization of UEs to remediate threats described by KI#1.

This solution (i.e., steps 2 – 4, and 7) adds the additional workload for the Network Nodes and NFs on board of satellite in the S&F mode. Such an additional workload can be an acceptable tradeoff for remediation of (D)DOS attack under the conditions where the remediation of vulnerability to both, malicious and benign (D)DOS is more important than the additional workload.

Further, this solution adds one full round-trip (i.e., steps 1-5) designed specifically to provide unauthenticated (D)DOS remediation before the AKA run. This roundtrip causes the associated delay that is the result of the proposed method of (D)DOS remediation. This additional delay represents a disadvantage of this solution.

For this solution to work:

The equipment on board of satellite (e.g., eNB or gNB) has to be able to produce puzzles and associated parameters. No additional cryptographic functionality above hash and encryption is envisioned to be needed.

The UE has to be able to solve the selected puzzle and produce the evidence. No additional cryptographic functionality above hash and encryption is envisioned to be needed.

Editor’s Note: Further evaluation is FFS.

.*** End of 1st Change ***