3GPP TSG-SA3 Meeting #117	S3-243629

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Solution for secure retrieval of 5G UE privacy related information based on AKMA

# Document for:	Approval

Agenda Item:	5.14

# 1	Decision/action requested

It is required to approve this contribution for TR 33.749.

# 2	References

[1]	3GPP TR 33.749 Study on security aspects of enhancement of support for edge computing in the 5G Core (5GC) phase 3

# 3	Rationale

This contribution proposes a solution for KI#1 of TR 33.749 [1], corresponding to the secure retrieval of 5G system UE Ids and privacy related information. The solution addresses both security requirements, i.e., the verification of the user information provided by the EEC, as well as the protection of the UE privacy. The overall procedure is based on AKMA.

# 4	Detailed proposal

****START of 1st CHANGES*****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-49: "Study on Enhancement of support for Edge Computing in 5G Core network - Phase 3".

[3]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[4]	3GPP TS 33.558: "Security aspects of enhancement of support for enabling edge applications; Stage 2".

[5]	3GPP TR 33.839: "Study on security aspects of enhancement of support for edge computing in the 5G Core ".

[6]	3GPP TR 33.739: "Study on security enhancement of support for edge computing phase 2".

[7]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[8]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[xx]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)"

****END of 2nd CHANGES*****

****START of 2nd CHANGES*****

## 6.Y	Solution #Y: Secure retrieval of 5G UE privacy related information based on AKMA

### 6.Y.1	Introduction

This solution addresses the security requirements exposed in key issue #1 related to the secure retrieval of 5G system UE privacy related information, i.e., the verification of the user information provided by the EEC, and the protection of the UE privacy.

The main design principle of the solution is based on AKMA feature, more specifically on the A-KID, an application specific ID derived from the long term symmetric pre-shared key K between the UE and the network. The EDGE application server (EAS) and/or enabler server (EES), use the AKMA protocol to retrieve the 5G UE privacy related information.

### The solution assumes that AF (EES/EAS) communicates with 5GC through NEF.6.Y.2	Solution details

Figure 6.Y.2-1 illustrates the procedure to fetch the 5G system UE privacy related information.

Figure 6.Y.2-1: Procedure to fetch the 5G system UE Id and privacy related information

Preliminary steps:

- Primary authentication

- AKMA Key derivation (A-KID) as per procedure in TS 33.535 [xx].

- Creation of AKMA context in AAnF (SUPI, KAKMA, A-KID) as per procedure in [xx].

1. The application client in the UE (EEC) requests a service to the AF, i.e., EDGE application server (EAS, EES), including the A-KID (AKMA Key Identifier) in the request.

2-5. AKMA procedure intended to provide KAF to the AF from the AAnF via NEF as specified in clause 6.3 of [xx].

NOTE: AF specific UE Id does not need to be part of the of the incoming request (step 2), since the NEF will provide the GPSI (external ID) to AF.

6. The AF requires to know privacy information of the UE available in the 5G Core, such as the the location of UE, and makes the request to the 5G Core via NEF, using the given GPSI as UE identifier.

7. The NEF checks the GPSI and if the request is authorized, the corresponding information, in this example location of the UE, is provided to the AF.

8. The AF responds to the UE (EEC).

### 6.Y.3	Evaluation

The main impact of the solution is that it requires the support of AKMA feature in the 5G system.

The 5G UE Id is not required to be fetched by the AF, since 5G Core through AKMA feature provides already the authentication service, and the trust between UE, 5G Core and AF.

Editor’s Note: The evaluation should state the applicability of the solution, i.e. what is the solution solving and what are the assumptions.

Editor’t Note: Further evaluation is to be defined.

****END of 2nd CHANGES*****