3GPP TSG-SA3 Meeting #118 	S3-244175

# Hyderabad, India, 14 - 18 October 2024

# Source:	Apple

# Title:	New solution on mutual authentication

# Document for:	Approval

Agenda Item:	5.6

# 1	Decision/action requested

Approve the pCR to TR 33.713

# References

- TR 33.713 v0.3.0

# 3	Rationale

This contribution proposes a new solution on mutual authentication.

# 4	Detailed proposal

*** Start of 1st Change ***

## 6.X	Solution #X: Mutual authentication for AIoT system

### 6.X.1	Introduction

This solution addresses key issue#5: “Authentication in Ambient IoT service.”

### 6.X.2	Details

Preassumption:

- AIoT AUSF and AIoT UDM are independent network entities for AIoT system, they can also be collocated with legacy AUSF and UDM, depending on operators’ deployment.

- The Applacation Function is out of operator domain, which can be the AIoT device manufacture.

- It is assumed AIoT UDM owns the AIoT device credentials, while Application Function has no access to those credentials.

- Step 1-3 are following RAN2 procedure using 2-step RACH.

Step 0a. the AIoT device is configured with Device ID and K as the root key in the manufacturing time.

Step 0b. AIoT UDM is configured with K as the root key for the device. Every AIoT device owns a unique K.

Step 0c. The AF may sync up with AIoT UDM the device IDs after manufacturing.

Step 1. Reader sends the paging message including the device ID(s).

NOTE: what IDs to be included will be decided by RAN2.

Step 2-3. AIoT device sends random ID, device ID in Msg1. Reader echoes back the random ID in Msg2.

Step 4. AIoTF triggers the authentication through Reader.

Step 5. AIoT device calculates the RES using K and device ID, using HASH function.

Step 6. AIoT device sends the device ID, RES, and random number Counter to Reader in uplink AS message.

NOTE: The message details to be dependant on SA2 decision.

Step 7-8. Reader sends the Authentication Request to AIoT AUSF for authentication.

Step 9. AIoT AUSF sends authentication request to AIoT UDM including the device ID and Counter.

Step 10, AIoT UDM calculates the XRES and Token. AIoT UDM then sends both values back to AIoT AUSF.

Step 11. AIoT AUSF compares the RES and XRES, if they are equal, AIoT AUSF sends Token back to Reader. Otherwise, the authentication fails.

Step 12-13. the AIoT AUSF passes the authentication result and Token back to Reader.

Step 14. If the authentication success, the Reader continue with step 15, otherwise, the Reader ceases the authentication procedure.

Step 15. Reader sends the Token to AIoT devices in DL command message.

Step16. AIoT device verifies if the Token is correct. If yes, then AIoT device replies with the UL message. Otherwise, the AIoT device will cease the procedure.

### 6.X.3	Evaluation

TBD

*** End of 2nd Change ***