3GPP TSG-SA3 Meeting #119AdHoc-e	draft_S3-250013-r431

Online, Electronic meeting, 13 -16 January 2025

- Source:	Lenovo, ZTE, OPPO

- Title:	Conclusion to key issue#1

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This contribution proposes to add conclusion of key issue #1.

# 2	References

[1]		TR 23.700-13

# 3	Rationale

The protection mechanism for disabling device operation shall be treated in the same way as the solution for the message protection of Key issue #4: Protection of information during AIoT service communication. The same solution for confidentiality and integrity protection of the command message shall be applied.

# 4	Detailed proposal

Start of Changes

## 7.X	Conclusion to Key Issue#1 on Protection for disabling device operation

The protection mechanism for temporarily disabling device operation and permanently disabling device operation shall use the inventory and command procedure.

Editor’s Note: Whether and how the enabling device operation is feasible is FFS.

The message protection conclusion of Key issue #4: Protection of information during AIoT service communication shall apply to the protection of the disabling/enabling operations.conclusion

End of Changes