3GPP TSG-SA3 Meeting #119 	S3-244804

# Orlando, US, 11 - 14 November 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2070 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Proposal for changing the description of SN counter in SCPAC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2024-11-4 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The description for SCPAC counter caused confusion in other working groups, it is not clear whether the SN counter needs to be changed when the KEY (KMN) was changed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The description on how to derive the KSN for DC and SCPAC has caused some confusion in other working group, it is proposed to change the wording and clarify the requirement on not to reuse the SN counter for the same KEY. In the case of “within DC”, when the KSN needs to be updated and the KMN (KgNB) is not changed, SN counter shall not be reused. In the case of “across DC”, when MN needs to establish the SN addition procedure with a second SN, and the KMN (KgNB) is not changed, SN counter used to derive the new KSN shall not be reused. In the case of “SCPAC”, when MN is deriving KSN for different SNs and the KMN (KgNB) is not changed, SN counter for deriving different KSN shall not be reused. Whenever there is KMN (KgNB) update, KSN will be updated due to KEY change, then this requirement is not applied any more. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusing description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGES ***

# A.16	Derivation of KSN for dual connectivity

This input string is used when the MN and UE derive KSN during dual connectivity. The following input parameters shall be used:

-	FC =0x79,

-	P0 = Value of the SN Counter as a non-negative integer,

-	L0 = length of the SN Counter value (i.e. 0x00 0x02).

The input key KEY shall be KeNB when the MN is an ng-eNB and KgNB when the MN is a gNB.

NOTE: The same input string is used for both DC and SCPAC. Therefore, it is crucial that the SN Counter value input is never reused for the same KEY within DC, across DC and SCPAC.

*** END OF CHANGES ***