3GPP TSG-SA3 Meeting #120	S3-250321

Athens, Greece, 17 - 21 February 2025

# Source:	3GPP CT6

# Title:	Reply LS on UE behaviour in case of SUCI calculation failure

# Document for:	Information, Discussion

Agenda Item:                3.1

3GPP TSG-CT WG6 Meeting #120-bis	C6-240721

# Orlando, US, 19th Nov – 22nd Nov 2024

Title:	Reply LS on UE behaviour in case of SUCI calculation failure

Response to:	Reply LS S3-245137/C6-240624 on UE behaviour in case of SUCI calculation failure

Release:	Rel-19

Work Item:	5GProtoc19

Source:	3GPP CT WG6

To:	3GPP SA WG3

Cc:	3GPP CT WG1

Contact Person:	Sergi Espi

sergi.espi@gi-de.com

Send any reply LS to:		3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: 	none

# 1	Overall description

Request 1: SA3 asks CT6 to provide SA3 with details on what could lead to the incorrect configuration of the information required for the SUCI calculation in the USIM, e.g. when and how it happens and also why the error case is needed.

When the USIM Application processes any APDU command (e.g. AUTHENTICATE), there may be scenarios where it cannot fulfil its function. In such cases, the USIM Application reports the malfunction with a status word indicating the cause of the error.

The GET IDENTITY command is not an exception. The purpose of C6-240155 (CR #1030 for TS 31.102) is to clarify the behavior when the information required for the SUCI calculation is not correctly configured in the USIM and to inform the ME about this problem.

The information required for the SUCI calculation, as per clause 7.5.1.1 of TS 31.102, includes:

- Home network identifier

- Routing indicator

- Home network public key

- Home network public key identifier

- Protection scheme identifier

- SUPI

A configuration error occurs if the above information is either not correctly provisioned to the USIM (e.g. technical mechanism to provision the information has a failure, incomplete information is provided, etc.) or the provisioned information is inconsistent (e.g. mismatched Home network public key size, invalid protection scheme identifier).

In case such errors prevent the USIM from calculating the SUCI, the CR proposes to return an error status word as defined in TS 31.102 clause 7.3.2, because none of the conditions to allow to fallback to the “null” scheme, as defined in clause 6.12.2 of TS 33.501, is fulfilled:

The UE shall generate a SUCI using "null-scheme" only in the following cases:

- if the UE is making an unauthenticated emergency session and it does not have a 5G-GUTI to the chosen PLMN, or

- if the home network has configured "null-scheme" to be used, or

- if the home network has not provisioned the public key needed to generate a SUCI.

It is unrealistic for the CR to define an exhaustive list of scenarios where such misconfigurations occur because it depends on how the information required for the SUCI calculation is stored and managed by the USIM Application, which is beyond the scope of the specification as described in clause 7.5.1.1 of TS 31.102 (except for the Routing indicator and SUPI). It also depends on the implementation of the SUCI calculation by the USIM Application, where behaviors (for the parts that are beyond what is defined in the specification) may vary across different implementations.

Similar situations can also occur in the case that the ME is supposed to calculate the SUCI and any of the required parameters for the SUCI calculation are not correctly configured or the SUCI calculation failed for any other reason. Also none of the conditions for using the “null” scheme is satisfied.

Request 2: CT6 are requested to let SA3 know on their view on the expected frequency of the error case.

A configuration error as described above is considered an exceptional and rare situation because operators take the utmost care in configuring this information correctly by performing a validation process of the USIM configurations (including the information required for the SUCI calculation) that will be used on their UICCs. Such a validation process will easily detect a configuration error in the SUCI part.

The probability is as low as the risk that the information required for network authentication (e.g. K and OpC) is not correctly configured to the USIM.

# 2	Actions

To SA3

ACTION: CT6 asks SA3 to take into consideration the above information.

# 3	Dates of next TSG CT WG 6 meetings

3GPP TSG CT6#121	18 Feb - 21 Feb 2025	Athens, GR

3GPP TSG CT6#122	20 May - 23 May 2025	Bratislava, SK