3GPP TSG-SA3 Meeting #120	S3-250323

Athens, Greece, 17 - 21 February 2025

# Source:	3GPP RAN1

# Title:	LS on signalling feasibility of dataset and parameter sharing

# Document for:	Information, Discussion

Agenda Item:                3.1

3GPP TSG RAN WG1 #119                                                                                                    R1-2410922  Orlando, Florida, USA, November 18th – 22nd, 2024

Title: 			LS on signalling feasibility of dataset and parameter sharing

Response to:		-

Release:		Release 19

Work Item:		FS_NR_AIML_air

Source:		RAN1

To:			RAN2

CC:			SA2, SA3, SA5

Contact Person:

Name:	Taesang Yoo

E-mail Address:	taesangy@qti.quallcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# Overall description

The Rel-19 two-sided AI/ML CSI compression study for AI/ML Air Interface has the following study objectives (RP-242399; revision of RP-240774)

Study objectives: - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing necessary specification impact analysis, as well as, other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

To fulfil the study objectives, RAN1 has been discussing various Options, Sub-Options, Directions for alleviating/resolving issues related to inter-vendor training collaboration. In particular, Direction A of RAN1’s discussion requires the following procedure:

- NW-side shares model parameters and/or dataset to the UE or UE-side via standardized signaling, where the model parameters and/or dataset received at the UE or UE-side go through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., model (re-)training.

Please note that RAN1 has been using various terminologies that are synonymous

- UE-part of a two-sided CSI compression model = CSI generation part = encoder

- NW-part of a two-sided CSI compression model = CSI reconstruction part = decoder

There are several Options and Sub-Options for the contents for the sharing. They are shown in the RAN1 agreements in the Appendix. Among the various Options and Sub-Options for Direction A, after down-selection, RAN1 is considering the following Sub-Options for further study with potential further down-selection.

- Option 4-1: sharing dataset containing N1 samples of {target CSI, CSI feedback} corresponding to the input and output of a nominal encoder, respectively

- Option 3a-1 without target CSI: sharing parameters corresponding to a nominal encoder

- Option 3a-1 with target CSI: sharing parameters corresponding to a nominal encoder, along with dataset containing N2 samples of {target CSI}

The dataset and/or parameter sharing is per training collaboration instance, originating from a NW-side training entity and consumed at a UE-side training entity.

RAN1 concluded that the standardized signaling may be

- over-the-air, or

- other approaches

RAN1 concluded that a standardized signaling, if feasible and specified, can be used for parameter and/or dataset exchange to alleviate/resolve the inter-vendor training collaboration complexity.

Conclusion (RAN1 #117) Standardized signalling, if feasible and specified, can be used for parameter / model exchange in option 3a/5a and 3b to alleviate/resolve the inter-vendor training collaboration complexity. - Standardized signalling may be reused for exchanging CSI generation part, CSI reconstruction part, or both, etc, when necessary and feasible. - Standarized signalling may be over-the-air, or other approaches. Standardized signalling, if feasible and specified, can be used for dataset exchange in option 4 to alleviate/resolve the inter-vendor training collaboration complexity. - Standardized signalling may be reused for dataset exchanging, when necessary and feasible. - Standarized signalling may be over-the-air, or other approaches. Note: feasibility will be discussed separately.

RAN1 would like to ask RAN2’s feedback on the feasibility of a standardized signaling, via over-the-air and/or other approaches, for the dataset and/or parameter exchange in the above Option 4-1 and Option 3a-1, considering their overhead and any other aspects that may affect their feasibility.

RAN2 may assume the following for the contents for the feasibility discussion.

For Option 4-1: sharing {target CSI, CSI feedback} dataset:

  - Data size for target CSI depends on the format. There is no agreement on the format or necessary precision of the target CSI. Some examples based on companies’ evaluations are: eType-II format (up to ~1000 bits), eType-II-like format (~ a few 1000 bits), and float32 format (up to ~ 150K bits). The data size may also vary depending on the configuration, and the captured value indicates the order of magnitude of the typical data size per sample as a guideline.

  - There is no agreement on the CSI feedback size. Values in the order of eType II payload size may be assumed (up to ~ 1000 bits) for RAN2 discussion.

  - There is no agreement on the total number of samples N1 in the dataset. The range of the number of samples N1 used in RAN1 evaluations is 40K-1200K. As a representative number, RAN2 may assume that the total number of samples is around N1=600K.

  - Summing up the above, and assuming eType-II-like format of a size 2000 bits, the total dataset size for Option 4-1 may be roughly in the order of 600K * (2000 bits + 1000 bits) / (8bits/Byte) = 225 MB. The size may be smaller for eType-II format. The size may be larger if we consider the dataset consisting of supporting multiple configurations.

For Option 3a-1 without target CSI: sharing encoder parameter:

  - The number of parameters in the encoder models used in RAN1 evaluations have a wide range, ranging from 36K parameters to 13M parameters, with the median value of 5.8M.

  - Based on the above range, the size of encoder parameters may range from 36K*(8/8)=36KB based on the smallest model size in RAN1 evaluations and the use of float8, to 13M*(32/8)=52M based on the largest model size in RAN1 evaluations and the use of float32. Based on the median size, RAN2 may assume the size of 5.8M*(16/8) = 11.6 MB based on the medium model size and the use of float16.

For Option 3a-1 with target CSI: sharing encoder parameters, along with {target CSI} dataset:

  - The size of the encoder parameters is the same as in the Option 3a-1 without target CSI. The size of the dataset is smaller than the Option 4-1, because the dataset only contains the target CSI but not the CSI feedback. The number of samples N2 in the dataset may be the same or smaller than N1. The size becomes N2 * (2000 bits) / (8bits/Byte) + 11.6 MB.

Regarding latency and frequency of the dataset and/or parameter sharing: As the purpose of the dataset and/or parameter sharing is for a UE-side offline training, the dataset and/or parameter sharing is expected to have relaxed latency (e.g., days/weeks) and be infrequent.

Regarding how many of such dataset and/or parameter sharing is needed: RAN1 evaluations shows good generalization ability, so RAN2 may assume that the number of dataset and/or parameter sharing instances will be limited to a small number.

# Actions

To: RAN2

CC: SA2/SA3/SA5

Action: RAN1 respectfully asks RAN2’s feedback on the feasibility of standardized signaling (over-the-air and/or other approaches) for NW-side sharing model parameters and/or dataset to the UE or UE-side for the following options

- Dataset sharing consisting of {(Target CSI, CSI feedback)}

- Encoder parameter sharing

- Encoder parameter sharing + dataset sharing consisting of {target CSI}

# Dates of next TSG-WG1

RAN1#120	from 2025-02-17	to 2025-02-21		Athens

RAN1#120bis	from 2025-04-07	to 2025-04-11		China

RAN1#121	from 2025-05-19	to 2025-05-2		Malta

# Appendix – list of related relevant RAN1 agreements

## RAN1-116

Agreement

To alleviate / resolve the issues related to inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, study the following options:

- Option 1: Fully standardized reference model (structure + parameters)

- Option 2: Standardized dataset

- Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side

- Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side

- Option 5: Standardized model format + Reference model exchange between NW-side and UE-side

Note 1: The above options may not be mutually exclusive and may be used together.

Note 2: Other options are not precluded.

Note 3: The study should consider how different methods of exchanging the parameters / dataset / reference model would affect the feasibility and collaboration complexity of options 3 / 4 / 5 respectively, e.g., over the air-interface, offline delivery, etc.

Note 4: “Dataset” refers to a set of data samples of CSI feedback and associated target CSI.

Agreement

For the study of inter-vendor collaboration issues for AI/ML-based CSI compression using a two-sided model, consider at least the following aspects when comparing different options:

- Inter-vendor collaboration complexity, e.g., whether bilateral collaboration is required between vendors.

- Performance.

- Interoperability and RAN4 / testing related aspects.

- Feasibility.

## RAN1 #116-bis

Agreement

- For Option 3, further define the two sub-options:

  - 3a: Parameters received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing.

  - 3b: Parameters received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations.

- For Option 5, further define the two sub-options:

  - 5a: Model received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing.

  - 5b: Model received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations.

- For Option 4, it is clarified that:

  - Dataset received at the UE or UE-side goes through offline engineering at the UE- side (e.g., UE-side OTT server), e.g., model training or offline testing.

- Note: The descriptions under each option are only for the purpose of simplified discussion and do not mean deprioritizing any other flavors (such as an exchange originating from the UE-side and ending at the NW-side) from potential specification.

Agreement

- For Option 3/4/5, focus further discussion on the following assumptions:

  - Option 3a/5a

    - The model(5a)/parameter(3a) exchange originates from the NW-side and ends at the UE-side.

    - Model(5a)/parameters(3a) exchanged from the NW-side to UE-side is either CSI generation or reconstruction part or both.

      - Option 3a-1/5a-1: Model/Parameters exchanged from the NW-side to UE-side is CSI generation part.

      - Option 3a-2/5a-2: Model/Parameters exchanged from the NW-side to UE-side is CSI reconstruction part.

      - Option 3a-3/5a-3: Model/Parameters exchanged from the NW-side to UE-side are both CSI generation part and CSI reconstruction part.

      - Some additional information, if necessary, may be shared from the NW-side to help UE-side offline engineering and provide performance guidance.

        - Performance target

        - Dataset or information related to collecting dataset

    - Study different methods of exchanging, e.g., over the air-interface, offline delivery, etc.

  - Option 3b

    - The method of exchanging is over the air-interface via model transfer/delivery Case z4.

    - The parameter exchange is from NW to UE.

    - Parameters exchanged from the NW-side to UE-side is CSI generation part.

  - Option 5b

    - The method of exchanging is over the air-interface via model transfer/delivery Case z4, assuming that the model structure is aligned based on offline inter-vendor collaboration.

    - The model exchange is from NW to UE.

    - Model exchanged from the NW-side to UE-side is CSI generation part.

  - Option 4:

    - The dataset exchange originates from the NW-side and ends at the UE-side.

    - Option 4-1: Dataset exchanged from the NW-side to UE-side consists of (target CSI,  CSI feedback).

    - Option 4-2: Dataset exchanged from the NW-side to UE-side consists of (CSI feedback, reconstructed target CSI).

    - Option 4-3: Dataset exchanged from the NW-side to UE-side consists of (target CSI, CSI feedback, reconstructed target CSI).

    - Some additional information, if necessary, may be shared from the NW-side to help UE-side offline engineering and provide performance guidance.

      - Performance target

    - Study different methods of exchanging, e.g., over the air-interface, offline delivery, etc.

  - Note: For each option/sub-option of interest, companies to bring discussion on how inter-vendor collaboration complexity, interoperability, and feasibility may be addressed. Companies to strive to provide solution(s) that can address all the following aspects: inter-vendor collaboration complexity, performance, interoperability, and feasibility.

  - Note: The descriptions under each option are only for the purpose of simplified discussion and do not mean deprioritizing any other flavors (such as an exchange originating from the UE-side and ending at the NW-side) from potential specification.

## RAN1 #117

Conclusion

Standardized signalling, if feasible and specified, can be used for parameter / model exchange in option 3a/5a and 3b to alleviate/resolve the inter-vendor training collaboration complexity.

- Standardized signalling may be reused for exchanging CSI generation part, CSI reconstruction part, or both, etc, when necessary and feasible.

- Standarized signalling may be over-the-air, or other approaches.

Standardized signalling, if feasible and specified, can be used for dataset exchange in option 4 to alleviate/resolve the inter-vendor training collaboration complexity.

- Standardized signalling may be reused for dataset exchanging, when necessary and feasible.

- Standarized signalling may be over-the-air, or other approaches.

Note: feasibility will be discussed separately.

Agreement

- For option 3a/3b/4/5a and their sub-options, at least the following potential specification impacts have been identified. Further study the necessity, feasibility, their specification impact.

- Exchange

  - Parameter / model exchange methods, format/contents, and related spec impacts (3a/3b/5a)

  - Dataset exchange methods, format/type/contents of data/dataset, and related spec impacts (4)

  - Additional information, if necessary, that may be shared from the NW-side to help UE-side offline engineering and provide performance guidance (3a/5a/4)

    - Performance target (3a/5a/4)

    - Dataset or information related to collecting dataset (3a/5a)

    - Any other additional information

- Model pairing (3a/3b/4/5a)

- UE capability (3a/3b/4/5a)

- Model related aspects, such as scalability (e.g., payload sizes, antenna ports, bandwidth), rank and layer handling (3a/3b/4/5a)

- Quantization of feedback (3a/3b/4/5a)

- Model structure details (3a/3b)

Note: Option 3a/4/5a and option 3b serve two different deployment time scales, UE capabilities, device-side optimizations, and training methods, and therefore may be complementary to each other, with potential specification of both.

- Specification of option 1, if needed from RAN1, can reuse specification of option 3a/3b, with the additional specification of parameters.

## RAN1 #118

Observation

Option 5 alone doesn’t fully address inter-vendor collaboration complexity, but Option 5 has potential performance benefit and flexibility compared to Option 3.

Agreement

Continue the study of following directions - on-device operation and UE side offline engineering

- Direction A: Sharing parameters/reference model/dataset that enables UE-side offline engineering (Inter vendor collaboration option 3a/5a/4)

  - Potential down-selection into one or more among sub-options 3a/5a-1, 3a/5a-2, 3a/5a-3, 4-1, 4-2, and 4-3 considering their feasibility and performance, including at least the following issues

    - [Issue 1] What additional information should be shared from NW-side to UE-side to enable UE-side encoder training, validation, and testing?

    - [Issue 2] Is there concern for NW’s proprietary information disclosure, and if so, how to address it?

    - [Issue 3] Is there an overhead concern, and if so, how to address it?

    - [Issue 4] Is there performance impact due to mismatch between NW side data distribution and UE side data distribution, and if so, how to address it?

- Direction B: Sharing NW side encoder parameter to UE side for UE side inference directly with on-device operation (Inter vendor collaboration option 3b), including at least the following issues

  - [Issue 3] Is there an overhead concern, and if so, how to address it?

  - [Issue 5] Whether it is feasible to use a common encoder across UEs, and whether it is feasible for NW-side to train multiple encoders for different UEs?

  - [Issue 6] Is there performance impact due to mismatch between NW side data distribution and UE side inference data distribution, and if so, how to address it?

  - [Issue 7] Is there concern for NW’s and UE’s proprietary information disclosure, and if so, how to address it?

- Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1), including at least the following issues

  - [Issue 8] Whether to consider 3GPP’s statistical channel model or field data for reference model(s) training. In case of the latter, how does RAN1 collect field data and agree on them?

  - [Issue 9] Is there performance impact due to mismatch between the distribution of the dataset used for reference model(s) training, UE-side data distribution, and NW-side data distribution, and if so, how to address it?

  - [Issue 10] What additional information should be specified to enable UE-side encoder training, validation, and testing, and NW-side decoder training, validation, and testing?

  - Note:

    - 1-1: Only reference encoder is specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference encoder.

    - 1-2: Only reference decoder is specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference decoder.

    - 1-3: Both reference encoder and reference decoder are specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately that are compatible to the reference decoder/encoder.

- Note: UE-side data and NW-side data in “UE-side data distribution” and “NW-side data distribution” are field data.

- Note: Some issues identified in one direction may/may not be applicable for other Direction.

- Note: potential down selection among the 3 directions is not precluded

- Study of data distribution mismatch to consider the use of synthetic data and/or field data.

## RAN1 #118-bis

Conclusion

For issues listed for inter-vendor collaboration Direction A, conclude the following

- Answer to Issue 2: From RAN1 perspective, at least,

  - For option 4-1, there is no proprietary information concern of disclosing encoder input/output from NW side to UE side

  - For option 3a-1, there is no proprietary information concern of disclosing encoder parameters from NW side to UE side

  - Note: If some additional information is disclosed from network side to UE side, proprietary concerns for option 4-1 and 3a-1 can not be precluded by default.

Agreement

For issues listed for inter-vendor collaboration direction B, further study the overhead in Direction B based on the size of the encoder (i.e., number of parameters and quantization level), the number of encoders, and how often the parameters need to be transferred.

Agreement

For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression,

- For Direction A:

  - Further study the following options with potential down-selection

    - 4-1: dataset is {target CSI, CSI feedback},

    - 3a-1: encoder parameter exchange, with {target CSI}

    - Note: Additional information beyond what’s listed above may need to be exchanged if necessary.

Agreement

- For Direction C:

  - It is clarified that, the purpose of a fully specified CSI generation (reconstruction) part is to enable development/training of compatible CSI reconstruction (generation) parts at the NW (UE) side.

  - Study whether the fully specified reference models or their compatible models may provide reasonable performance for deployment at NW and UE

    - Including performance evaluation of training/finetuning the models by modeling data distribution mismatch between synthetic and field data, and/or by using actual field data

    - Including performance evaluation of training/finetuning the models by modelling data  distribution mismatch between NW-side field data and UE-side field data, and/or by using actual field data from both NW-side and UE-side

    - Note: Training/finetuning is done at either or both sides without inter-vendor collaboration.

  - FFS whether the reference model is the encoder (1-1) or decoder (1-2) or both (1-3)

  - FFS whether the reference model is the same as RAN4 specified model

  - FFS whether a single or multiple reference (and how many) models need to be specified