3GPP TSG-SA WG3#120	S3-250363

Athens, Greece, 17-21 February 2025

Agenda Item:	4.1.15

Source: 	MediaTek Inc., Ericsson

Title:  	Mitigation of GNSS spoofing

Document for:	Discussion, decision

# 1	Introduction

This document gives a brief discussion of the issue of GNSS signal spoofing in field operation and suggests a way forward to allow receivers to help mitigate such situations.  An LS to RAN2 to trigger specification work is proposed (section 5 below).

# 2	Discussion

## 2.1	Background

Spoofing of GNSS signals is an increasing problem in the field.  Commercial aviation in some regions has been significantly impacted ([1]), with the expectation that UAS and ground receivers relying on 3GPP support for GNSS positioning would also be affected.  As noted in [1], a bad baseline location can propagate to affect subsequent location estimates even after the spoofed signal is no longer present.

A receiver may be able to detect spoofing or jamming, but in general it cannot defend itself.  In the presence of a degraded GNSS signal of which the receiver is aware, the confidence of the result could be indicated as poor, but LPP today does not support a suitable cause for indicating such degradation.  Depending on server implementation, a server faced with a poor GNSS location estimate might try again with the same result.

## 2.2	Attack detection and reporting

There are known methods for a receiver to detect a probable attack heuristically ([2]), ranging from relatively simple power- or doppler-based methods to sophisticated machine-learning approaches.  Some GNSSs also provide authentication in the signal, allowing the receiver to identify spoofing.  When an attack is detected, there is no particular technical difficulty in reporting the information to the positioning server via LPP.

The LPP GNSS error reporting format in [3] (A-GNSS-Error inside A-GNSS-ProvideLocationInformation) can already support sending an error code along with measurements or a location estimate, and this mechanism clearly could be extended to report an attack as a new error type.  Other approaches such as involving the integrity framework could be considered, though the specification work would be larger and not all systems/UEs will support integrity.  Any form of reporting gives the server the opportunity to take an implementation-dependent decision on how to handle the situation, such as falling back to another positioning method (or a different constellation where applicable).

We anticipate that a server would largely act on crowdsourced information rather than individual reports, since an individual UE might report an attack erroneously.  For a UE that does report a nonexistent attack (by mistake or maliciously), the system seems to be self-correcting; the server may apply alternative positioning to that UE based on the report, which should still result in a valid location estimate and in any case does not affect the performance of the system as a whole.  For this reason, the UE can be free to apply implementation-dependent detection algorithms; a UE that mistakenly detects an attack only interferes with its own location service.

## 2.3	Spec impact

Reporting a new LPP error cause has minimal impact to [3] (extending the IE GNSS-TargetDeviceErrorCauses).  A solution using the integrity framework would have wider impact and require discussion in RAN2, and it would not be available to all UEs; thus we see a solution with an error cause as preferable.

We do not identify impact to LCS signalling or location management services, both of which are generally agnostic to specific positioning methods.  However, it seems beneficial to inform CT1 and CT4 of the issue and allow them to determine if there is impact.  There also seems clearly to be no impact to NRPPa, whose only involvement with GNSS is for assistance data broadcast.  However, RAN3 may want to consider whether related measures should be taken related to GNSS sync for base stations, so we suggest including RAN3 on the LS as well.

Proposal 1: Send an LS to RAN2/RAN3 (Cc: CT1/CT4) requesting the introduction of an LPP indication for detection of GNSS spoofing.

A draft LS is provided.

# 3	Conclusion

This document promulgated the following proposal:

Proposal 1: Send an LS to RAN2/RAN3 (Cc: CT1/CT4) requesting the introduction of an LPP indication for detection of GNSS spoofing.

# 4	References

[1]	“Electronic Warfare Spooks Airlines, Pilots and Air-Safety Officials”, The Wall Street Journal, 2024.09.23, https://www.wsj.com/business/airlines/electronic-warfare-spooks-airlines-pilots-and-air-safety-officials-60959bbd [paywall]

[2] “Recent Advances on Jamming and Spoofing Detection in GNSS”, Sensors 2024, 24(13), 4210, https://www.mdpi.com.1424-8220/24/13/4210

[3] TS 37.355

# 5	Draft LS

Title:	LS on Mitigation of GNSS Spoofing

Response to:

Release:	Rel-19

Work Item:	TBD

Source:	MediaTek Inc., Ericsson (to be SA3)

To:	RAN2, RAN3

Cc:	CT1, CT4

Contact person:	Nathan Tenny nathan.tenny@mediatek.com

Markus Hanhisalo markus.hanhisalo@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA3 have discussed the issue of GNSS spoofing and concluded that there is a benefit if the location server can be made aware when a target device detects a probable spoofing attack.  SA3 would like to request RAN2 to introduce an indication in LPP for such situations, e.g., as a GNSS target device error cause.

SA3 have not identified impacts to other protocols, but SA3 think that RAN3 may want to evaluate whether related issues arise in connection with GNSS synchronization for gNBs, and if so, whether analogous measures should be taken.

# 2	Actions

To RAN2:

ACTION: 	SA3 kindly asks RAN2 to introduce an indication in LPP for the case that the target device detects a probable spoofing attack on GNSS.

To RAN3:

ACTION:	SA3 kindly asks RAN3 to consider whether related measures should be taken in connection with GNSS synchronization for gNBs.

# 3	Dates of next SA3 meetings

TSG-SA3 Meeting #121	7-11 April 2025	Göteborg, SE

TSG-SA3 Meeting #122	19-23 May 2025	Fukuoka, JP