3GPP TSG-SA3 (Security) 	 S3-100649

- SA3#59, 26-30 April 2010, Lisbon, Portugal	revision of S3-100436

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.812 | CR | 0022 |  rev | 1 |  Current version: | 9.1.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | TR 33.812 Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Alcatel Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code:  | FS_UM2M |  |  |  |  |  |  | Date:  |  | 29/04/2010 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-9 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | Current text in section 4.2 contains some requirements that should be combined together in order to avoid confusion. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Two requirements are merged to one. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Text on requirements will seem non-organised |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

************* START OF 1. CHANGE *************

## 4.3	Requirements

### 4.3.1	Security requirements

From the analysis in clause 4.2, the following requirements can be derived:

I.	It should be possible to prevent theft of or tampering with the subscription. The following options could be considered:

Option A:	The physical UICC is mechanically attached to the M2M equipment (i.e. the UICC is not physically removable from the M2M equipment): At least one of the following solutions is required:

-	A mechanism to logically bind a UICC with the M2ME, e.g. a device pairing mechanism (the implementation details of this may be out of scope of 3GPP).

-	Additional mechanical protection mechanisms, of which implementation aspects are out of scope of 3GPP.

-	Optionally network based restrictions of services assigned to a subscription (Service Profile).

Option B:	The MCIM application is integrated within the M2M equipment in a trusted environment (TRE) (without a physical UICC). At least one of the following solutions is required:

-	A secure execution environment;

-	A secure storage environment that protects secrets;

-	A mechanism to prevent the loading of unauthorised software on the M2M equipment, both for the case of operating system boot-up (“secure boot”) and for the case of downloaded software that would cause the incorrect execution of the MCIM application;

-	Sufficient physical protection against attacks;

-	Support of at least one authentication algorithm and cryptographic mode of operation, preferably two authentication algorithms and mode of operation

-	Tamper resistance;

-	Secure storage and use of credentials that are established using secure methods (e.g. no plaintext input of credentials over the air);

NOTE:	The network can also restrict the range of services the M2M can use e.g. by setting the M2M subscription to a data-only subscription with a limited low data volume in the network.

-	Optional provision of means of detection and reporting (to a TBD network entity) of evidence of tampering on the MCIM functionality or the trusted environment (TRE) within the M2M equipment that provides such functionality;

-	To fulfil other relevant requirements. Those relevant requirement might be originated from one or several of the following documents OMTP TR0 [5], OMTP TR1 [6], TCG [3], GSMA/EICTA Principles concerning handset theft [7] and other relevant industry standards on prevention against attack.

-	Network based restrictions of services assigned to a subscription (Service Profile)

Option C: Physically removable UICC. At least one of the following solutions is required:

- A mechanism to logically bind a UICC with the M2ME, e.g. a device pairing mechanism (the implementation details may be out of scope of 3GPP)

- Additional mechanical protection mechanisms of which implementation details are out of scope of 3GPP

- Network based restrictions of services assigned to a subscription (Service Profile)

For the integrated MCIM option, it should be possible for the mobile operator to verify the secure execution environment prior to provisioning of the downloadable MCIM application.

-	For the integrated MCIM option, it should be possible to securely initially provision a new MCIM application to the M2M equipment.

-	For the mechanically attached UICC and integrated MCIM options, it may be required to securely change the subscription in the M2M equipment remotely.

-	For the mechanically attached UICC and integrated MCIM options, it may be required to remotely upgrade security credentials, cryptographic contexts, cryptographic algorithms and methods.

-	Data traffic sent or received by an M2M terminal should have the same protection against eavesdropping or modifications as traffic processed by any 3GPP UE.

-	Exposure of subscriber authentication keys to unauthorised 3rd parties would have severe consequences for the GSM and UMTS industry and shall therefore be prevented.

-	Any new security relevant functionality or process shall not jeopardise an operator's ability to fulfil obligations towards regulators and government authorities to guarantee secure authentication and billing.

### 4.3.2	Other Requirements

-	It should be possible to find a viable business model with the technical solution.

-	If the solution has an impact on existing subscriber management business processes, then the benefits of the solution should compensate for any additional cost and complexity of the new processes.

-	If the solution has an impact on existing network infrastructure, then the benefits of the solution should compensate for any additional cost and complexity of the new infrastructure.

-	If the solution has an impact on existing terminal architectures, then the benefits of the solution should compensate for any additional cost and complexity of the new terminal architectures.

-	The MCIM should support a number of lifecycle states (e.g. installed but not activated, activated, suspended).

-	It should be possible to securely update the software and firmware of the M2M equipment OTA.

-	It may be required to prevent the replacement of one operator’s MCIM application with that of another operator without the consent of all parties involved.

-	Appropriate software isolation will be enforced between the secure environment or UICC and the main processing environment of the M2M equipment, and possibly within the secure environment or UICC itself.

-	It should be possible for an operator who has a MCIM application installed in the M2M equipment to configure some aspects of the security policy of the M2M equipment.

-	It should be possible for the MCIM application to be updated OTA.

-	Initializing, bootstrapping, updating, and other related procedures should be automated, scalable, and efficient with respect to message exchanges and number of roundtrips, without compromises on security.

-	Any solution must preserve the ability of an operator to fulfil obligations towards regulators and government authorities to guarantee secure authentication and billing.

************* END OF 1. CHANGE *************