- 3GPP TSG-SA3 (Security)	S3-100587

- SA3#59, 26-30 April 2010, Lisbon, Portugal	revision of S3-100482

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.401 | CR | 0416 |  rev | 1 |  Current version: | 9.3.1 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Correction on key sending in S1 HANDOVER REQUIRED message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code:  | TEI9 |  |  |  |  |  |  | Date:  |  | 08/04/2010 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-9 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | There are two conflicted texts in the current 33.401. In section 7.2.8.4.3, it said that “When an S1-handover is performed and the KASME which is used to compute this {NH,NCC} pair and its corresponding eKSI, the source eNB shall not send any keys to the MME in the S1 HANDOVER REQUIRED message.” It indicates that the source eNB will not send any keys to the MME. But in section 7.4.3 of TS 33.401 which states “But for S1 handover, the target eNB discards the multiple KeNB*s received from the source eNB, and derives the KeNB*s as described in Annex A.5 based on the received fresh {NH, NCC} pair from MME for forward security purpose”. It indicates that the source eNB can send the multiple KeNB* to the target eNB so the target eNB can discard them. According to RAN2’s point of view, it is for the consistence of S1 handover and X2 handover procedures to send multiple keys to the target eNB from the source eNB. So the texts in section 7.2.8.4.3 should be deleted to correct the in consistent texts in 33.401. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Delete the sentence in section 7.2.8.4.3. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | It makes a confusion that the source eNB never send KeNB*s to the MME in the S1 HANDOVER REQUIRED message. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 7.2.8.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

## *********************start first change **********************************

##### 7.2.8.4.3	S1-Handover

Upon reception of the HANDOVER REQUIRED message the source MME shall increase its locally kept NCC value by one and compute a fresh NH from its stored data using the function defined in Annex A.4. The source MME shall store that fresh pair and send it to the target MME in the S10 FORWARD RELOCATION REQUEST message. The S10 FORWARD RELOCATION REQUEST message shall in addition contain the KASME that is currently used to compute {NH, NCC} pairs and its corresponding eKSI.

The target MME shall store locally the {NH, NCC} pair received from the source MME.

The target MME shall then send the received {NH, NCC} pair to the target eNB within the S1 HANDOVER REQUEST.

Upon receipt of the S1 HANDOVER REQUEST from the target MME, the target eNB shall compute the KeNB to be used with the UE by performing the key derivation defined in Annex A.5 with the fresh{NH, NCC} pair in the S1 HANDOVER REQUEST and the target PCI and its frequency EARFCN-DL. The target eNB shall associate the NCC value received from MME with the KeNB. The target eNB shall include the NCC value from the received {NH, NCC} pair into the HO Command to the UE and remove any existing unused stored {NH, NCC} pairs.

NOTE: The source MME may be the same as the target MME in the description in this subclause. If so the single MME performs the roles of both the source and target MME, i.e. the MME calculates and stores the fresh {NH, NCC} pair and sends this to the target eNB.

For S1-handover, the source eNB shall include AS algorithms used in the source cell (ciphering and integrity algorithms) in the source to target transparent container that shall be sent to the target eNB. The AS algorithms used by in the source cell are provided to the target eNB so that it can decipher and integrity verify the RRCReestablishmentComplete message on SRB1 in the potential RRCConnectionRe-establishment procedure.

## **********************end first change **********************************