# 3GPP TSG SA WG3 (Security) Meeting #81	S3-152440

# November 9 – 14, 2015, Anaheim, CA (USA)	revision of S3-152304

Source:	Alcatel-Lucent

Title:	Commenting contribtion on Merging of Logging Events in 5.2.3.6 and 5.2.6.3

Document for:	Discussion/approval

Agenda Item:	7.10

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution:  Commenting contribution on Merge of logging event sections 5.2.3.6 and 5.2.6.3 in TS 33.117.

# Introduction

S3-152304 proposes to merge two sections related to security event logging requirements in TS.33.117. This commenting contribution adds entries for 5 missing entries under the Description heading.

# Proposed pCR

***	BEGIN OF FIRST CHANGE	***

##### 5.2.3.6.1	Security event logging

Requirement Name: Security event logging

Requirement Reference: TBA

Requirement Description: Security relevant events shall be logged with a precise timestamp and a unique system reference. Security events shall be logged together with a unique system reference (e.g. host name, IP or MAC address) and the exact time the incident occurred. For each security event, the log entry shall include user name, timestamp, performed action and result.

IETF RFC 3871 [4], section 2.11.10 specifies the minimum set of security events. Each vendor shall document what security events that the product logs so it can be verified by testing.

Typical event that reasonably should be logged in many cases are:

| Event |  | Description |  | Event data to be logged |  |
| --- | --- | --- | --- | --- | --- |
| Incorrect login attempts |  | Records any user incorrect login attempts to the network product |  | • Account, • Source (IP address) of remote access |  |
| System access with accounts with administrator rights |  | Records any access attempts to accounts that have system privileges. |  | • Account, • Access timestamp, • Length of session, • Source (IP address) of remote access |  |
| Account administration |  | Records all account administration activity. |  | • Administrator account, • Administered account, • Activity performed (configure, delete, enable and disable) |  |
| Change of group membership for accounts |  | Records any group membership administrative activities including changes to group membership. |  | • Administrator account, • Administered account, • Activity performed (group added or removed) |  |
| Critical rise in system values such as disk space, CPU load over a longer period |  | Records events that have been triggered when system parameter values have exceeded their defined thresholds. |  | • Value exceeded, • Value reached (Here suitable threshold values shall be defined depending on the individual system.) |  |
| Configuration change | Changes to configuration of the network device |  | • Change made • User |  |
| Reboot/shutdown/crash | This event records any action on the network device that forces a reboot or shutdown OR where the network device has crashed. |  | • Action performed (reboot, shutdown, etc.) • User (for intentional actions) |  |
| Interface status change | Change to the status of interfaces on the network device (e.g. shutdown) |  | • Interface name and type • Status (shutdown, missing link, etc.) |  |

Threat References: TBA

Security Objective references: tba.

Test case: TBA

***	NEXT CHANGES	***

#### 5.2.6.3 	VOID

##### 5.2.6.3.1	VOID

***	END OF CHANGES	***