3GPP TSG SA WG3 (Security) Meeting #81	S3-152571

9 – 13 November, 2015, Anaheim, USA	                                               revision of S3-152367

Source:	Nokia Networks, Vodafone

Title:	pCR to 33.863 - Introducing key server for solution#2

Document for:	Discussion/approval

Agenda Item:	8.4 - CIoT BEST

Work Item / Release:	Rel-13

Abstract of the contribution:

This contribution proposes introducing an end-to-middle key server EMKS for solution#2, to which the HSS would push an end-to-middle intermediate key at the same time the HSS generates and returns an authentication vector. The end-to-middle key used for protecting data between UE and EMSE is then derived from the end-to-middle intermediate key. The advantage of this approach is that the HSS can remain stateless. The approach was discussed in a conf call on BEST.

A separate pCR by Vodafone in S3-152247 introduces the concept of EMKS in solution#1.

# 1 Proposed pCR to TS 33.863 v 0.3.0

***	BEGIN OF CHANGES	***

## 6.2	Solution #2: “End-to-middle security based on AKA”

### 6.2.1	Introduction

The present solution builds on solution#1. It is presented as a delta description to solution#1, i.e. the description of the present solution contains only those elements that differ from solution#1.

Solution#1 is explicitly meant for “mobile network with a Cellular Internet-of-Things RAT”. The present solution can be used for CIoT as well (which is why it is included in the present TR). It should be noted, though, that there is no technical reason why the present solution should be restricted to the CIoT use case; it can provide e2m security whenever AKA is run in a mobile network.

While solution#1terminates the additional user plane protection in a HPLMN Security Endpoint (HSE) the present solution is applicable to any endpoint, called “E2M Security Endpoint” (EMSE), that is authorised to obtain e2m keys from the E2M Key Server (EMKS). The EMKS derives an e2m key from an intermediate key (e2m_int_key) the EMKS obtains from the HSS. .

Editor’s Note: the exact placement of the EMSE in the authentication path has to be defined.

An EMSE may reside in an operator’s home network, but also in a trusted third party domain (e.g. a CIoT application provider platform). The EMKS always resides in an operator’s home network.

There may be multiple instances of an EMSE, with which a UE engages simultaneously or sequentially. There is typically only one instance of an EMKS per HSS.

As usual in 3GPP standardisation, EMSE and EMKS denote functional entities that could be implemented stand-alone or combined with HSS or GGSN or P-GW. EMSE and EMKS could also be combined. The advantage of introducing the EMKS is that the HSS could continue to operate in a stateless fashion.

### 6.2.2	Solution description

#### 6.2.2.1	End-to-middle security solution based on AKA

Transparency for core network nodes: 
SGSNs, GGSNs, MMEs, S-GWs, P-GWs may work with the present solution without any CIoT-specific enhancements. This is not to say that such enhancements would not be useful for certain use cases,  but it means that there is no mandatory pre-condition on the UMTS or EPS core network elements. This transparency of the solution for the core network is important for deployments as many operators may be reluctant to upgrade their core networks for the purpose of supporting CIoT. (To be sure, the HSS and HLR would need CIoT-specific enhancements.)

In contrast to the present solution, solution#1 puts the following pre-condition: “The SGSN/MME has indicated the supported security configuration of the GERAN/E-UTRAN to the H-PLMN…”. Imposing such a pre-condition on access security reporting may be unnecessary for certain use cases as there is enhanced user plane protection anyhow provided by e2m security. Furthermore, vulnerabilities to attacks in the signalling plane may be reduced through the particularities of the use case (e.g. meters and sensors that mostly have stationary UEs and UE-initiated traffic) and through the enforcement of minimum security requirements by the UE.

Transparency for the USIM: 
The possibility to re-use USIMs without CIoT-specific additions is important as particular CIoT use cases may not warrant the modification of USIMs for their purposes. The present solution can re-use existing USIMs. CIoT specific operations can be done in the ME.

In contrast to the present solution, solution#1 has the following text: “USIM … performs the following additional key derivations: - CK´/IK´…” and again “The key derivation function should be supported by the UICC…”. These key derivations on the USIM are CIoT-specific.

Editor’s note: It is ffs whether the key derivation as in the preceding sentence could be done in the ME also for solution#1, taking into account security aspects. If the description of solution#1 was modified accordingly then the preceding sentence could be deleted.

Push vs. pull procedures

The present solution can work with any of the push or pull procedure variants from solution#1 that are used to transport the e2m key from the EMKS to the EMSE. For transporting the intermediate key e2m_int_key from the HSS to the EMKS, only a push procedure is used.

#### 6.2.2.2	Key derivation rules

As for solution#1, the present solution uses a new key pair CK´, IK´for UMTS PS or GPRS access security between UE and SGSN. For EPS, KASME can be re-used as defined today.

The present solution differs from solution#1 in the derivation of the additional user plane key. In solution#1, this key is called “e2mKEYSET”. It is used between UE and HSE. In the present solution, this key is called “e2m_key”. It is used between UE and EMSE.

The e2m key is derived in two steps:

(1) Whenever an Authentication Information Request from the SGSN or the MME arrives at the HSS, then the HSS checks the subscription profile for the need to derive an e2m_int_key. The HSS then generates authentication vectors and sends them back to the SGSN or MME. If needed according to the subscription profile, the HSS derives e2m_int_key from CK, IK and a string pointing to the purpose of the key use, namely e2m security for CIoT, where the derivation of e2m_int_key has the form

e2m_int_key = KDF (CK||IK, string)

and the string could be set to e.g. "e2m_ CIoT". The HSS sends e2m_int_key to the EMKS.

(2) The EMKS derives e2m_key from e2m_int_key. As there may be multiple instances of EMSE it becomes necessary to include an identifier of the EMSE in the key derivation in order to achieve key separation between EMSEs. We therefore propose the following key derivation rule:

e2m_key = KDF (e2m_int_key, EMSE_Id, string)

Editor's Note: it is ffs whether the additional input 'string' is needed in the derivation of e2m_key. It may be appropriate to allocate FC values for the purpose of the two key derivations in steps 1) and 2) according to TS 33.220, B.2.2.

#### 6.2.2.3	Usage of e2m security

Solution#1 mentions the use of a, yet to be specified, bit in the Authentication Management Field (AMF) to indicate to the UE whether the UE has to derive the keys CK´ and IK´ for access security. The information on whether this specific AMF-bit is to be set or not needs to be fed into the Authentication Centre. How this information is stored and fed into the AuC does not need to be standardised from an interoperability point of view. However, it may be beneficial to do so for HSS or HLR vendors as standardisation would reduce the number of implementation variants requested by customers.

As the information on whether this specific AMF-bit is to be set is subscriber-specific it should be stored in the subscriber records. However, it is not part of the subscriber profile that is sent to the serving node.

NOTE: The use of an AMF bit, as described above, is still open. If used it would imply that there are two kinds of AKA runs: (1) AMF bit set: access security keys for use between UE and SGSN and e2m keys for use between UE and EMSE are derived; (2) AMF bit not set: only access keys are derived. The third case, namely an AKA run that establishes ONLY e2m keys, should not arise as it would necessitate significant changes to the security as defined today between UE and SGSN: the SGSN would have to support running an AKA without deriving access keys, and, in particular, the SGSN would have to interpret the AMF bit. Cf., however, solution#3 where a different approach is taken to decouple AKA runs for access security from AKA runs for e2m security.

### 6.2.3	Solution evaluation

Editor’s note: FFS

***	END OF CHANGES	***