3GPP TSG SA WG3 (Security) Meeting #84	S3-161071

25-29 July 2016, Chennai (India)		revision of S3-13abcd

Source:	Nokia

Title:	Key issue on privacy protection of network slice identifier

Document for:	Approval

Agenda Item:	8.6

Work Item / Release:	FS_NSA / Rel-14

Abstract of the contribution:

#### Discussion

A new key issue #7.X on “Privacy protection of network slice identifier” is proposed under security area #7 “subscriber identifier privacy”.

pCR

<< editor: text is new >>

#### 5.7.3.5	Key issue #7.X: Privacy protection of network slice identifier

##### 5.7.3.5.1	Key issue details

NextGen architecture introduces the concept of network slices. A slice will contain core network and RAN elements.

In order for the user to choose and to attach to a slice, an identification will be needed. The UE or the provider of the slice may use such slice identifier to select the relevant functions within a certain network slice. One option discussed in 3GPP TR 23.799 [X] is the usage of a multi-dimensional descriptor (MDD) for a network slice.

Based on such network slice identification and on other information (e.g. subscription) available in the network, the UE can use different network slices depending on the type of application and service it requires.

##### 5.7.3.5.2	Security and privacy threats

If the slice identifier can be connected to the subscriber identity, it is possible to identify a group of users that use the same slice, i.e. they belong to the same interest group, e.g. a group of police men or firefighters in MCPTT. If an attacker wants to deny service for a particular user group, the connection between subscriber identity and network slice identification may lead to such attack.

Note: Similar threats as in clause 5.7.3.2.2 also apply.

##### 5.7.3.5.3	Potential security requirements

Ed.note: It is ffs whether for privacy purposes the identifier of a network slice may be temporary.

The identifier of a network slice may be confidentially protected.

It shall be not possible by an unauthorized party to correlate a network slice identifier with user identifiers.