3GPP TSG SA WG3 (Security) Meeting #84                                                        S3-161125

25-29 July 2016, Chennai, India

Source:	China Mobile, Huawei, Hisilicon

Title:	pCR choice of authentication methods

Document for:	Discussion/approval

Agenda Item:	8.6

Work Item / Release:	Architecture and Security For Next Generation System / Rel-14

Abstract of the contribution: This pCR identifies the need to agree upon an authentication method between UE and the network. The related security threats and potential security requirements are given.

Introduction

This contribution identifies that a UE and the network shall agree upon an authentication method before starting the authentication procedure, because a UE is allowed to use a variety of authentication methods to access the 5G network.

Proposed pCR

***	BEGIN OF THE FIRST CHANGE	***

5.2.3.x 	Key Issue #2.8:  Choice of authentication methods

5.2.3.x.1 	Key issue details

As stated in Section 5.2.1 and 5.2.2 of TR.33.899, alternative methods may be deployed in the next generation network for authentication besides AKA methods.  Thus a UE and the network have to determine which kind of authentication methods shall be used before performing authentication.

5.2.3.x.2 	Security threats

Since the choice of authentication methods is performed before the UE accessing to the network, there are not any security protections to this procedure.  An attacker therefore could tamper with the authentication methods supported and type of service.   The chosen authentication method cannot meet the service requirements. Consequently the UE cannot access to the network normally.

5.2.3.x.3	Potential security requirements

The 3GPP system shall support the choice of authentication methods.

The messages exchanged between a UE and the network for the choice of authentication methods shall be integrity protected.

***	END OF THE FIRST CHANGES	***