# 3GPP TSG SA WG3 (Security) Meeting #88	S3-171817

# 7-11 Aug, 2017, Dali, China	revision of S3-17xabc

# Source:	Alibaba (China) Group., Ltd.

# Title:	Updating security threats, requirements and solution on key issue #2.8

# Document for:	Approval

Agenda Item:	8.3.2   FS_NSA

1	Decision/action requested

It is kindly proposed to approve the pCR below

2	References

[1]            3GPP TR 33.899-120

[2]            ITU-T X.eaasd: ”Framework of enhanced authentication in telebiometric environments using anti-spoofing detection mechanisms”; https://www.itu.int/ITU-T/workprog/wp_item.aspx?isn=13593

3		Rationale

In addition to the security threats listed in TR 33.899 for key issue#2.8, there are other security threats.

If an attacker register or authenticate with fake biometric information to a service, then it may lead to fraud, for example, using other users’ biometric information may transfer the bill to that other user; using of artificial biometric information (gummy finger, video of face) may lead to economic fraud and can not trace the source. So there is a potencial security requirement that whether the user is the person he claims.

Solution #2.3 and solution #2.4 cannot meet the above reqirement. There should be an enhanced authentication framework using anti-spoofing detection mechanisms. Anti-spoofing detection mechanisms are based on remote authentication, and can determine that the user who sends the biometric authentication request is the active person who owns the biometric data. This can avoid the situation that illegal user may use fake or copied biometric data for biometric authentication.

4	pCR

*******************START OF CHANGES 1*******************************************

5.2.3.8.2 	Security threats

A device may be used by an attacker other than an intended user, compromising privacy of user data and/or integrity of data reported by the device (e.g. the user’s medical information may be contaminated with data pertaining to the attacker). If the same device has multiple users, then these users may be able to compromise each other’s privacy and/or the integrity of each other’s data.  If actions are billed on a per-user basis, then fraudulent usage may occur (by pretending to be a different user of the device, the fraudulent user transfers the bill to that other user).

An attacker may learn or compromise user authentication information (PINs, biometrics etc.) from a device or from a server. This may lead to loss of user privacy and authentication fraud. Use of a central server may lead to a mass compromise of multiple users’ information at once. If an attacker register or authenticate with fake biometric information to a service, then it may lead to fraud, e.g. using other users’ biometric information may transfer the bill to that other user; using of artificial (gummy finger, video of face) biometric information may lead to economic fraud and can not trace the source.

User privacy may be compromised if the user is authenticated using the same credentials toward multiple services and relying parties. Use of a common credential may allow the user’s actions across services to be linked together, and so create pervasive tracking of the user beyond the user’s awareness and consent.

Where IoT devices are authenticated as a group, an attacker may get his own IoT device registered as part of another user’s group, or may take control of a device which is part of some other user’s group. The actions of the attacker would then be attributed (e.g. billed) to the victim user.

*******************END OF CHANGES 1*******************************************

*******************START OF CHANGES 2*******************************************

5.2.3.8.3 	Potential security requirements

It shall be possible for devices to securely authenticate a single registered user, or any one of a set of registered users.

For any given device, it shall be possible for a mobile network and/or an IoT service to determine: whether a registered user has been authenticated; which user has been authenticated (if there are several registered users); the means that were used to authenticate the user (e.g. was it PIN, password or biometric); whether the user is the person he claims; and/or other factors related to the reliability of authentication (e.g. how recently was this done, were there retries?)

*******************END OF CHANGES 2*******************************************

*******************START OF CHANGES 3*******************************************

5.2.4.z	Solution #2.z: Enhanced authentication framework for the user authentication

5.2.4.z.1	Introduction

This solution addresses key issue #2.8.

The proposed solution is to provide an enhanced authentication framework using anti-spoofing detection (ASD) mechanisms for the user authentication.  The remote server authenticates the user, and then securely reports (to the mobile network, service provider etc.) which user has been authenticated and how it was done.

5.2.4.z.2	Solution details

Several specifications for this solution have been developed by ITU-T [2]. Some key points are as follows:

- ASD aims to determine that the user who sends the biometric authentication request is the active person who owns the telebiometric data. Liveness detection can be used together with biometric verification to enhance security, and avoid fake biometric situations. For example, an application may ask user to nod, shake heads, blink, open mouth etc., to verify that the user is the actual person who made the authentication request.

- ASD functionality can be designed in the trusted execution environment in the device to enhance security and avoid the biometric data leakage.

- Biometric information must be desensitized and encrypted before transmission, and the eigenvalues cannot be reduced to the original biometric information to protect users’ privacy information.

5.2.4.z.3	Evaluation

The solution meets the following requirements listed in 5.2.3.8.3:

"It shall be possible for devices to securely authenticate a single registered user, or any one of a set of registered users."

"For any given device, it shall be possible for a mobile network and/or an IoT service to determine: whether a registered user has been authenticated; which user has been authenticated (if there are several registered users); the means that were used to authenticate the user (e.g. was it PIN, password or biometric); whether the user is the person he claims; and/or other factors related to the reliability of authentication (e.g. how recently was this done, were there retries?)"

"Mass compromise of many users’ data shall be prevented."

"Tracking of a user across different services without the user’s consent shall be prevented."

"Sensitive user authentication data (especially biometric information) shall be stored in a way which makes compromise very difficult."

*******************END OF CHANGES 3*******************************************