3GPP TSG-SA WG3 Meeting #91Bis 	S3-181685

# La Jolla (US), 21-25 May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0156 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cluase 6.3.1.2 on unused AVs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  |  | Date: |  | 2018-04-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - It has been agreed in SA3#91, that unused AVs are not transferred between two SEAFs. But it is possible that a legacy MME might send unused AVs in mobility scenarios, to an SEAF/AMF during context transfer. In this scenario, clarification is needed that SEAF/AMF is not expected to process these AVs. Rather this AV will be simply dropped, if received by the SEAF/AMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add text in 6.3.1.2 that If SEAF recives any unused authentication vectors (e.g. in mobility scenarios from legacy MME) they shall be dropped without any processing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Undefined behavior for SEAF processing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# *** BEGINNING OF CHANGE ***

## 6.3	Security contexts

### 6.3.1	Distribution of security contexts

#### 6.3.1.1	General

The present clause focuses on the security contexts themselves; the handling of security contexts in mobility procedures is described in clause 6.5.

#### 6.3.1.2	Distribution of subscriber identities and security data within one 5G serving network domain

The transmission of the following subscriber identities and security data is permitted between 5G core network entities of the same serving network domain:

-	SUPI in the clear

-	5G security contexts, as described in clause 6.5

Unused 5G authentication vectors shall not be transmitted between SEAFs. If SEAF recives any unused authentication vectors (e.g. in mobility scenarios from legacy MME) they shall be dropped without any processing.

Once the subscriber identities and security data have been transmitted from an old to a new network entity the old network entity shall delete the data.

#### 6.3.1.3	Distribution of subscriber identities and security data between 5G serving network domains

The transmission of the following subscriber identities and security data is permitted between 5G core network entities of different serving network domains:

-	SUPI in the clear

-	5G security contexts, as described in clause 6.5, if the security policy of the transmitting 5G serving network domain allows this.

Unused 5G authentication vectors or non-current 5G security contexts shall not be transmitted to a different 5G serving network domain.

#### 6.3.1.4	Distribution of subscriber identities and security data between 5G and EPS serving network domains

NOTE 1: 	No direct interworking between 5G networks and network of generations prior to EPS are foreseen. Therefore, only the interaction between 5G and EPS serving network domains is addressed here.

The transmission of the SUPI in the clear is permitted between 5G and EPS core network entities if it has the form of an IMSI.

The transmission of any unmodified 5G security contexts to a EPS core network entity is not permitted. Details of security context transfer between EPS and 5G core network entities can be found in clause 9.

The transmission of any unused 5G authentication vectors to a EPS core network entity is not permitted. The transmission of any unused EPS authentication vectors to a 5G core network entity is not permitted.

NOTE 2: 	The rules above differ from the corresponding rules in 3GPP TS 33.401, clause 6.1.6: The latter allows forwarding of UMTS authentication vectors from an SGSN to an MME and back to the same SGSN under certain conditions. But this feature goes against a strict security separation of EPS and 5G domains. As its performance advantage is questionable it was not copied into 5G.

NOTE 3: 	Security context mapping between EPS and 5G serving networks is allowed, according to clause 8.

### 6.3.2	Multiple registrations in same or different serving networks

#### 6.3.2.0	General

There are two cases where the UE can be multiple registered in different PLMN's serving networks or in the same PLMN's serving networks. The first case is when the UE is registered in one PLMN serving network over a certain type of access (e.g. 3GPP) and is registered to another PLMN serving network over the other type of access (e.g. non-3GPP). The second case is where the UE is registered in the same AMF in the same PLMN serving network over both 3GPP and non-3GPP accesses. The UE will establish two NAS connections with the network in both cases.

NOTE: 	The UE belongs to a single HPLMN.

#### 6.3.2.1	Multiple registrations in different PLMNs

The UE shall independently maintain and use two different 5G security contexts, one per PLMN's serving network. Each security context shall be established separately via a successful primary authentication procedure with the Home PLMN.

The ME shall store the two different 5G security contexts on the USIM if the USIM supports the 5G parameters storage. If the USIM does not support the 5G parameters storage, then the ME shall store the two different 5G security contexts in the ME non-volatile memory. Both of the two different 5G security contexts are current 5G security context.

Editor's Note: It is FFS to define the event(s) that triggers the storage of the key in the ME or in the USIM. Also, the appropriate clause needs to be considered.

#### 6.3.2.2	Multiple registrations in the same PLMN

When the UE is registered in the same AMF in the same PLMN serving network over both 3GPP and non-3GPP accesses, the UE shall establish two NAS connections with the network. Upon receiving the registration request message, the AMF should check whether the UE is authenticated by the network. The AMF may decide to skip a new authentication run in case there is an available 5G security context for this UE by means of 5G-GUTI, e.g. when the UE successfully registered to 3GPP access, if the UE registers the same AMF via non-3GPP access, the AMF can decide not to run a new authentication if it has an available security context to use. The AMF and the UE shall establish a common NAS security context consisting of a single set of NAS keys and algorithm at the time of first registration over any access. The AMF and the UE shall also include parameters specific to each NAS connection in the common NAS security context. The connection specific parameters are specified in clause 6.4.2.2 of the present document.

Editor's Note: Above requirements needs to be revisited after studying concurrency, mobility and interworking use cases.

# *** END OF CHANGES ***