3GPP TSG-SA WG3 Meeting #91BIS 	S3-181959

# La Jolla (US), 21-25 MAY 2018														revision of S3-181792

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0196 | rev | 1 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to section 13.4.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  |  | Date: |  | 2018-05-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the section 13.4.1.1, there is a sentence” - The NF Service producer verifies the integrity of the token by checking the signature using the shared secret or NRF’s public key. “ The signature can not be checked by using the shared secret, and the shared secret can be only used for checking MAC value. So the sentence should be revised as follows: “- The NF Service producer verifies the integrity of the token by verifying the signature using NRF’s public key or checking MAC value using the shared secret. “ |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes the following correction: The NF Service producer verifies the integrity of the token by verifying the signature using NRF’s public key or checking the MAC value using the shared secret. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An error is still there. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.4.1.1（Service access request based on token verification） |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGES ****

13.4.1.1	Service access authorization within the PLMN

OAuth 2.0 roles, as defined in clause 1.1 of [43], are as follows:

a.	The Network Resource Function (NRF) shall be  the OAuth 2.0 Authorization server.

b.	The NF service consumer shall be the OAuth 2.0 client.

c.	The NF service producer shall be the OAuth 2.0 resource server.

OAuth 2.0 client (NF service consumer) registration with the OAuth 2.0 authorization server (NRF)

The NF service registration procedure, as defined in clause 4.17.1 of TS 23.502 [8], shall be used to register the OAuth 2.0 client (NF service consumer) with the OAuth 2.0 Authorization server (NRF), as described in clause 2.0 of [x]. The client id, used during OAuth 2.0 registration, shall be the NF Instance Id of the NF.

Access token request during NF service discovery

The NF service discovery procedure between NFs and NRF in the same PLMN as defined in clause 4.17.4 of TS 23.502 [8]. It may be used as the underlying procedure by the NF service consumer to obtain access token from the OAuth 2.0 server (NRF).

Figure 13.4.1.1-1: NF service consumer obtaining access token during NF service discovery

Pre-requisite:

a.	The NF Service consumer is registered with the NRF (Authorization Server)

b.	The NRF and NF service producer share the required credentials when token validation is performed by the NF service producer.

1.	The NF service consumer invokes Nnrf_NFDiscovery_Request (Expected NF service Name, NF Type of the expected NF instance, NF type of the NF consumer) from NRF in the same PLMN. As an OAuth 2.0 client the NF service consumer also shall send its client id (i.e. NF Instance Id) in the request message as described in clause 4.4.2 of [43].

Editor’s Note: Whether per UE subscription level authorization in NRF is needed is FFS.

For authorization at subscription level, the NF service consumer shall also include UE related information in the request.

2.	After successful authentication of the client and authorization of the discovery, the NRF decides whether the NF service consumer can be authorized based on subscription data.

3.	If the NRF decides that NF service consumer can be authorized, the NRF shall generate an access token with appropriate claims included. The NRF shall digitally sign the generated access token based on a shared secret or private key as described in [45].

The claims in the token shall include the NF Instance Id of NRF (issuer), NF Instance Id of the NF Service consumer (subject), NF Instance Id of the NF Service Producer (audience), authorized services (scope) and expiration time (expiration).

Editor’s Note: Additional claims in the token are FFS.

4. The access token is included in Nnrf_NFDiscovery_Request Response message.

Access token request independently before service access

The NF service consumer may also obtain an access token before service access using the following procedure. Pre-requisite of this procedure is the same as in obtaining access token during NF service discovery.

Figure 13.4.1.1-2: NF service consumer obtaining access token before NF service access

1. The NF service consumer requests an access token from the NRF. It includes the client id, expected NF service name, NF types of the expected NF instance and NF consumer. The service consumer may also include UE related information.

2-3. Steps 2-3 from the “Access token request during NF service discovery" procedure.

4. The token is included in the response sent to the NF service consumer.

Service access request based on token verification

The following figure and procedure describe how authorization is performed during Service request of the NF service consumer.

Figure 13.4.1.1-3: NF service consumer requesting service access with an access token

Pre-requisite: The NF service consumer is in possession of a valid access token before requesting service access from the NF Service producer.

1.	The NF Service consumer requests service from the NF service producer. The NF Service Consumer includes the access token.

Editor’s Note: Additional parameters needed in step 1 are based on the included claims in the token and are FFS.

The NF Service consumer and NF service producer authenticate each other following clause 13.3.

2.	The NF Service producer verifies the token in one of the following ways:

-	Request the NRF to verify the token. The NRF verifies the token and responds back to the NF service producer.

-	- The NF Service producer verifies the integrity of the token by verifying the signature using NRF’s public key or checking the MAC value using the shared secret. If integrity check is successful, the NF Service producer verifies the claims in the token.

Editor’s Note: Details of verification of claims in the token are FFS.

3.	If the verification is successful, the NF Service producer executes the requested service and responds back to the NF Service producer.

**** End of Changes ****