3GPP TSG-SA WG3 Meeting #93	 		S3-183437

Spokane (US) 12-16 November 2018

Title:	DRAFT Reply LS on security requirements for Integrity protection for DRBs in MR-DC

Response to:	S3-183281/R2-1816054

Release:	Rel-15

Work Item:	5GS_Ph1-SEC

Source:	Huawei to be SA3

To:	RAN2

Cc:

Contact Person:

Name:	Ahmad Muhanna

Tel. Number:	+1-972-814-8203

E-mail Address:	ahmad dot muhanna at huawei dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA3 has discussed RAN2 LS on “on security requirements for Integrity protection for DRBs in MR-DC” and would like to provide answers to RAN2 questions. SA3 would like to indicate that UP integrity protection is always based on UP security policy received from the SMF. With that in mind, SA3 would like to provide the following answers;

Q1: Which of the bearer types (MN and SN terminated bearers) need integrity protection for each of NE-DC, NR-DC and NG-EN-DC architectures?

SA3 Answer to Q1:

- For the case of NE-DC:

  - MN terminated can have UP integrity protection activation on. However, in this case, the MN shall not at any point offload such PDU session to the SN.

  - SN terminated PDU sessions always have the UP integrity protection activation as “off”.

- For the case of NR-DC:

  - MN and SN terminated PDU session can have UP integrity protection activation either on or off.

- For the case of NG-EN-DC:

  - In release 15, neither the MN terminated nor the SN terminated PDU session can have the UP integrity protection activation as “on”.

SA3 would like to inform RAN2 that in later releases, SA3 expects ng-eNB connected to 5GC to support UP integrity protection.

Q2: When the policy for PDU session is set to “DRB IP preferred”, is it required to activate integrity protection for all MN and SN terminated bearers of the PDU session?  Or can DRB IP be activated differently for MN terminated and SN terminated bearers?

SA3 Answer to Q2:

In NE-DC and NG-EN-DC scenarios, the answers to Q1 above is still applicable and valid. In other words, all DRBs for the same PDU session will have the same UP IP activation.

However, in the case of NR-DC, for the same PDU session, DRBs which are terminated at the MN may have different UP IP activation other than those DRBs which are terminated at the SN.

NOTE: If SA3 agrees to allow SN (gNB) in the case of NG-EN-DC to optionally activate UP integrity protection, then the above answer for NG-EN-DC case needs to be updated accordingly.

2. Actions:

To RAN2 group.

ACTION: SA3 kindly requests RAN2 to take the above answers into account and let SA3 know if RAN2 still needs any clarification.

3. Date of Next TSG-SA WG3 Meetings:

SA3#94	                28 Jan. – 1 Feb. 2019			Kochi, India

SA3#95	                6-10 May 2019			TBD