# 3GPP TSG SA WG3 (Security) Meeting #93	S3-183481

# 12-16 November 2018, Spokane (US)	revision of

# Source:	China Mobile, Vodafone

# Title:	Key derivation function in TR 33.841

# Document for:	Approval

Agenda Item:	8.3

1	Decision/action requested

The pCR regarding the key derivation function is kindly asked to be approved by SA3.

2	Reference

[1]		TR 33.841, Study on the support of 256-bit algorithms for 5G

3	Rationale

Current version of TR 33.841 lacks a survey of  key derivation functions used in 5G system. There still exists an Editor’s note : It is FFS how the key derivation function is affected by advances in quantum computing. This pCR provides a brief review on the key derivation functions used in 5G system, and removes the editor’s note for the reason that hash functions used in the key derivation functions are less affected by the advances of quantum computers.

4	pCR

**** Begin of first change ****

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 		Markus Grassl, Brandon Langenberg, Martin Roettler, Rainer Steinwandt (2015); "Applying Grover’s algorithm to AES: quantum resource estimates". https://arxiv.org/pdf/1512.04965.pdf

[3] 		Vlad Gheorghiu, Michele Mosca (2017); "GRI quantum risk assessment report: A resource estimation framework for quantum attacks against cryptographic functions". http://globalriskinstitute.org/download/summary-report-3/

[4] 					Scott Fluhrer (2017); "Reassessing Grover’s Algorithm". https://eprint.iacr.org/2017/811.pdf

[5] 	Matthew Amy, Olivia Di Matteo, Vlad Gheorghiu, Michele Mosca, Alex Parent, John Schanck (2016); "Estimating the cost of generic quantum pre-image attacks on SHA-2 and SHA-3". https://arxiv.org/pdf/1603.09383.pdf

[6] 	Daniel J. Bernstein (2009); "Cost analysis of hash collisions: Will quantum computers make SHARCS obsolete?" https://cr.yp.to/hash/collisioncost-20090517.pdf

[7] 		NISTIR 8105 (2016); "Report on Post-Quantum Cryptography". https://csrc.nist.gov/publications/detail/nistir/8105/final

[8] 		John M. Martinis (2015); "Qubit metrology for building a fault-tolerant quantum computer".  https://www.nature.com/articles/npjqi20155.pdf

[9] 		Martin Roetteler, Michael Naehrig, Krysta M. Svore, and Kristin Lauter, "Quantum Resource Estimates for Computing Elliptic Curve Discrete Logarithms”. In Proc. ASIACRYPT 2017. https://www.microsoft.com/en-us/research/wp-content/uploads/2017/09/1706.06752.pdf

[10] 		Austin G. Fowler, Matteo Mariantoni, John M. Martinis, and Andrew N. Cleland, "Surface codes: Towards practical large-scale quantum computation”. October 26, 2012. https://web.physics.ucsb.edu/~martinisgroup/papers/Fowler2012.pdf

[11]		Markus Grassl, Brandon Langenberg, Martin Roetteler, and Rainer Steinwandt, "Applying Grover’s algorithm to AES: quantum resource estimates”. 
https://www.microsoft.com/en-us/research/wp-content/uploads/2016/04/1512.04965-1.pdf

[12] 		MIT Technology Review, Practical Quantum Computers. https://www.technologyreview.com/s/603495/10-breakthrough-technologies-2017-practical-quantum-computers/

[13]		Thomas Häner, Martin Roetteler, and Krysta M. Svore. "Factoring using 2n+2 qubits with Toffoli based modular multiplication". Quantum Information and Computation, 18(7&8):673–684, 2017

[14A]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF) "

[15B]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security"

[16C]	3GPP TS 33.501: "Security architecture and procedures for 5G system"

[17D]	3GPP TR 33.834 v0.4.0: "Study of Long Term Key Update Procedure"

[18E]	IETF RFC 7165: "Use Cases and Requirements for JSON Object Signing and Encryption (JOSE) "

[19F]	IETF RFC 7515: "JSON Web Signature (JWS)"

[20G]	IETF RFC 7516: "JSON Web Encryption (JWE)"

[21H]	IETF RFC 7517: "JSON Web Key (JWK)"

[22I]	IETF RFC 7518: "JSON Web Algorithms (JWA)"

[23J]	IETF RFC 7519: "JSON Web Tokens (JWT)"

[24K]	3GPP TS 33.117: "Catalogue of general security assurance requirements"

[25] 		NIST Special Publication 800-38B: "Recommendation for Block Cipher Modes of Operation: The CMAC Mode for Authentication"

[26]		NIST project on Post-Quantum Cryptography

https://csrc.nist.gov/Projects/Post-Quantum-Cryptography

[27]		SOG-IS Crypto Evaluation Scheme "Agreed Cryptographic Mechanisms"

https://www.sogis.org/documents/cc/crypto/SOGIS-Agreed-Cryptographic-Mechanisms-1.1.pdf

[28]	Dustin Moody (2018), "Let’s Get Ready to Rumble - The NIST PQC 'Competition'" https://csrc.nist.gov/CSRC/media/Presentations/Let-s-Get-Ready-to-Rumble-The-NIST-PQC-	Competiti/images-media/PQCrypto-April2018_Moody.pdf

[29]	NIST (2016), "Submission Requirements and Evaluation Criteria for the Post-Quantum Cryptography Standardization Process" https://csrc.nist.gov/csrc/media/projects/post-quantum-cryptography/documents/call-for-proposals-final-dec-2016.pdf

[30]	Mihir Bellare, Björn Tackmann (2017); "The Multi-User Security of Authenticated Encryption: AES-GCM in TLS 1.3". https://eprint.iacr.org/2016/564.pdf

[31]	IETF Internet draft draft-mcgrew-iv-gen: "Generation of Deterministic Initialization Vectors (IVs) and Nonces".

[32]	Report ITU-R M.2410-0, "Minimum requirements related to technical performance for IMT-2020	radio interface(s)", 2017.  https://www.itu.int/pub/R-REP-M.2410-2017

[33]	IEEE 802.15.4-2011, "IEEE Standard for Local and metropolitan area networks--Part 15.4: Low Rate Wireless Personal Area Networks (LR-WPANs)", 2011	https://standards.ieee.org/standard/802_15_4-2011.html

[aa]	3GPP TS 33.220, Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)

[bb]	RFC 5689, HMAC-based Extract-and-Expand Key Derivation Function (HKDF)

[cc]	RFC 2104, HMAC: Keyed-Hashing for Message Authentication

**** End of first change ****

**** Begin of second change ****

3	Definitions abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AKA	Authentication and Key Agreement

CK	Ciphering Key

DHE	Diffie-Hellman Ephemeral

EAP	Extensible Authentication Protocol

ECDHE	Elliptic Curve Diffie-Hellman Ephemeral

ECIES	Elliptic Curve Integrated Encryption Scheme

GKDF	Generic Key Derivation Function

HKDF	HMAC-based Extract-and-Expand Key Derivation Function

HN	Home Network

IKEv2	Internet Key Exchange version 2

IPX	IP Exchange

JSON	JavaScript Object Notation

KDF	Key Derivation Function

NAS	Non Access Stratum

NDS	Network Domain Security

OTA	Over The Air

RES	RESponse

RRC	Radio Resource Control

SBA	Service Based Architecture

SEAF	Security Anchor Function

SEPP	Security Edge Protection Proxy

SUCI	SUbscription Concealed Identifier

SUPI	SUbscription Permanent Identifier

TLS	Transport Layer Security

UE	User Equipment

**** End of second change ****

**** Begin of third change ****

6.3.2 Key derivation function

There are two kinds of key derivation functions deployed in the 5G system. One is generic key derivation function (GKDF) defined in TS 33.220[aa], which is used for all key derivations in 5GC as shown in Figure 6.2.2-1 and Figure 6.2.2.-2 in TS 33.501. The other one is the HMAC-based Extract-and-Expand Key Derivation Function (HKDF) specified in RFC 5869[bb], which has been applied to IKEv2, TLS 1.2, TLS 1.3, N32-f, and EAP-AKA’ for the key derivation.

The base of GKDF and HKDF is the keyed hash function HMAC-SHA-256. The security of HMAC depends on the cryptographic strength of the used hash function [cc]. As a result, the key derivation functions used in 5G system are not substantially affected by the advances in quantum computing based on the conclusion in section 4.4.

**** End of third change ****