# 3GPP TSG SA WG3 (Security) Meeting #94	S3-190248

# 28 January – 1 February 2019, Kochi (India)

# Source:	Ericsson

# Title:	New KI: Key Revocation

# Document for:	Approval

Agenda Item:	8.5

# 1	Decision/action requested

It is suggested that the new Key issue is added to the study in 33.835

# 2	References

[1]	3GPP TR 33.835, v.0.2.0, Study on authentication and key management for applications based on 3GPP credential in 5G

[2]	3GPP TR 33.220, v.15.3.0, Generic Authentication Architecture (GAA) Generic Bootstrapping Architecture (GBA)

# 3	Rationale

In key issue #12 of 33.835, [1], lifetimes for the anchor key and derived sub-keys are discussed. A potential requirement is made that the lifetime of derived sub-keys (application keys) shall not exceed the lifetime of the anchor key.

This can be interpreted in different ways. Either the sub-key is given the lifetime equal to the remaining lifetime at the time of the sub-key derivation (option 1), or the sub-key gets the full maximal lifetime (option 2). These options are illustrated in Figure 1a and 1b.

The second option can be chosen to avoid all sub-keys being derived at the same time, causing potential network congestion.

However, in option 2, failure of the negotiation of a new anchor key implies that the UE is no longer authenticated. But according to the above, the derived sub-keys might still be in use. Hence, a revocation procedure for application keys is needed in case there is no longer a valid anchor key.

This document adds a new Key issue to 33.835 about revocation of application keys in case option 2 is implemented.

Figure 1a: Option 1

Figure 1b: Option 2

# 4	Detailed proposal

********** 1st CHANGE **********

All text below is new. For better readability, no change marks are used.

5.x	Key Issue #X_rev: Key revocation

5.x.1 Issue details

In key issue #12, lifetimes for the anchor key and derived sub-keys are discussed. A potential requirement is made that the lifetime of derived sub-keys (application keys) shall not exceed the lifetime of the anchor key.

To avoid re-negotiation of all sub-keys when the anchor key expires, one possibility is to continue to use these keys until their individual lifetime expires.

However, failure of the negotiation of a new anchor key implies that the UE is no longer authenticated. But according to the above, the derived sub-keys might still be in use.

Hence, a revocation procedure for application keys is needed in case there is no longer a valid anchor key.

### 5.x.2 Security Threats

If application keys cannot be revoked, there is a risk that a UE continues to use applications although the re-authentication of the UE fails or if the anchor key is compromised.

If an attacker can revoke application keys, there is a risk of DoS.

### 5.x.3 Potential security requirements

It shall be possible to revoke application keys securely.

********** END OF CHANGES **********