# 3GPP TSG-SA WG3 Meeting #95-BIS 	S3-192366

# Sapporo (Japan), 24-28 June 2019	revision of S3- 191929

# Source:	Huawei, HiSilicon

# Title:	Conclusions to KI #1 on Slice Authentication

# Document for:	Approval

Agenda Item:	8.11

# 1	Decision/action requested

This contribution proposes a pCR for the conclusion for KI #1 of TR 33.813

# 2	References

[1]

# 3	Rationale

With the agreement reached in SA2 on slice authentication procedure in R16 TS23.501/TS23.502, it is proposed to conclude the SA3 study on KI#1 on slice authentication,

# 4	Detailed proposal

pCR

***	BEGINNING of CHANGES	***

## 8.x 	General Conclusions on Slice Authenticaiton

- Slice authentication is optional to use

- Slice authentication uses a User ID and credentials, different from the 3GPP subscription credentials (e.g. SUPI and credentials used for PLMN access) and takes place after the primary authentication.

- AMF confirms, locally or based on ARPF/UDM, whether slice authentication is required for each S-NSSAI

- Slice Authentication is based on EAP framework where AMF takes the role of the pass through Authenticator

## 8.y	Conclusions for key issue #1 - Authentication for access to specific Network Slices

For KI#1, it is concluded that, a merge of Solution#1, solution#2 and Solution#4 shall be the basis for the normative work

***	END of CHANGES	***