# 3GPP TSG SA WG3 (Security) Meeting #96-Adhoc	S3-193357

# Chongqing (China), 14-18 October 2019	revision of S3-19abcd

# Source:	Qualcomm Incorporated

# Title: 	F1 security establishment

# Document for:	Approval

Agenda Item:	5.15

# 1	Decision/action requested

This contribution proposes a solution for F1 security setup

# 2	References

[1]	TS 33.501

[2]	TR 33.824

# 3	Rationale

Solution # 3.1 proposes F1 security establishment based on the KgNB. However, it is not clear why F1 security needs to be based on MT security which is mainly used for RLC channel setup. Using KgNB for F1 security setup (i.e., for IKEv2 between IAB node and IAB donor) is possible, yet doing such imposes additional requirements/complexity at IAB donor. For example, the IAB donor needs to distinguish wireline F1 (with DU) and wireless F1 (with IAB) and perform a different procedure depending on the identification of which F1 it is establishing. Furthermore, authorization information supplied to the RAN by the 5GC during the IAB initial registration procedure is intended to authorize MT setup, i.e., the RLC channel setup as an IAB node with its parent node, as clearly mentioned in solution #2.1 (S3-193152). However, the update on solution #3.1 based on the same contribution (S3-193152), namely, “By using the AS security context key KgNB, the IAB Donor ensures that the IAB Node is authorised already by the core network.” is not needed. In other words, it is not necessary to authorize DU based on a CN indication nor is binding between IAB MT and DU security required.

Assuming IAB node is provisioned with the credentials (e.g., certificate) for F1 establishment in the same manner as DU, it is unnecessary to derive a key for F1 establishment from the MT security key. Instead, it is better to use a unified procedure for F1 establishment for both wireline F1 and wireless F1 unless there are specific requirements for wireless F1 which are not applicable to wireline F1, e.g., support for IAB node mobility. Therefore, it is proposed that F1 security between IAB donor and IAB is established in the same way as that of CU and DU as specified TS 33.501 [1].

This solution describes the IAB node integration procedure.

# 4	Detailed proposal

It is proposed that the below pCR is included in the TR 33.824 [2].

**** START OF CHANGES ****

### 6.4.x	Solution #4.x: F1 security context establishment

#### 6.4.x.1	Introduction

This solution addresses the F1 security establishment requirements for IAB in key issue #4.1.

#### 6.4.x.2	Solution details

IAB integration procedure consists of 3 phases [3].

- IAB-MT setup:

The IAB-MT performs registration to the network as a UE and consequently, the AS security is established between the IAB-MT and IAB donor.

- Backhaul RLC channel establishment and routing update:

If the IAB-MT is authorized to run as an IAB node based on the CN indication to the IAB donor, the backhaul RLC channles and the BAP layer are established and configured using the secure RRC signalling to support routing between the IAB node and the IAB donor.

- IAB-DU setup:

F1 security establishement for IAB is performed over the RLC channel using the procedure specified in clause 9.8.2 of TS 33.501 [5].

The first phase results in AS security establishment between the IAB donor and IAB node; and the third phase results in the establishment of secure F1 between the IAB donor and IAB node.

#### 6.4.x.3	Evaluation

TBD

**** END OF CHANGES ****