# 3GPP TSG SA WG3 (Security) Meeting #96-Adhoc	S3-193391

# Chongqing (China), 14-18 October 2019	revision of S3-19wxyz

# Source:	Qualcomm Incorporated

# Title: 	AMF verification of the UE radio capabilities for CP optimization only CIoT UE

# Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This contribution proposes AMF verification of the UE radio capabilities for CP optimization only CIoT UE

# 2	References

[1]	TR 33.861

# 3	Rationale

The below pCR proposes a solution to protect the UE radio capability transfer for CIoT UEs.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR 33.861 [1].

***** START OF CHANGES *****

## 6.y	Solution #y: AMF verification of the UE radio capabilities for CP optimization only CIoT UE

### 6.y.1	Introduction

This solution addresses key issue #x: Security for UE radio capability transfer for CIoT UE.

### 6.y.2	Solution details

This solution proposes to protect the UE radio capabilities of control-plane only CIoT UEs in the following way.

- UE includes the hash of its radio capabilities in the Registration Request message to the network

- AMF verifies the UE radio capabilities when it acquires the UE radio capabilities based on the UE capability enquiry procedure

The procedure below only shows the messages required for the UE radio capability verification.

Figure 6.y.2-1: UE Radio Capability verification procedure for CP only CIoT devices

1. UE sends a Registration Request message with Hash of the UE radio capabilities (Hash_RC).

2. AMF determines to trigger a UE radio capability enquiry procedure by sending a UE radio capability information request message with the received Hash_RC to NG-RAN node serving the UE.

3. The NG-RAN node performs UE Capability Enquiry procedure with the UE to get the UE radio capabilities.

4. The NG-RAN node sends the UE radio capabilities to the AMF.

5. The AMF calculates the hash of the received UE radio capabilities and compares it with the hash received from the UE. If they match (i.e., verification is successful), the AMF stores the UE radio capabilities with a verified flag set.

NOTE 1: The verified flag can be used by the upgraded AMF (i.e., AMF that understands the verified flag) to confirm that the UE radio capability has already verified by another AMF when it receives the UE radio capability from a legacy AMF.

NOTE 2: If hash verification fails, the AMF would throw the UE Radio capabilities away. In this case, the UE radio capabilities would need to be re-fetched at the next connection.

### 6.y.3	Evaluation

TBD

***** END OF CHANGES *****