3GPP TSG-SA WG3 Meeting #96 Ad Hoc 	S3-193438

# Chongqing (CN), 14-18 October 2019																		revision of S3-19xabc

# Source:	Intel

# Title:	Security solution for UE Capability Transfer for UE with no AS security.

# Document for:	Approval

Agenda Item:	5.3.2

# 1	Decision/action requested

It is proposed to approve and add this solution to KI #X to TR 33.861

# 2	References

# 3	Rationale

It is proposed to approve and add this solution to TR 33.861

# 4	Detailed proposal

#### ***** Start of Change *****

## 6.Y	Solution #XX: Security solution for UE Capability Transfer for UE with no AS security.

6.Y.1	Introduction

This solution addresses key issue #X: Security Protection for UE capability transfer in this TR.

UE using CIOT CP optimization does not have AS security establishment. As a result, the existing solutions for normal UEs to protect the UE capability transfer does not prevent tampering of UE capability transfer using MITM attack as described in key issue detail of Key Issue #X.

This solution proposes to use the NAS based keys to protect the UE capability transfer from UE to the core network.

### 6.Y.2	Solution details

Figure 6.Y.2.1: UE Capability Transfer for UEs with No AS Security.

- The network initiates the UE capability enquiry procedure to a UE in RRC_CONNECTED when it needs (additional) UE radio access capability information. RAN sends a NAS UE capability indication message to network entity. Network calculates a DL_MAC using currently used NAS integrity algorithm with KNASInt as the key and UE Capability enquiry message as input. Optionally, Network performs encryption on the UE Capability Enquiry message using currently used NAS encryption algorithm and KNASEnc as the key.

- RAN receives the DL_MAC and optionally encrypted payload for the UE capability enquiry message.

- RAN sends the UE Capability Enquiry, optionally encrypted, along with DL_MAC, counter.

- Upon receiving the UE capability enquiry, UE first verifies the message using KNAS_INT and optionally decrypts message using KNAS_ENC. If successful, UE calculates UL_MAC for UE capability information using KNAS_INT and currently used the NAS integrity algorithm and a counter. Optionally UE encrypts the UE capability information IE using the KNAS_ENC outside of the PDCP layer. UE sends the UE capability information message to RAN along with UL_MAC and optionally encrypted Capability IE.
Note: UE modelling to handle UL and DL MAC should be left to RAN2 and CT groups.

- RAN forwards the UL_MAC, counter and optionally encrypted Capability IE to the core network for verification. Upon successful verification, the network sends decrypted capability information and notification to RAN.

6.Y.3	Evaluation

Editor’s Note: Each solution should motivate how the potential security requirements of the key issues being addressed are fulfilled.

#### ***** End of Change *****