3GPP TSG-SA3Meeting #96 Ad-hoc	S3-193512

# Chongqing, China, 14th Oct 2019 - 18th Oct 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | XXXX | rev | - | Current version: | 16.0.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CAG ID Privacy for non-public networks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  | Date: |  |  | 2019-10-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Security of CAG ID privacy is not specified and hence the feature cannot be deployed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding CAG ID privacy for non-public networks. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Privacy of CAG ID is not protected for non-public networks. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex I.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************************************************start of change************************************************

I.X	CAG ID privacy for non-public networks

In registration procedure, when the UE generate a integrated SUCI, the scheme-input further include the CAG ID. Or

The UE generate a concealed CAG ID using the protection scheme with the Home Network Public Key and with the CAG ID as the scheme-imput.

The concealment of CAG ID shall be performed by the UE whenever a SUCI is required, including UE sending a Registrating Request message and UE responding to an Identity Response message.

The RR message further includes the integrated SUCI or the concealed CAG ID.

UDM/SIDF is responsible for de-concealing the CAG ID from the integrated SUCI if the UE generated the SUCI with adding CAG ID to the scheme-input. Or

UDM/SIDF is responsible for de-concealing the CAG ID from the concealed SUCI using the same mechanism of decealing SUPI from SUCI.

**************************************************end of change************************************************