# 3GPP TSG SA3 Meeting #97 	S3-194036

# Reno, US, 18 – 22 November 2019

# Source:	Huawei, HiSilicon

# Title:	Handover UE under MitM FBS attacks

# Document for:	Approval

Agenda Item:	8.4

# 1	Decision/action requested

Approve this pCR for a new solution to handover victim UEs.

# 2	References

[1]	3GPP TR 33.809 V0.1.0, Study on 5G security enhancements against false base stations

[2]	3GPP TS 38.331 V15.4.0, Radio Resource Control (RRC)

# 3	Rationale

When the network detects a MitM FBS, gNB can handover a UE to another real gNB to disconnect the UE from the MitM FBS, and maintain the ongoing service continuity meanwhile.

# 4	Detailed proposal

**** Start of pCR to TR 33.809****

## 6.Y Solution #Y: Handover UE under MitM FBS attacks

### 6.Y.1	Introduction

This solution addresses the second security requirement in the following key issue:

-	5G system should be able to employ methods to prevent UEs from connecting to false base stations.

It is assumed that the network can detect a MitM FBS. The network can inform relevant gNBs to handover the UE to a real gNB, with a new cause value of MitM-FBS.

### 6.Y.2	Solution details

After the network detects a MitM FBS and informs the gNB, which are connected with victim UEs through the FBS, to handover victim UEs to real gNBs.

NOTE 1: This function is triggered by the network and the triggering condition is implementation specific.

Figure 6.Y.2-1 handover the UE to another real cell with cause of MitM-FBS

Steps 1: the serving gNB, which discovers or is informed that there is a MitM-FBS linking the gNB and a UE, decides to handover the UE to a real gNB.

Step 2: the serving gNB chooses a target gNB for the UE, and sends HO request to the selected target gNB, with the cause of MitM-FBS.

NOTE 2: the new cause of MitM-FBS is to allow the target gNB to prioritize this handover.

Step 3: the target gNB responds a HO request ACK message to the serving gNB.

Step 4: the serving gNB sends HO command message to the UE, and this message can include the cause of MitM-FBS and the FBS info.

NOTE 3: the cause of MitM-FBS and the FBS info are optional, and may be used for UE future cell reselection procedure.

Step 5: the UE executes handover to the target gNB, and stores the received FBS info for future cell reselection procedure.

### 6.Y.3	Evaluation

UE connecting to a FBS in the middle can be handed over to a real gNB and maintain the ongoing service continuity.

**** END OF CHANGES ****