3GPP TSG SA3 Meeting #97                  	                      S3-194041

# Reno, US, 18 – 22 November 2019	                                          revision of S3-19xabc

# Source:	Huawei, HiSilicon

# Title:	Solution on secure ID conversion in groupcast communication.

# Document for:	Approval

Agenda Item:	8.13 (FS_eV2X_Sec)

# 1	Decision/action requested

This pCR proposes a new solution for key issue #9 in TR 33.836

# 2	References

[1]	TR 33.836 Study on Security Aspects of 3GPP support for Advanced V2X Services

[2]	TS 23.287 Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services.

# 3	Rationale

This contribution proposes a solution that addresses the security requirements of the key issue #4 and part of key issue #3 and key issue #9.

# 4	Detailed proposal

*** START OF CHANGE ***

## 6.Y	Solution #Y: Solution on securely creating destination Layer-2 ID in groupcast communication

### 6.Y.1	Introduction

This solution addresses the KI #4 "Security of identifier conversion in groupcast communication", and part of KI #3 “Privacy protection for groupcast messages over PC5” and KI #9 “Minimizing the impact of privacy protection mechanism in the application layer communication”.

### 6.Y.2	Solution details

The detailed solution is illustrated in Figure 6.Y.2-1

Figure 6.Y.2-1: Procedure for secure conversion of application layer group ID to destination Layer-2 ID

- Group Setup: Once the group is created, the group management server will send the group ID to the associated UEs and a timer T. It will also send a set of random numbers and a specific sequence in which these random number are to be used.	 It is assumed that the application layer signalling is protected.

1.	ID Conversion: All the UEs use the group ID and the first random number according to the sequence as an input to a hash function to generate the destination Layer-2 ID.

2.	ID Update: When the timer T expires, a new destination Layer-2 ID is calculated using the next random number according to the sequence. The UEs can listen to both old a new destination Layer-2 IDs, to avoid any time synchronization issues, for a certain period of time or until receives a message with the new ID.

The destination Layer-2 ID is updated until the application layer changes the group ID.

The group management server can also send the corresponding materials to generate random numbers rather than sending the random numbers itself.

### 6.Y.3	Solution Evaluation

The solution fulfils the security requirement outlined by key issue #4 and in part the security requirements of key issue #3 and key issue #9. The solutions provides a mechanism to securely convert the group ID to destination Layer-2 ID as well as preserves the privacy of the destination Layer-2 ID by updating it regularly. Listening to both L2 IDs after conversion make sure that the application layer communication is also not impacted.

*** END OF CHANGE ***