3GPP TSG-SA3Meeting #97	S3-194066

# Reno, Nevada, United States, 18th Nov 2019 - 22nd Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0665 | rev | - | Current version: | 16.0.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security handling in registration with AMF re-allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  | Date: |  |  | 2019-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 33.501 specifies security handling in registration with AMF reallocation via direct NAS route in its clause 6.9.6. The initial AMF shall use its local policy to determine whether to perform horizontal KAMF derivation on current KAMF. However, what shall be included in the message from initial AMF to Target AMF, and the handling in Target AMF upon receiving the message is not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Detail of message of initial AMF to Target AMF and handling in Target AMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incompleted specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 6.9.6. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************************************************start of change************************************************

### 6.9.6	Security handling in registration with AMF reallocation via direct NAS reroute

In registration with AMF reallocation via direct NAS reroute, the initial AMF shall use its local policy to determine whether to perform horizontal KAMF derivation on current KAMF. If the initial AMF determines not to perform horizontal KAMF derivation, the initial AMF shall transfer current security context to the target AMF. If the initial AMF determines to perform horizontal KAMF derivation, the initial AMF shall derive a new key KAMF from the currently active KAMF and the uplink NAS COUNT value in the received Registration Request message. The ngKSI for the newly derived KAMF key is defined such as the value field and the type field are taken from the ngKSI of the current KAMF. The initial AMF shall transfer the new KAMF, the new ngKSI, the UE security capability, the  keyAmfHDerivationInd to the target AMF. The key derivation of the new KAMF is specified in Annex A.13. If the initial AMF has derived a new key KAMF, the initial AMF shall not transfer the old KAMF to the target AMF and the initial AMF shall in this case also delete any stored non-current 5G security context, and not transfer any non-current 5G security context to the target AMF.

If the target AMF receives the indication of horizontal KAMF derivation (i.e., keyAmfHDerivationInd), it shall derive the NAS keys from the received KAMF as specified in clause A.8 and set the NAS COUNTs to zero. It shall send the  K_AMF_change_flag set to 1 to the UE in the NAS SMC including replayed UE security capabilities, the selected NAS algorithms and the ngKSI for identifying the new KAMF from which the UE shall derive a new KAMF to establish a new NAS security context between the UE and target AMF. If the target AMF does not receive keyAmfHDerivationInd but wants to change the NAS algorithms, it shall send NAS SMC using the selected NAS security algorithms in the same manner as the case for the horizontal KAMF derivation. However, the target AMF shall not set the NAS COUNTs to zero.

If the target AMF, according to its local policy, decides to not use the KAMF received from the source AMF, it can perform a re-authentication procedure to the UE to establish a new NAS security context.

**************************************************end of change************************************************