3GPP TSG-SA WG3 Meeting #97 	S3-191432

# Reno (US), 18-22 November 2019	revision of S3-19xabc

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.512 | CR | 0002 | rev |  | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fixing the reference numbers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G |  |  |  |  |  | Date: |  |  | 2019.11.18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Fixing the reference numbers. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fixing the reference numbers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong reference number will cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1.1, 4.2.2.1.2, 4.2.2.3.1, 4.2.2.3.2, 4.2.2.4.1, 4.2.2.4.2, 4.2.2.5.1, 4.2.2.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** START OF CHANGE

### 4.2.2	Security functional requirements on the AMF deriving from 3GPP specifications and related test cases

#### 4.2.2.1 	Authentication and key agreement procedure

##### 4.2.2.1.1	Synchronization failure handling

Requirement Name: Synchronization failure handling

Requirement Reference: TS 33.501 [2], clause 6.1.3.3.2

Requirement Description: "Upon receiving an authentication failure message with synchronisation failure (AUTS) from the UE, the SEAF sends an Nausf_UEAuthentication_Authenticate Request message with a "synchronisation failure indication" to the AUSF.

An SEAF will not react to unsolicited "synchronisation failure indication" messages from the UE.

The SEAF does not send new authentication requests to the UE before having received the response to its Nausf_UEAuthentication_Authenticate Request message with a "synchronisation failure indication" from the AUSF (or before it is timed out)."

as specified in TS 33.501[2], clause 6.1.3.3.2.

Threat References: TR 33.926 [6], clause K.2.2.1, Resynchronization

Test Case:

Test Name: TC_SYNC_FAIL_SEAF_AMF

Purpose:

Verify that synchronization failure is correctly handled by the SEAF/AMF.

Pre-Conditions:

-	Test environment with UE and AUSF. The UE and the AUSF may be simulated.

-	AMF network product is connected in emulated/real network environment.

Execution Steps

1)	The UE sends an authentication failure message to the SEAF/AMF with synchronisation failure (AUTS).

2)	The SEAF/AMF sends a Nausf_UEAuthentication_Authenticate Request message with a "synchronisation failure indication" to the AUSF.

3a)	The AUSF sends a Nausf_UEAuthentication_Authenticate Response message to the SEAF/AMF.

3b)	 The timer for receiving Nausf_UEAuthentication_Authenticate Response message from the AUSF runs out in the SEAF/AMF.

Expected Results:

Before receiving Nausf_UEAuthentication_Authenticate Response message from the AUSF and before the timer for receiving Nausf_UEAuthentication_Authenticate Response message runs out, the SEAF/AMF does not send any new authentication request to the UE.

After receiving Nausf_UEAuthentication_Authenticate Response message from the AUSF or after the timer for receiving Nausf_UEAuthentication_Authenticate Response message runs out, the SEAF/AMF may initiate new authentication towards the UE.

##### 4.2.2.1.2 	RES* verification failure handling

Requirement Name: RES* verification failure handling

Requirement Reference: TS 33.501 [2], clause 6.1.3.2.2

Requirement Description:

"The SEAF shall proceed with step 10 in Figure 6.1.3.2-1 and after receiving the Nausf_UEAuthentication_Authenticate Request message from the AUSF in step 12 in Figure 6.1.3.2-1, proceed as described below:

-	if the AUSF has indicated in the Nausf_UEAuthentication_Authenticate Response message to the SEAF that the verification of the RES* was not successful in the AUSF, or

-	if the verification of the RES* was not successful in the SEAF,

then the SEAF shall either reject the authentication by sending an Authentication Reject to the UE if the SUCI was used by the UE in the initial NAS message or the SEAF/AMF shall initiate an Identification procedure with the UE if the 5G-GUTI was used by the UE in the initial NAS message to retrieve the SUCI and an additional authentication attempt may be initiated.

Also, if the SEAF does not receive any Nausf_UEAuthentication_Authenticate Request message from the AUSF as expected, then the SEAF shall either reject the authentication to the UE or initiate an Identification procedure with the UE."

As specified in TS 33.501 [2], clause 6.1.3.2.2.

Threat References: TR 33.926 [6], clause K.2.2.3, RES* verification failure

Test Case:

Test Name: TC_RES*_VERIFICATION_FAILURE

Purpose:

1)	Verify that the SEAF/AMF correctly handles RES* verification failure detected in the SEAF/AMF or/and in the AUSF, when the SUCI is included in the initial NAS message.

2)	Verify that the SEAF/AMF correctly handles RES* verification failure detected in the SEAF/AMF or/and in the AUSF, when the 5G-GUTI is included in the initial NAS message.

Procedure and execution steps:

Pre-Conditions:

Test environment with UE and AUSF. The UE and the AUSF may be simulated.

Execution Steps

A.	Test Case 1

1)	The UE returns an incorrect RES* to the SEAF/AMF in the NAS Authentication Response message.

2)	The SEAF/AMF computes HRES* from RES* and compares HRES* and HXRES*.

3)	The SEAF/AMF sends RES* received from the UE together with the corresponding SUCI in a Nausf_UEAuthentication_Authenticate Request message to the AUSF.

4a)	The SEAF/AMF receives the Nausf_UEAuthentication_Authenticate Response message from the AUSF which indicates unsuccessful RES* verification in the AUSF; or

4b)	The SEAF/AMF does not receive the Nausf_UEAuthentication_Authenticate Response message from the AUSF until the timer runs out.

B.	Test Case 2

1)	The UE returns an incorrect RES* to the SEAF/AMF in the NAS Authentication Response message.

2)	The SEAF/AMF computes HRES* from RES* and compares HRES* and HXRES*.

3)	The SEAF/AMF sends RES* received from the UE together with the corresponding SUPI in a Nausf_UEAuthentication_Authenticate Request message to the AUSF.

4a)	The SEAF/AMF receives the Nausf_UEAuthentication_Authenticate Response message from the AUSF which indicates unsuccessful RES* verification in the AUSF; or

4b)	The SEAF/AMF does not receive the Nausf_UEAuthentication_Authenticate Response message from the AUSF until the timer runs out.

Expected Results:

For test case 1, the SEAF/AMF rejects the authentication by sending an Authentication Reject to the UE.

For test case 2, the SEAF/AMF initiates an Identification procedure with the UE to retrieve the SUCI.

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot containing the operational results.

#### 4.2.2.2 	Key derivation and distribution

#### 4.2.2.3 	Security mode command procedure

##### 4.2.2.3.1 	Replay protection of NAS signalling messages

Requirement Name: Replay protection of NAS signalling messages

Requirement Reference: TS 33.501 [2], clause 5.5.1.

Requirement Description: "AMF shall support replay protection of NAS signalling messages between UE and AMF on N1 interface." as specified in TS 33.501 [2], clause 5.5.1.

Threat References: TR 33.926 [6], clause K.2.3.1, Bidding Down

Test case:

Test Name: TC_NAS_REPLAY_AMF

Purpose:

Verify that the NAS signalling messages are replay protected by AMF over N1 interface between UE and AMF.

Procedure and execution steps:

Pre-Condition:

-	AMF network product is connected in emulated/real network environment.

-	Tester shall have access to the NAS signalling packets sent between UE and AMF over N1 interface.

-	Tester shall ensure that integrity protection algorithm other than NIA0 is used.

Execution Steps:

1.	The tester shall capture the NAS SMC procedure taking place between UE and AMF over N1 interface using any network analyser.

2.	The tester shall filter the NAS security mode command complete message by using a filter.

3.	The tester shall check for the NAS SQN of filtered NAS security mode command complete message and using any packet crafting tool the tester shall create a NAS security mode command complete message containing same NAS SQN of the filtered NAS security mode command complete message or the tester shall replay the captured NAS signalling packets.

4.	Tester shall check whether the replayed NAS signalling packets were processed by the AMF by capturing over N1interface to see if any corresponding response message is received from the AMF.

5.	Tester shall confirm that AMF provides replay protection by dropping/ignoring the replayed packet if no corresponding response is sent by the AMF to the replayed packet.

6.	Tester shall verify from the result that if the crafted NAS security mode command complete message or replayed NAS signalling messages are not processed by the AMF then the N1 interface is replay protected

Expected Results:

The NAS signalling messages sent between UE and AMF over N1 interface are replay protected.

##### 4.2.2.3.2 	NAS NULL integrity protection

Requirement Name: NAS NULL integrity protection

Requirement Reference: TS 33.501 [2], clause 5.5.2

Requirement Description: "NIA0 shall be disabled in AMF in the deployments where support of unauthenticated emergency session is not a regulatory requirement." as specified in TS 33.501 [2], clause 5.5.2

Threat References: TR 33.926 [6], clause K.2.3.3, NAS NULL integrity protection

Test Case:

Test Name: TC_NAS_NULL_INT_AMF

Purpose:

Verify that NAS NULL integrity protection algorithm is used correctly.

Pre-Conditions:

Test environment with a UE. The UE may be simulated.

The UE was successfully authenticated.

The vendor shall provide documentation describing how NIA0 is disabled and enabled in the AMF.

Execution Steps

1.	The AMF derives the KAMF and NAS signalling keys after successful authentication of the UE.

2.	The AMF sends the NAS Security Mode Command message to the UE containing the selected NAS algorithms.

Expected Results:

The integrity algorithm selected by the AMF in NAS SMC message is different from NIA0.

The NAS Security Mode Command message is integrity protected by the AMF.

#### 4.2.2.4 	Security in intra-RAT mobility

##### 4.2.2.4.1	Bidding down prevention in Xn-handover

Requirement Name: Bidding down prevention in Xn-handovers

Requirement Reference: TS 33.501 [2], clause 6.7.3.1

Requirement Description: "In the Path-Switch message, the target gNB shall send the UE's 5G security capabilities received from the source gNB to the AMF. The AMF shall verify that the UE's 5G security capabilities received from the target gNB are the same as the UE's 5G security capabilities that the AMF has locally stored. If there is a mismatch, the AMF shall send its locally stored 5G security capabilities of the UE to the target gNB in the Path-Switch Acknowledge message. The AMF shall support logging capabilities for this event and may take additional measures, such as raising an alarm."

as specified in TS 33.501 [2], clause 6.7.3.1.

Threat References: TR 33.926 [6], clause K.2.4.1, Bidding down on Xn-Handover

Test Case:

Test Name: TC_BIDDING_DOWN_XN_AMF

Purpose:

Verify that bidding down is prevented by the AMF under test in Xn handovers.

Pre-Conditions:

Test environment with (target) gNB may be simulated.

The AMF under test is configured with the UE’s security context for the UE.

The AMF under test is configured to log UE security capability mismatch.

Execution Steps

The tester sends 5G security capabilities for the UE, different from the ones stored in the AMF, to the AMF under test using a Path-Switch message.

Expected Results:

The tester captures the Path-Switch Acknowledge message sent by AMF under test to the target gNB, which includes the locally stored 5G security capabilities in the AMF under test for that UE.

The tester verifies that a log entry showing the capability mismatch is logged.

Expected format of evidence

Evidence suitable for the interface, e.g., Screenshot containing the operational results.

##### 4.2.2.4.2 	NAS protection algorithm selection in AMF change

Requirement Name: NAS protection algorithm selection in AMF change

Requirement Reference: TS 33.501 [2], clause 6.7.1.2

Requirement Description: "If the change of the AMF at N2-Handover or mobility registration update results in the change of algorithm to be used for establishing NAS security, the target AMF shall indicate the selected algorithm to the UE as defined in Clause 6.9.2.3.3 for N2-Handover (i.e., using NAS Container) and Clause 6.9.3 for mobility registration update (i.e., using NAS SMC). The AMF shall select the NAS algorithm which has the highest priority according to the ordered lists (see sub-clause 6.7.1.1 of the present document)."

as specified in TS 33.501 [2], clause 6.7.1.2.

Threat References: TR 33.926 [6], clause K.2.4.2, NAS integrity protection algorithm selection in AMF change

Test Case:

Test Name: TC_NAS_ALG_AMF_CHANGE _AMF

Purpose:

Verify that NAS protection algorithms are selected correctly.

Pre-Conditions:

Test environment with gNB, source AMF. Source AMF may be simulated.

Execution Steps

Test case 1: N2-Handover

The AMF under test receives the UE security capabilities and the NAS algorithms used by the source AMF from the source AMF. The AMF under test selects the NAS algorithms which have the highest priority according to the ordered lists. The lists are configured such that the algorithms selected by the AMF under test are different from the ones received from the source AMF.

Test case 2: Mobility registration update

The AMF under test receives the UE security capabilities and the NAS algorithms used by the source AMF from the source AMF. The AMF under test selects the NAS algorithms which have the highest priority according to the ordered lists. The lists are configured such that the algorithms selected by the AMF under test are different from the ones received from the source AMF.

Expected Results:

For Test case 1, the tester captures the NASC of the NGAP HANDOVER REQUEST message sent by the AMF under test to the gNB, which includes the chosen algorithm.

For Test case 2, the AMF under test initiates a NAS security mode command procedure and includes the chosen algorithms.

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot containing the operational results.

#### 4.2.2.5 	5G-GUTI allocation

##### 4.2.2.5.1	5G-GUTI allocation

Requirement Name: 5G-GUTI allocation

Requirement Reference: TS 33.501 [2], clause 6.12.3

Requirement Description: "A new 5G-GUTI shall be sent to a UE only after a successful activation of NAS security. The 5G-GUTI is defined in TS 23.003 [4].

Upon receiving Registration Request message of type "initial registration" or "mobility registration update" from a UE, the AMF shall send a new 5G-GUTI to the UE during the registration procedure.

Upon receiving Registration Request message of type "periodic registration update" from a UE, the AMF should send a new 5G-GUTI to the UE during the registration procedure.

Upon receiving Service Request message sent by the UE in response to a Paging message, the AMF shall send a new 5G-GUTI to the UE. This new 5G-GUTI shall be sent before the current NAS signalling connection is released.

NOTE 1:	It is left to implementation to re-assign 5G-GUTI more frequently than in cases mentioned above.

NOTE 2:	It is left to implementation to generate 5G-GUTI containing 5G-TMSI that uniquely identifies the UE within the AMF."

as specified in TS 33.501 [2], clause 6.12.3.

Threat References: TR 33.926 [6], clause K.2.7.1, Failure to allocate new 5G-GUTI

Test Case:

Test Name: TC_5G_GUTI_ALLOCATION _AMF

Purpose:

Verify that a new 5G-GUTI is allocated by the AMF under test in these scenarios accordingly.

Pre-Conditions:

Test environment with a UE. The UE may be simulated.

Tester has access to the NAS signalling packets sent over N1 interface.

Tester has the knowledge of the UE’s security context used for protecting the Registration Request of type "mobility registration update" and Service Request, including the old 5G-GUTI, ngKSI, UE NR security capability, NAS security context. And the tester shall configure the UE’s security context on the AMF under test.

Execution Steps

Test case 1:

Upon receiving Registration Request message of type "initial registration" from a UE, the AMF sends a new 5G-GUTI to the UE during the registration procedure.

Test case 2:

Upon receiving Registration Request message of type "mobility registration update" from a UE, the AMF sends a new 5G-GUTI to the UE during the registration procedure.

Test case 3:

Upon receiving Service Request message sent by the UE in response to a Paging message, the AMF sends a new 5G-GUTI to the UE.

Expected Results:

For Test case 1, 2, 3, the tester retrieves a new 5G-GUTI by accessing the NAS signalling packets sent by the AMF under test over N1 interface during registration procedure.

For Test case 1, 2, 3, the NAS message encapsulating the new 5G-GUTI is confidentiality and integrity protected by the AMF under test using the NAS security context, which is same as the UE’s NAS security context.

The new 5G-GUTI is different from the old 5G-GUTI.

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot containing the operational results.

#### 4.2.2.6 	Security in registration procedure

##### 4.2.2.6.1 	Invalid or unacceptable UE security capabilities handling

Requirement Name: Invalid or unacceptable UE security capabilities handling

Requirement Reference: TS 24.501 [5], clause 5.5.1.2.8

Requirement Description:"

…

i)	UE security capabilities invalid or unacceptable

If the REGISTRATION REQUEST message is received with invalid or unacceptable UE security capabilities (e.g. no 5GS encryption algorithms (all bits zero), no 5GS integrity algorithms (all bits zero), mandatory 5GS encryption algorithms not supported or mandatory 5GS integrity algorithms not supported, etc.), the AMF shall return a REGISTRATION REJECT message."

as specified in TS 24.501 [5], clause 5.5.1.2.8.

Threat References: TR 33.926 [6], clause K.2.6.1, Invalid or unacceptable UE security capabilities

Test Case:

Test Name: TC_UE_SEC_CAP_HANDLING_AMF

Purpose:

Verify that UE security capabilities invalid or unacceptable are not accepted by the AMF under test in registration procedure.

Pre-Conditions:

Test environment with (target) UE, which may be simulated.

The tester configures invalid/unacceptable UE security capabilities (no 5GS encryption algorithms (all bits zero), no 5GS integrity algorithms (all bits zero), mandatory 5GS encryption algorithms not supported or mandatory 5GS integrity algorithms not supported) on the UE.

Execution Steps

The UE sends UE security capabilities to the AMF under test using registration request message.

Expected Results:

The tester captures the Registration reject message sent by AMF under test to the UE.

Expected format of evidence

Evidence suitable for the interface, e.g., Screenshot containing the operational results.

***************** END OF CHANGE