# 3GPP TSG SA WG3 (Security) Meeting #97	S3-194230

# 18-22 November, Reno, Nevada (USA)	revision of S3-19xabc

# Source:	China Mobile

# Title:	Add the conclusion on the UP IP for 5G RAN connected to 5GC

# Document for:	Approval

Agenda Item:	8.9

1	Decision/action requested

This  pCR adds the conclusion on the UP IP for 5G RAN connected 5GC, and is kindly asked to be approved by SA3.

2	Reference

[1]	 TR 33.853 v0.6.0 (2019-10), Technical report on key issues and potential solutions for Integrity protection of the User Plane

3	Introduction

As shown in the accompanied paper, there is a need to provide an option to protect the integrity of a data packet header besides support UP IP at full data rate.  Thus this recommendation should be added to the conclusion on the UP IP for 5G RAN connected to 5GC.

4	Detailed Proposal

**** Beginning of Change ****

7	Conclusions

It is concluded that the Rel-15 architectural approach of applying user plane integrity protection at NR PDCP layer in TS 33.501 is adopted as the basis for normative work for the following network options:

- Option 2 – NR standalone with 5G Core

- Option 4 – 5G Core based Dual Connectivity (NR master – eUTRA secondary)

- Option 5 – 5G Core with eUTRA

- Option 7 – 5G Core based Dual Connectivity (eUTRA master – NR secondary)

Besides support UP IP at full data rate, an option to protect the integrity of a data packet header is recommended.

Editor's Note: Further conclusion(s) are FFS.

**** End of Change ****