3GPP TSG-SA3 Meeting #97	S3-194275

# Reno, USA 18 - 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | <CR#> | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Draft CR] Security requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB |  |  |  |  |  | Date: |  |  | 11-11-2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are no security requirements on the IAB-node, the parent IAB-node and the 5GC supporting IAB architecture. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new clauses for security requirements on the IAB-node, the parent IAB-node and the 5GC supporting IAB architecture. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are no security requirements on the IAB-node, the IAB-donor and the 5GC supporting IAB architecture. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### **** START OF CHANGES ****

# Annex X (normative): Security for Integrated Access and Backhaul

# X.1	General

This Annex provides the security procedures applied to NR IAB architecture and functional entities for supporting wireless backhauling of NR base stations. Security requirements applied to IAB architecture are defined in the main body of the present specification.

The overall stage 2 description for IAB architecture and functional entities are described in 3GPP TS 23.501 [XX] and 3GPP TS 38.401 [yy].

# X.2	IAB-node Integration Procedure

## X.2.1	Authentication and Authorization of IAB-node

## X.2.2 	Security mechanisms for F1 interface between the IAB-node (gNB-DU) and the IAB-donor-CU

# X.x	Security requirements and features

## X.x.1	Requirements on the IAB-node (IAB-MT)

The IAB-node (IAB-MT) shall support ciphering, integrity protection and replay protection of NAS-signalling between the IAB-node (IAB-MT) and the 5GC supporting IAB architecture.

The IAB-node (IAB-MT) shall support ciphering, integrity protection and replay protection of RRC-signalling between the IAB-node (IAB-MT) and the parent IAB-node.

Mutual authentication between the IAB-node (IAB-MT) and the 5GC supporting IAB architecture shall be supported.

## X.x.2	Requirements on the parent IAB-node

The parent IAB-node shall support ciphering, integrity protection and replay protection of RRC-signalling between the parent IAB-node and the IAB-node (IAB-MT).

## X.x.3	Requirements on the 5GC supporting IAB architecture

The 5GC supporting IAB architecture shall support ciphering, integrity protection and replay protection of NAS-signalling between the 5GC supporting IAB architecture and the IAB-node (IAB-MT).

Mutual authentication between the 5GC supporting IAB architecture and the IAB-node (IAB-MT) shall be supported.

The 5GC shall decide whether the IAB-node is authorized to operate as IAB-node.

**** END OF CHANGES ****