3GPP TSG-SA3 Meeting #97	S3-194278

# Reno, USA 18 - 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | <CR#> | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Draft CR] Authentication of IAB-nodes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB |  |  |  |  |  | Date: |  |  | 11-11-2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The procedures for authentication of IAB-nodes in the IAB architecture have not been described in TS 33.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause for security procedures for authentication of IAB-nodes in the IAB architecture. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The security procedures for authentication of IAB-nodes in the IAB architecture are not described in TS 33.501. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X.2.1.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### **** START OF CHANGES ****

# Annex X (normative): Security for Integrated Access and Backhaul

# X.1	General

This Annex provides the security procedures applied to NR IAB architecture and functional entities for supporting wireless backhauling of NR base stations. Security requirements applied to IAB architecture are defined in the main body of the present specification.

The overall stage 2 description for IAB architecture and functional entities are described in 3GPP TS 23.501 [XX] and 3GPP TS 38.401 [yy].

# X.2	IAB-node Integration Procedure

## X.2.1	Authentication and Authorization of IAB-node

### X.2.1.1	Authentication of IAB-node

For mutual authentication between the IAB-node and the 5GC supporting IAB architecture, the procedures for primary authentication in the present specification apply. The IAB-node plays the role of the UE in these procedures.

**** END OF CHANGES ****