3GPP TSG-SA3 Meeting # 97	S3-194281

# Reno, USA 18 - 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.401 | CR | <CR#> | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Draft CR] Security for Integrated Access and Backhaul in EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB |  |  |  |  |  | Date: |  |  | 2019-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In an IAB architecture, the IAB-node can access the network using either standalone mode or EN-DC mode. The security aspects of IAB in standalone mode are define in 3GPP TS 33.501. Similarly, the security aspects of IAB in the EN-DC mode need to be defined in 3GPP TS 33.401. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Relevant references and normative Annex are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete security, unimplementable standard |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### **** START OF CHANGES ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

>>>> Skip

[21]	3GPP TS 36.331:"Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC); Protocol specification".

>>>> Skip

[42]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[43]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[aa]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[gg]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP)".

### **** NEXT CHANGE ****

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

>>>> Skip

HO	Hand Over

HSS	Home Subscriber Server

IAB	Integrated Access and Backhaul

IK	Integrity Key

IKE	Internet Key Exchange

>>>> Skip

### **** NEXT CHANGE ****

Annex X (normative):
Security for Integrated Access and Backhaul in EN-DC

# X.1	Introduction

In an IAB (Integrated Access and Backhaul) architecture, the IAB-node can access the network using either standalone mode or EN-DC mode. Overall description of IAB feature in standalone mode is in 3GPP TS 38.300 [aa]. The EN-DC specific details are defined in 3GPP TS 36.331 [21], TS 36.413 [42], and TS 36.423 [gg].

When using the EN-DC mode, as shown in Figure X.1-1, the IAB-node connects via E-UTRA to a MeNB, and the IAB-donor terminates X2-C as SgNB.

Figure X.1-1: IAB architecture when IAB-node is using EN-DC

The present document only deals with security aspects of IAB in the EN-DC mode. The security aspects of IAB in standalone mode (including authentication and authorization of IAB-nodes, and security of F1 interfaces) are defined in 3GPP TS 33.501 [43].

# X.2	Authentication and authorization of IAB-nodes

For authentication and authorization described in this clause, the IAB-node plays the role of a UE and a summary is illustrated in Figure X.2-1.

Figure X.2-1: Summary of authentication and authorization of IAB-nodes

-	The indication of being an IAB-node is signalled from the IAB-node to the eNB as defined in 3GPP TS 36.331 [21].

-	This indication is signalled from the eNB to the MME as defined in 3GPP TS 36.413 [42].

-	Mutual authentication between the IAB-node and the EPC supporting the IAB architecture shall be performed using the authentication and key agreement procedure defined in the clause 6.1 of the present document.

-	After successful authentication and validity check, the indication that the IAB-node is authorized is signalled from the MME to the eNB as defined in 3GPP TS 36.413 [42].

-	During EN-DC procedures, the indication of IAB-node is signalled from the MeNB to the SgNB as defined in 3GPP TS 36.423 [gg].

-	During handover procedures, the indication of IAB-node is signalled from the one eNB to another eNB as defined in 3GPP TS 36.423 [gg].

**** END OF CHANGES ****