3GPP TSG-SA3 Meeting # 97	S3-194344

# Reno,US, 18 - 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.513 | CR | 0002 | rev | - | Current version: | 16.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for clean-up and alignment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G |  |  |  |  |  | Date: |  |  | 2019-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The release of the reference (TS 33.501) needs to be confined to R15. The sub-clauses in 4.3 need to be aligned with those in TS 33.117 R16. The clause numbers of TR 33.926 referenced in the test cases need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added the release number (R15) for TS 33.501 in the reference list. - Added a new sub-clause (4.3.x) in clause 4.3 for structure alignment with TS 33.117 R16. - Updated the clause numbers of TR 33.926 referenced in the test cases for alignment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mis-alignment |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2.2.1, 4.2.2.2, 4.2.2.3, 4.2.2.4, 4.2.2.5, 4.2.2.6, new sub-clause 4.3.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the 1st Change ****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501 (Release 15): "Security architecture and procedures for 5G system".

[3]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[4]	3GPP TS 23.501: "System Architecture for 5G system".

[5]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U) ".

[6]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[7]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

*************** Start of the 2nd Change ****************

#### 4.2.2.1	Confidentiality protection of user data transported over N3 interface.

Requirement Name: Confidentiality protection of user data transported over N3 interface.

Requirement Reference: TS 33.501 [2], Clause 9.3

Requirement Description: "The transported user data between gNB and UPF shall be confidentiality protected." As specified in TS 33.501 [2], clause 9.3.

Threat Reference: TR 33.926 [7], Clause L.2.2, "No protection or weak protection for user plane data ".

TEST CASE:

Test Name: TC_UP_DATA_CONF_UPF

Purpose:

Verify that the transported user data between gNB and UPF are confidentiality protected over N3 interface.

Procedure and execution steps:

Pre-Condition:

-	UPF network product is connected in simulated/real network environment.

-	The tunnel mode IPsec ESP and IKE certificate authentication is implemented.

-	Tester shall have knowledge of the security parameters of tunnel for decrypting the ESP packets.

-	Tester shall have access to the N3 interface between gNB and UPF.

-	Tester shall have knowledge of the confidentiality algorithm and confidentiality protection keys used for encrypting the encapsulated payload.

Execution Steps:

The requirement mentioned in this clause is tested in accordance with the procedure mentioned in clause 4.2.3.2.4 of TS 33.117 [3].

Expected Results:

The user data transported between gNB and UPF is confidentiality protected.

Expected format of evidence:

Evidence suitable for the interface, e.g., evidence can be presented in the form of screenshot/screen-capture.

#### 4.2.2.2	Integrity protection of user data transported over N3 interface

Requirement Name: Integrity protection of user data transported over N3 interface.

Requirement Reference: TS 33.501 [2], Clause 9.3

Requirement Description: "The transported user data between gNB and UPF shall be integrity protected" as specified in TS 33.501 [2], clause 9.3.

Threat Reference: TR 33.926 [7], Clause L.2.2, "No protection or weak protection for user plane data"

TEST CASE:

Test Name: TC_UP_DATA_INT_UPF

Purpose:

Verify that the transported user data between gNB and UPF are integrity protected over N3 interface.

Procedure and execution steps:

Pre-Condition:

-	UPF network product is connected in simulated/real network environment.

-	The tunnel mode IPsec ESP and IKE certificate authentication is implemented.

-	Tester shall have knowledge of the security parameters of tunnel for decrypting the Encapsulated Security Payload (ESP) packets.

-	Tester shall have knowledge of the authentication algorithm (Hash Message Authentication Code) and the protection keys.

Execution Steps:

The requirement mentioned in this clause is tested in accordance to the procedure mentioned in clause 4.2.3.2.4 of TS 33.117 [3].

Expected Results:

The user data transported between gNB and UPF is integrity protected.

Expected format of evidence:

Evidence suitable for the interface, e.g., evidence can be presented in the form of screenshot/screen-capture.

#### 4.2.2.3	Replay protection of user data transported over N3 interface

Requirement Name: Replay protection of user data transported over N3 interface

Requirement Reference: TS 33.501 [2], Clause 9.3

Requirement Description: "The transported user data between gNB and UPF shall be replay protected." As specified in TS 33.501, clause 9.3.

Threat Reference: TR 33.926 [7], Clause L.2.2, "No protection or weak protection for user plane data"

TEST CASE:

Test Name: TC_UP_DATA_REPLAY_UPF

Purpose:

Verify that the transported user data between gNB and UPF are replay protected.

Procedure and execution steps:

The following procedure is executed if UPF supports IPsec.

Pre-Condition:

-	UPF network product is connected in simulated/real network environment.

-	The tunnel mode IPsec ESP and IKE certificate authentication is implemented.

-	Tester shall have knowledge of the security parameters of tunnel for decrypting the ESP packets.

-	Tester shall have access to the original user data transported via N3 reference point between gNB and UPF.

Execution Steps:

The requirement mentioned in this clause is tested in accordance with the procedure mentioned in clause 4.2.3.2.4 of TS 33.117 [3].

Expected Results:

The user data transported between UE and UPF is replay protected.

Expected format of evidence:

Evidence suitable for the interface, e.g., evidence can be presented in the form of screenshot/screen-capture.

#### 4.2.2.4	Protection of user data transported over N9 interface Within a PLMN

Requirement Name: Protection of user data transported over N9 within a PLMN.

Requirement Reference: TS 33.501 [2], Clause 9.3

Requirement Description: As specified in clause 9.9 in TS 33.501 [2], "Interfaces internal to the 5G Core can be used to transport signalling data as well as privacy sensitive material, such as user and subscription data, or other parameters, such as security keys. Therefore, confidentiality and integrity protection is required.

For the protection of the non-SBA internal interfaces, such as N4 and N9, NDS/IP shall be used as specified in [3]."

Threat Reference: TR 33.926 [7], Clause L.2.2, "No protection or weak protection for user plane data "

TEST CASE:

Test Name: TC_UP_DATA_CONF_UPF_N9

Purpose:

Verify that the protection mechanism implemented for user data transport over N9 interface in a PLMN conforms to the selected security profile.

Procedure and execution steps:

Pre-Condition:

-	UPF network products are connected in simulated/real network environment.

-	The tunnel mode IPsec ESP and IKE certificate authentication is implemented.

-	Tester shall have knowledge of the security parameters of tunnel for decrypting the ESP packets.

-	Tester shall have access to the N9 interface between two UPFs within a PLMN.

-	Tester shall have knowledge of the confidentiality algorithm and confidentiality protection keys used for encrypting the encapsulated payload.

Execution Steps:

The requirement mentioned in this clause is tested in accordance with the procedure mentioned in clause 4.2.3.2.4 of TS 33.117 [3].

Expected Results:

The user data transported on N9 within a PLMN is protected.

Expected format of evidence:

Evidence suitable for the interface, e.g., evidence can be presented in the form of screenshot/screen-capture.

#### 4.2.2.5	Signalling Data Protection

Requirement Name: Protection of signalling data transported over N4 interface.

Requirement Reference: TS 33.501 [2], Clause 9.9

Requirement Description:  As specified in clause 9.9 in TS 33.501 [2], "Interfaces internal to the 5G Core can be used to transport signalling data as well as privacy sensitive material, such as user and subscription data, or other parameters, such as security keys. Therefore, confidentiality and integrity protection is required.

For the protection of the non-SBA internal interfaces, such as N4 and N9, NDS/IP shall be used as specified in [3]."

Threat Reference: TR 33.926 [7], Clause L.2.3, "No protection or weak protection for signalling data over N4 interface"

TEST CASE:

Test Name: TC_CP_DATA_CONF _UPF_N4

Purpose:

Verify that the protection mechanism implemented for signalling data transmitted over N4 conforms to selected security profile.

Procedure and execution steps:

Pre-Condition:

-	UPF and SMF network products are connected in simulated/real network environment.

-	The tunnel mode IPsec ESP and IKE certificate authentication is implemented.

-	Tester shall have knowledge of the security parameters of tunnel for decrypting the ESP packets.

-	Tester shall have access to the N4 interface between SMF and UPF.

-	Tester shall have knowledge of the confidentiality algorithm and confidentiality protection keys used for encrypting the encapsulated payload.

Execution Steps:

The requirement mentioned in this clause is tested in accordance with the procedure mentioned in clause 4.2.3.2.4 of TS 33.117 [3].

Expected Results:

The signalling data transported over N4 interface is protected.

Expected format of evidence:

Evidence suitable for the interface, e.g., evidence can be presented in the form of screenshot/screen-capture.

#### 4.2.2.6	TEID uniqueness

Requirement Name: TEID uniqueness.

Requirement Reference:

TS 23.501 [4], Clause 5.8.2.3.1; TS 29.281 [5], Clause 5.1; TS 23.060 [6], Clause 14.6

Requirement Description:

"Allocation and release of CN Tunnel Info is performed when a new PDU Session is established or released. This functionality is supported either by SMF or UPF, based on operator’s configuration on the SMF" as specified in TS 23.501[4], clause 5.8.2.3.1.

"Tunnel Endpoint Identifier (TEID): This field unambiguously identifies a tunnel endpoint in the receiving GTP U protocol entity. The receiving end side of a GTP tunnel locally assigns the TEID value the transmitting side has to use" as specified in TS 29.281[5], clause 5.1.

"The TEID is a unique identifier within one IP address of a logical node." As specified in TS 23.060 [6], clause 14.6.

Threat Reference: TR 33.926 [7], Clause L.2.4, "Failure to assign unique TEID for a session"

TEST CASE:

Test Name: TC_TEID_ID_UNIQUENESS_UPF

Purpose:

Verify that the TEID generated by UPF under test for each new GTP tunnel is unique.

Pre-Conditions:

Test environment is set up with SMF, which may be real or simulated, and UPF under test. The tester is able to trace traffic between the UPF under test and the SMF (real or simulated). SMF configures UPF under test to generate the TEIDs.

Execution Steps:

1)	The tester intercepts the traffic between the UPF under test and the SMF.

2)	The tester triggers the maximum number of concurrent N4 session establishment requests.

3)	The tester captures the N4 session establishment responses sent from UPF to SMF and verifies that the F-TEID created for each generated response is unique.

Expected Results:

The F-TEID set in each different N4 session establishment response is unique.

Expected format of evidence:

Files containing the triggered GTP messages (e.g. pcap trace).

*************** Start of the 3rd Change ****************

## 4.3	UPF-specific adaptations of hardening requirements and related test cases

### 4.3.1	Introduction

This clause specifies the UPF-specific adaptations of hardening requirements and related test cases.

### 4.3.2	Technical baseline

There are no UPF-specific additions to clause 4.3.2 in TS 33.117 [3].

### 4.3.3	Operating systems

There are no UPF-specific additions to clause 4.3.3 in TS 33.117 [3].

### 4.3.4	Web Servers

There are no UPF-specific additions to clause 4.3.4 in TS 33.117 [3].

### 4.3.5	Network Devices

There are no UPF-specific additions to clause 4.3.5 in TS 33.117 [3].

### 4.3.x	Network Functions in service-based architecture

There are no UPF-specific additions to clause 4.3.6 in TS 33.117 [3].

*************** End of the Changes ****************