3GPP TSG-SA3 Meeting # 97	S3-194345

# Reno,US, 18 - 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.514 | CR | 0001 | rev | - | Current version: | 16.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for clean-up and alignment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G |  |  |  |  |  | Date: |  |  | 2019-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The sub-clauses in 4.3 need to be aligned with those in TS 33.117 R16. The clause numbers of TR 33.926 referenced in the test cases need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added a new sub-clause (4.3.x) in clause 4.3 for structure alignment with TS 33.117 R16. - Updated the clause numbers of TR 33.926 referenced in the test cases for alignment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mis-alignment |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1.1, 4.2.2.1, 4.2.2.2, new sub-clause 4.3.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the 1st Change ****************

### 4.2.1	User Privacy Procedure

#### 4.2.1.1	De-concealment of SUPI from the SUCI based on the protection scheme used to generate the SUCI.

Requirement Name: De-concealment of SUPI from the SUCI based on the protection scheme used to generate the SUCI.

Requirement Reference: TS 33.501 [2], clause 5.8.2.

Requirement Description: "The SIDF shall resolve the SUPI from the SUCI based on the protection scheme used to generate the SUCI." as specified in TS 33.501 [2], clause 5.8.2.

Threat References: TR 33.926 [4], clause E.2.2.1, Incorrect SUCI de-concealment.

TEST CASE:

Test Name: TC_DE-CONCEAL_SUPI_from_SUCI_UDM

Purpose:

Verify that the SIDF De-conceals the SUPI from the SUCI based on the protection scheme used to generate the SUCI.

Procedure and execution steps:

Pre-Condition:

-	UDM network product is connected in simulated/real network environment.

-	Tester shall have access to the subscription data stored in UDR.

-	Tester shall record the SUPI from the UE.

Execution Steps:

Test case:

Tester shall capture the entire authentication procedure between UE and AMF over N1, N12 and N13 interface using any network analyser.

1.	Tester shall filter the Nudm_Authentication_Get Response message sent from UDM to AUSF over N13 interface containing the SUPI.

2.	Tester shall compare the SUPI gotten from UE and the SUPI retrieved from Nudm_Authentication_Get Response message.

Expected Results:

SIDF resolves the SUPI from the SUCI based on the protection scheme used to generate the SUCI.

Expected format of evidence:

Evidence suitable for the interface, e.g., evidence can be presented in the form of screenshot/screen-capture.

### 4.2.2 	Authentication and key agreement procedure

#### 4.2.2.1	Synchronization failure handling

Requirement Name: Synchronization failure handling

Requirement Reference: TS 33.501 [2], clause 6.1.3.3.2.

Requirement Description: "When the UDM/ARPF receives an Nudm_UEAuthentication_Get Request message with a "synchronisation failure indication" it acts as described in TS 33.102 [9], clause 6.3.5 where ARPF is mapped to HE/AuC. The UDM/ARPF sends an Nudm_UEAuthentication_Get Response message with a new authentication vector for either EAP-AKA' or 5G-AKA depending on the authentication method applicable for the user to the AUSF.as specified in TS 33.501 [2], clause 6.1.3.3.2.

Threat References: TR 33.926 [4], clause E.2.2.2, Synchronization failure.

Test Case:

Purpose:

Verify that synchronization failure is recovered correctly in the home network.

Pre-Conditions:

Test environment with AUSF. The AUSF may be simulated.

Execution Steps

1.	The AUSF sends an Nudm_UEAuthentication_Get Request message to the UDM with a "synchronisation failure indication" and parameters RAND and AUTS.

2.	The UDM/ARPF performs steps 1-5 as described in TS 33.102, clause 6.3.5.

Expected Results:

The UDM sends an Nudm_UEAuthentication_Get Response message with a new authentication vector to the AUSF.

#### 4.2.2.2	Storing of authentication status of UE by UDM.

Requirement Name: Storing of authentication status of UE by UDM.

Requirement Reference: TS 33.501 [2], clause 6.1.4.1a

Requirement Description: "The UDM shall store the authentication status of the UE (SUPI, authentication result, timestamp, and the serving network name) after authentication" as specified in TS 33.501 [2], clause 6.1.4.1a.

Threat References: TR 33.926 [4], clause E.2.2.3, Failure to store of authentication status.

TEST CASE:

Test Name: TC_AUTH_STATUS_STORE_UDM

Purpose:

Verify that the UDM stores the authentication status of UE

Procedure and execution steps:

Pre-Condition:

-	UDM network product is connected in simulated/real network environment.

-	Tester shall have access to all the authentication specific data sent over N1 interface, N12 interface and N13 interface.

-	Tester shall have access to UDM.

Execution Steps:

1.	Tester shall capture the entire authentication procedure between UE and AMF over N1, N12 and N13 interface using any network analyser.

2.	Tester shall filter the Nudm_Authentication_Get Response message sent over N13 interface to find the SUPI.

3.	Tester shall filter the Nausf_UEAuthentication_Authenticate Response message sent over N12 interface to retrieve the Authentication status (EAP success/failure for EAP-AKA' or Result for 5G AKA) from the Nausf_UEAuthentication_Authenticate Response message.

4.	Tester shall verify the status retrieved authentication status with the authentication status stored in UDM.

5.	Tester shall compare the SUPI present in the Nudm_Authentication_Get Response message and SUPI stored in the UDM.

6.	Tester shall filter the Nudm_UEAuthentication_Get Request message sent over the N13 interface.

7.	Tester shall compare the Serving network name present in the Nudm_Authentication_Get Request message with the serving network name stored in UDM.

Expected Results:

The storing of authentication status (SUPI, authentication result, timestamp, and the serving network name) of UE at the UDM is verified.

Expected format of evidence:

Evidence suitable for the interface, e.g., evidence can be presented in the form of screenshot/screen-capture.

*************** Start of the 2nd Change ****************

## 4.3	UDM-specific adaptations of hardening requirements and related test cases

### 4.3.1	Introduction

The present clause contains UDM-specific adaptations of hardening requirements and related test cases.

### 4.3.2	Technical Baseline

There are no UDM-specific additions to clause 4.3.2 of TS 33.117 [3].

### 4.3.3	Operating Systems

There are no UDM-specific additions to clause 4.3.3 of TS 33.117 [3].

### 4.3.4	Web Servers

There are no UDM-specific additions to clause 4.3.4 of TS 33.117 [3].

### 4.3.5	Network Devices

There are no UDM-specific additions to clause 4.3.5 of TS 33.117 [3].

### 4.3.x	Network Functions in service-based architecture

There are no UDM-specific additions to clause 4.3.6 in TS 33.117 [3].

*************** End of the Changes ****************