3GPP TSG-SA3 Meeting # 97	S3-194373

# Reno,US, 18 - 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | <CR#> | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Draft CR] F1 interface set-up procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB |  |  |  |  |  | Date: |  |  | 11-11-2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce the F1 interface set-up procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Details of F1 interface set-up procedure is proposed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Security aspects of IAB are not supported in 5GS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X.2.3.1 (new), X.2.3.2 (new), X.2.3.3 (new), Annex A.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | To be implemented on top of baseline draft CR (S3-193808) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### **** START OF CHANGES ****

## X.2.2 	Security mechanisms for F1 interface between the IAB-node (gNB-DU) and the IAB-donor-CU

### X.2.3.1	General

The following clause applies to F1 interface between the IAB-node and the IAB-donor.

### X.2.3.2	Security mechanisms for the F1 interface

The F1 interface connects the IAB-node (gNB-DU) to the IAB-donor-CU. It consists of the F1-C for control plane and the F1-U for the user plane.

In order to protect the traffic on the F1-U interface, IPsec ESP and IKEv2 certificates-based authentication shall be supported as specified in sub-clause 9.1.2 of the present document with confidentiality, integrity and replay protection.

In order to protect the traffic on the F1-C interface, IPsec ESP and IKEv2 certificates-based authentication shall be supported as specified in sub-clause 9.1.2 of the present document with confidentiality, integrity and replay protection.

In order to protect the traffic on the F1-U interface, IPsec ESP and IKEv2 Pre-shared Secret Key (PSK) authentication method shall be supported with confidentiality, integrity and replay protection. IKEv2 Pre-shared Secret Key (PSK) authentication implementation shall be done according to TS 33.310 [5].  The IAB-node and the IAB-donor calculates the PSK (KIAB) as specified in the Annex A.y of this document.

In order to protect the traffic on the F1-C interface, IPsec ESP and IKEv2 Pre-shared Secret Key (PSK) authentication method shall be supported with confidentiality, integrity and replay protection. IKEv2 Pre-shared Secret Key (PSK) authentication implementation shall be done according to TS 33.310 [5]. The IAB-node and the IAB-donor calculates the PSK (KIAB) as specified in the Annex A.y of this document.

IPsec is mandatory to implement on the IAB-node and on the IAB-donor.

In addition to IPsec, for the F1-C interface, DTLS shall be supported as specified in RFC 6083 [58] to provide integrity protection, replay protection and confidentiality protection. Security profiles for DTLS implementation and usage shall follow the provisions given in clause 6.2 of TS 33.210 [3].

NOTE 1: 	The use of transport layer security, via DTLS, does not rule out the use of network layer protection according to NDS/IP as specified in TS 33.210 [3]. In fact, IPsec has the advantage of providing topology hiding.

NOTE 2: 	The use of cryptographic solutions to protect F1 is an operator's decision. In case the IAB has been supported in a physically secured environment then the 'secure environment' includes other nodes and links beside the IAB-node.

NOTE 3: 	The security considerations for DTLS over SCTP are documented in RFC 6083 [58].

### X.2.3.3	IAB-node migration procedure

Editor's note:	Topology adaptation, where the IAB-node migrates from its source parent node to a target parent node is under consideration in RAN WGs. Security aspects of this procedure will be included when RAN WGs have made further progress.

**** NEXT CHANGE ****

# A.y	KIAB generation function

This input string is used when the IAB-node and the IAB-donor derive KIAB (PSK) for establishment of secure F1 interface. The following parameters shall be used to form the input S to the KDF:

-	FC = 0xaa,

-	P0 = IAB-donor-CU IP address

-	L0 = length of IAB-donor-CU IP address

- 	P1 = IAB-node DU IP address

-	L1 = IAB-node DU IP address

The input key Key shall be KgNB. The key KgNB is in possession of the IAB-UE functionality in the IAB-node and in the IAB-donor-CU, after the IAB-UE setup procedure (Phase-1).

The entire output of the KDF (256 bits) is used as the KIAB.

**** END OF CHANGES ****