3GPP TSG-SA3 Meeting #97 	S3-194393

# Reno,US, 18 – 22 November 2019					Revision of S3-19xxxx

# Source:	Samsung

# Title:	Security procedure for S-KMC and S-KMS

# Document for:	Approval

Agenda Item:	7.15

# 1	Decision/action requested

This document proposes the security procedure for S-KMC and S-KMS for the new TS on SEAL TS 33.434. SA3 is kindly requested to approve this contribution.

# 2	References

[1]	3GPP TS 23.434 Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows.

[2]	SP-190901 New WID on Security aspects of SEAL.

# 3	Rationale

As S3-19xxxx proposes the skeleton for SEAL TS, it is proposed to approve this contribution to add details on security procedure for S-KMC and S-KMS scope for the TS. Details on Annex Y is proposed in S3-19xxxx.

# 4	Detailed proposal

*****Start of 1st Change*****

# 2	References

[1]	IETF RFC 6507: ''Elliptic Curve-Based Certificateless Signatures for Identity-Based Encryption (ECCSI)''.

[2]	IETF RFC 6508: ''Sakai-Kasahara Key Encryption (SAKKE)''.

[5]	IETF RFC 6509: ''MIKEY-SAKKE: Sakai-Kasahara Key Encryption in Multimedia Internet KEYing (MIKEY)''.

*****End of 1st Change*****

*****Start of 2nd Change*****

## 6.Y       Procedures and information flows for key management

### 6.Y.1       Functionality of the S-KMC and S-KMS

#### 6.Y.1.1       Security procedure for Key management

The procedure for the provision of identity-specific key material when the HTTP proxy is supported between the S-KMS and the S-KMC is described in figure 6.Y.1.1-1. The procedure is the same whether the key management client in the VAL UE, a VAL Server or a Group Management Server is making the request.

Figure 6.Y.1.1-1: Provisioning of key material via the HTTP proxy

The procedure in figure 6.Y.1.1-1 is now described step-by-step.

0)	The key management client establishes a connection to the S-KMS. As with other elements in the SEAL functional model, the connection is routed via, and secured by, the HTTP Proxy.

1)	The key management client makes a request for user key material from the S-KMS. The request contains an access token to authenticate the user as defined in clause 5.X.1. There are the following types of request (as defined in Annex X):

a)	S-KMSInit Request. This request is the first request sent to the S-KMS to setup the user. This type of request is permitted to the Home S-KMS or to Migration S-KMSs.

b)	S-KMSKeyProv Request: This request is to obtain new key material from the S-KMS. The request may contain details of a specific identity required for key management, and may contain a specific time for which the key material is required. This type of request is permitted to the Home S-KMS or to Migration S-KMSs.

c)	S-KMSCertCache Request: This request is to obtain external S-KMS certificates associated with external security domains (managed by another S-KMS). The request may contain details of the latest version of the cache received by the client. This type of request shall only be made to the Home S-KMS.

d)	S-KMS Redirect Response (KRR) upload: This procedure uploads a KRR received by the client to the Home S-KMS. This type of message shall only be sent to the Home S-KMS.

e)	S-KMS Cert Request: This request is to obtain a single S-KMS certificate based on a provided S-KMS URI.

f)	S-KMS Lookup: This message is to lookup the external S-KMS that should be used for a provided SIP URI.

g)	S-KMS Redirect Upload: This message is to upload a received discovery request response to the S-KMS for audit purposes.

2)	The S-KMS provides a response based upon the authenticated user and the user's request. For public safety use, the key material itself shall be encrypted using a 256-bit transport key (S-TrK). The response may also be signed by the S-TrK or the S-InK. The S-TrK and S-InK are initially distributed via an out-of-band mechanism along with their 32-bit identifiers, the S-TrK-ID and S-InK-ID, respectively. The responses are:

a)	S-KMSInit Response. This response contains domain parameters and optionally, a new S-TrK and/or a new S-InK.

b)	S-KMSKeyProv Response: This response provides new key material to the user and optionally, a new S-TrK and/or a new S-InK.

c)	S-KMSCertCache Response: This response contains new or updated home S-KMS certificates and/or external S-KMS certificates required by the user for communications with external security domains.

d)	S-KMS Cert Response: This response is an S-KMSCertCache Response containing a single S-KMS Certificate (or an error message).

e)	S-KMS Lookup Response: This response is to provide the client with information related to a Discovery Lookup request. Either the S-KMS URI that should be used for a user is provided, or permission is provided to use a specific External S-KMS.

The procedure for the provisioning of identity-specific key material when the HTTP proxy is not used between the S-KMS and the S-KMS client is as described in Figure 6.Y.1.1-2.

Figure 6.Y.1.1-2: Provisioning of key material without a proxy

The procedure in Figure 6.Y.1.1-2 is now described step-by-step:

0)	The key management client establishes a direct HTTPS connection to the S-KMS. The following message flow is within this secure connection.

1)	The key management client makes a request to the S-KMS. The same requests can be made as defined above with a proxy.

2)	The S-KMS provides a response based upon the authenticated user and the user's request. Optionally, the key material itself may also be encrypted using a 256-bit transport key (S-TrK). The response may also be signed using the S-TrK or the S-InK. The S-TrK and S-InK are initially distributed via an out-of-band mechanism along with their 32-bit identifiers (S-TrK-ID and S-InK-ID respectively).

As a result of this procedure, the key management client has securely obtained key material for use within the VAL system.

### 6.Y.2       Provisioning key material to support end-to-end communication security

End-to-end communication security for either group or private calls requires the provisioning of key material from the S-KMS. The key material provisioned to each user is listed below:

-	A S-KMSInit Response contains the KMS Certificate (domain specific key material associated to the S-KMS), and may contain:

-	An updated S-TrK for the user (to replace the offline-provisioned, bootstrap S-TrK).

-	Policy around the use of S-KMS key material (Home S-KMS only)

-	Address to which ‘S-KMSCertCache' requests should be sent.

-	Address to which ‘S-KRRupload' messages should be sent.

-	A S-KMSKeyProv Response contains zero, or more, S-KMSKeySets and may contain:

-	An updated S-TrK for the user (to replace existing S-TrK).

-	A S-KMSCertCache Response may contain:

-	The S-KMS's Certificate(s) (current, updated or future).

-	Migration S-KMSs (S-KMS URIs, access addresses, provisional S-TrKs).

-	External S-KMS Certificates. This is domain specific key material associated with other S-KMSs. It is required to enable secure communications across security domains.

-	A S-KMSCert Response may contain:

-	An External S-KMS Certificate. This is domain specific key material associated with the requested S-KMS URI. It is required to enable secure communications across security domains.

-	A S-KMS Lookup Response does not contain key material, but may contain S-KMS URIs.

### 6.Y.3	S-KMS Certificate

A S-KMS Certificate is defined in Annex X. An S-KMS Certificate contains the following:

-	A Role of 'Home' or 'External', depending on whether the certificate is the issuing S-KMS's or is provided by another external S-KMS.

-	The KMS Public Authentication Key (KPAK in IETF RFC 6507 [1]).

-	The KMS Public Confidentiality Key (Z_T in IETF RFC 6508 [2]).

-	The UID conversion (as described below).

-	Choice of cryptographic domain parameters (such as those listed in IETF RFC 6509 [3]).

-	The time period for which this information is valid.

Certificates are identified by the S-KMS (S-KMSUri) and a unique identifier (CertUri). A (logical) S-KMS should only have a single S-KMS certificate active at any one time (based upon the S-KMSUri). Certificates may be updated using the CertURI. Should a client receive a certificate with a CertURI of an existing certificate, the client shall replace this existing certificate with the newly provisioned certificate.

The UID conversion mechanism defines how UIDs are generated. Using this information a VAL client can take a user identifier, and the current time, (e.g. the year and month) and convert these to a UID.

EXAMPLE:	UID = Hash (VAL service ID, KMS URI, validity period info).

As a consequence, there is a one-to-one correspondence between VAL Service IDs and UIDs during each time period.

### 6.Y.4	S-KMS provisioned Key Set

S-KMSKeySet(s) are defined in Annex X and contain the following:

-	A user signing key for each UID for the current time period (SSK and PVT in IETF RFC 6507 [1]).

-	A user decryption key for each UID for the current time period (RSK in IETF RFC 6508 [2]).

-	The key period number associated with the current keys.

-	Optionally, the time period, for which the user key material is valid (e.g. month).

Editor's Note:	Key generation and key distribution is FFS.

***** End of 2nd Changes *****