3GPP TSG-SA3 Meeting # 97	S3-194596

# Reno,US, 18 - 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | <CR#> | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Draft CR]Solution for IAB Architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB |  |  |  |  |  | Date: |  |  | 7-10-2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Updates to amend the 5G system security architecture and procedures to support security of Integrated Access and Backhaul for NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause and also updates to other clauses to introduce the IAB security. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Security aspects of IAB are not supported in 5GS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2, 5.3.9, Annex (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3-193808: Baseline version from SA3#96AH S3-194596: Baseline version from SA3#97 |  |  |  |  |  |  |  |  |

### **** START OF CHANGES ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[4]	IETF RFC 4303: "IP Encapsulating Security Payload (ESP)".

[5]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[6]	IETF RFC 4301: "Security Architecture for the Internet Protocol".

[7]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[8]	3GPP TS 23.502: "Procedures for the 5G System".

[9]	3GPP TS 33.102: "3G security; Security architecture".

[10]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[11]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE); Security aspects of non-3GPP accesses".

[12]	IETF RFC 5448: " Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA')".

Editor’s note:	This reference will be removed and references to it updated when the IETF updates the RFC and publishes a new RFC that supercedes this RFC.

[13]	3GPP TS 24.301: " Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[14]	3GPP TS 35.215: " Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 1: UEA2 and UIA2 specifications".

[15]	NIST: "Advanced Encryption Standard (AES) (FIPS PUB 197)".

[16]	NIST Special Publication 800-38A (2001): "Recommendation for Block Cipher Modes of Operation".

[17]	NIST Special Publication 800-38B (2001): "Recommendation for Block Cipher Modes of Operation: The CMAC Mode for Authentication".

[18]	3GPP TS 35.221: " Specification of the 3GPP Confidentiality and Integrity Algorithms EEA3 & EIA3; Document 1: EEA3 and EIA3 specifications".

[19]	3GPP TS 23.003: "Numbering, addressing and identification".

[20]	3GPP TS 22.101: "Service aspects; Service principles".

[21]	IETF RFC 4187: "Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA)".

[22]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[23]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[24]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[25]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)"

[26]	IETF RFC 4303: "IP Encapsulating Security Payload (ESP)"

[27]	IETF RFC 3748: "Extensible Authentication Protocol (EAP)".

[28]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[29]	SECG SEC 1: Recommended Elliptic Curve Cryptography, Version 2.0, 2009. Available http://www.secg.org/sec1-v2.pdf

[30]	SECG SEC 2: Recommended Elliptic Curve Domain Parameters, Version 2.0, 2010. Available at http://www.secg.org/sec2-v2.pdf

[31]	3GPP TS 38.470: "NG-RAN; F1 General aspects and principles".

[32]	3GPP TS 38.472: "NG-RAN; F1 signalling transport".

[33] 	3GPP TS 38.474: "NG-RAN; F1 data transport".

[34]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)"

[35]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[36] 	3GPP TS 35.217: "Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 3: Implementors' test data".

[37] 	3GPP TS 35.223: "Specification of the 3GPP Confidentiality and Integrity Algorithms EEA3 & EIA3; Document 3: Implementors' test data".

[38]	IETF RFC 5216: "The EAP-TLS Authentication Protocol".

[39]	IETF RFC 4346: "The Transport Layer Security (TLS) Protocol Version 1.1".

[40]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[41]	3GPP TS 38.460: "NG-RAN; E1 general aspects and principles".

[42]	Void.

[43]	IETF RFC 6749: "OAuth2.0 Authorization Framework".

[44]	IETF RFC 7519: "JSON Web Token (JWT)".

[45]	IETF RFC 7515: "JSON Web Signature (JWS)".

[46]	IETF RFC 7748: "Elliptic Curves for Security".

[47]	IETF RFC 7540: " Hypertext Transfer Protocol Version 2 (HTTP/2)".

[48]	IETF RFC 5280: "Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List (CRL) Profile".

[49]	IETF RFC 6960: "X.509 Internet Public Key Infrastructure Online Certificate Status Protocol - OCSP".

[50]	IETF RFC 6066: "Transport Layer Security (TLS) Extensions: Extension Definitions".

[51]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[52]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[53]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[54]	3GPP TS28.533: " Management and orchestration; Architecture framework".

[55]	3GPP TS28.531: "Management and orchestration of networks and network slicing; Provisioning".

[56]	IETF RFC 4279 "Pre-Shared Key Ciphersuites for Transport Layer Security (TLS)".

[57]	IETF RFC 7542: "The Network Access Identifier".

[58]	IETF RFC 6083: " Datagram Transport Layer Security (DTLS) for Stream Control Transmission Protocol (SCTP)".

[59]	IETF RFC 7516: "JSON Web Encryption (JWE)".

[60]	IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3".

[61]	IETF RFC 5705,"Keying Material Exporters for Transport Layer Security (TLS)".

[62]	IETF RFC 5869 "HMAC-based Extract-and-Expand Key Derivation Function (HKDF)".

[63]	NIST Special Publication 800-38D: "Recommendation for Block Cipher Modes of Operation: Galois Counter Mode (GCM) and GMAC".

[64]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[65]	3GPP TS 31.115: "Secured packet structure for (Universal) Subscriber Identity Module (U)SIM Toolkit applications.

[66]	3GPP TS 31.111: "Universal Subscriber Identity Module (USIM), Application Toolkit (USAT)".

[67]	Internet draft draft-ietf-emu-rfc5448bis: "Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA')".

[68]	3GPP TS 29.510: "5G System; Network function repository services".

[69]	3GPP TS 36.331: "Radio Resource Control (RRC); Protocol specification".

[aa]	3GPP TS 38.401: "NG-RAN; Architecture description".

**** NEXT CHANGE ****

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5G security context: The state that is established locally at the UE and a serving network domain and represented by the "5G security context data" stored at the UE and a serving network.

NOTE 1:	The "5G security context data" consists of the 5G NAS security context, and the 5G AS security context for 3GPP access and/or the 5G AS security context for non-3GPP access.

NOTE 2:	A 5G security context has type "mapped", "full native" or "partial native". Its state can either be "current" or "non-current". A context can be of one type only and be in one state at a time. The state of a particular context type can change over time. A partial native context can be transformed into a full native. No other type transformations are possible.

5G AS security context for 3GPP access: The cryptographic keys at AS level with their identifiers, the Next Hop parameter (NH), the Next Hop Chaining Counter parameter (NCC) used for next hop access key derivation, the identifiers of the selected AS level cryptographic algorithms, the UE security capabilities, and the UP Security Policy at the network side, UP security activation status and the counters used for replay protection.

NOTE 3:	NH and NCC need to be stored also at the AMF during connected mode.

NOTE 4:	UP security activation status is sent from gNB/ng-eNB in step 1b in clause 6.6.2 corresponding to the active PDU session(s).

5G AS security context for non-3GPP access: The key KN3IWF, the cryptographic keys, cryptographic algorithms and tunnel security association parameters used at IPsec layer for the protection of IPsec SA.

5G AS Secondary Cell security context: The cryptographic keys at AS level for secondary cell with their identifiers, the identifier of the selected AS level cryptographic algorithms for secondary cell, the UP Security Policy at the network side, and counters used for replay protection.

5G Home Environment Authentication Vector: authentication data consisting of RAND, AUTN, XRES*, and KAUSF for the purpose of authenticating the UE using 5G AKA.

NOTE 3a: This vector is received by the AUSF from the UDM/ARPF in the Nudm_Authentication_Get Response.

5G Authentication Vector: authentication data consisting of RAND, AUTN, HXRES*, and KSEAF.

NOTE 3b: This vector is received by the SEAF from the AUSF in the Nausf_Authentication_Authenticate Response.

5G NAS security context: The key KAMF with the associated key set identifier, the UE security capabilities, the uplink and downlink NAS COUNT values.

NOTE 4:	The distinction between native 5G security context and mapped 5G security context also applies to 5G NAS security contexts. The 5G NAS security context is called "full" if it additionally contains the integrity and encryption keys and the associated identifiers of the selected NAS integrity and encryption algorithms.

5G Serving Environment Authentication Vector: a vector consisting of RAND, AUTN and HXRES*.

ABBA parameter: Parameter that provides antibidding down protection of security features against security features introduced in higher release to a lower release and indicates the security features that are enabled in the current network.

activation of security context: The process of taking a security context into use.

anchor key: The security key KSEAF provided during authentication and used for derivation of subsequent security keys.

applicaton Layer Security: mechanism by which HTTP messages, exchanged between a Network Function in one PLMN and a Network Function in another PLMN, are protected on the N32-f interface between the two SEPPs in the two PLMNs.

authentication data: An authentication vector or transformed authentication vector.

authentication vector: A vector consisting of CK, IK, RAND, AUTN, and XRES.

backward security: The property that for an entity with knowledge of Kn, it is computationally infeasible to compute any previous Kn-m (m>0) from which Kn is derived.

NOTE 5:	In the context of KgNB key derivation, backward security refers to the property that, for a gNB with knowledge of a KgNB, shared with a UE, it is computationally infeasible to compute any previous KgNB that has been used between the same UE and a previous gNB.

CM-CONNECTED state: This is as defined in TS 23.501 [2].

NOTE5a:	The term CM-CONNECTED state corresponds to the term 5GMM-CONNECTED mode used in TS 24.501 [35].

CM-IDLE state: As defined in TS 23.501 [2].

NOTE5b:	The term CM-IDLE state corresponds to the term 5GMM-IDLE mode used in TS 24.501 [35].

consumer's IPX (cIPX): IPX provider entity with a business relationship with the cSEPP operator.

consumer's SEPP (cSEPP): The SEPP residing in the PLMN where the service consumer NF is located.

current 5G security context: The security context which has been activated most recently.

NOTE5c:	A current 5G security context originating from either a mapped or native 5G security context can exist simultaneously with a native non-current 5G security context.

forward security: The fulfilment of the property that for an entity with knowledge of Km that is used between that entity and a second entity, it is computationally infeasible to predict any future Km+n (n>0) used between a third entity and the second entity.

NOTE 6:	In the context of KgNB key derivation, forward security refers to the property that, for a gNB with knowledge of a KgNB, shared with a UE, it is computationally infeasible to predict any future KgNB that will be used between the same UE and another gNB. More specifically, n hop forward security refers to the property that a gNB is unable to compute keys that will be used between a UE and another gNB to which the UE is connected after n or more handovers (n=1 or more).

full native 5G security context: A native 5G security context for which the 5G NAS security context is full according to the above definition.

NOTE6a:	A full native 5G security context is either in state "current" or state "non-current".

Home Network Identifier: An identifier identifying the home network of the subscriber.

NOTE6b: Described in detail in TS 23.003 [19].

Home Network Public Key Identifier: An identifier used to indicate which public/private key pair is used for SUPI protection and de-concealment of the SUCI.

NOTE6c: Described in this document and detailed in TS 23.003 [19].

IAB-donor-CU: As defined in TS 38.401 [aa].

IAB-donor-DU: As defined in TS 38.401 [aa].

IAB-node: As defined in TS 38.300 [52].

IAB-donor gNB: As defined in TS 38.300 [52].

mapped 5G security context: An 5G security context, whose KAMF was derived from EPS keys during interworking and which is identified by mapped ngKSI.

Master node: As defined in TS 37.340 [51].

N32-c connection: A TLS based connection between a SEPP in one PLMN and a SEPP in another PLMN.

NOTE 6d:	This is a long-lived connection that’s used between the SEPPs for cipher suite and protection policy exchange, and error notifications.

N32-f connection: Logical connection that exists between a SEPP in one PLMN and a SEPP in another PLMN for exchange of protected HTTP messages.

NOTE 6e:	When IPX providers are present in the path between the two SEPPs, an N32-f HTTP connection is setup on each hop towards the other SEPP.

native 5G security context: An 5G security context, whose KAMF was created by a run of primary authentication and which is identified by native ngKSI.

ng-eNB: As defined in TS 38.300 [52].

NG-RAN node: gNB or ng-eNB (as defined in TS 38.300 [52]).

non-current 5G security context: A native 5G security context that is not the current one.

NOTE 7:	A non-current 5G security context may be stored along with a current 5G security context in the UE and the AMF. A non-current 5G security context does not contain 5G AS security context. A non-current 5G security context is either of type "full native" or of type "partial native".

partial native 5G security context: A partial native 5G security context consists of KAMF with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values, which are initially set to zero before the first NAS SMC procedure for this security context.

NOTE 8:	A partial native 5G security context is created by primary authentication, for which no corresponding successful NAS SMC has been run. A partial native context is always in state "non-current".

producer's IPX (pIPX): IPX provider entity with a business relationship with the pSEPP operator.

producer's SEPP (pSEPP): The SEPP residing in the PLMN where the service producer NF is located.

Protection Scheme Identifier: An identifier identifying a protection scheme that is used for concealing the SUPI.

RM-DEREGISTERED state: This is as defined in TS 23.501 [2].

NOTE8a:	The term RM-DEREGISTERED state corresponds to the term 5GMM-DEREGISTERED mode used in TS 24.501 [35].

RM-REGISTERED state: As defined in TS 23.501 [2].

NOTE8b:	The term RM-REGISTERED state corresponds to the term 5GMM-REGISTERED mode used in TS 24.501 [35].

Routing Indicator: An indicator defined in TS 23.003 [19] that can be used for AUSF or UDM selection.

Scheme Output: the output of a public key protection scheme used for SUPI protection.

security anchor function: The function SEAF that serves in the serving network as the anchor for security in 5G.

Secondary node: As defined in TS 37.340 [51].

subscription credential(s): The set of values in the USIM and the ARPF, consisting of at least the long-term key(s) and the subscription identifier SUPI, used to uniquely identify a subscription and to mutually authenticate the UE and 5G core network.

subscription identifier: The SUbscription Permanent Identifier (SUPI).

NOTE8c: As defined in TS 23.501 [2] and detailed in 23.003 [19].

subscription concealed identifier: A one-time use subscription identifier, called the SUbscription Concealed Identifier (SUCI), which contains the Scheme-Output, and additional non-concealed information needed for home network routing and protection scheme usage.

NOTE8d: Defined in the present document; detailed in TS 23.003 [19].

subscription identifier de-concealing function: The Subscription Identifier De-concealing Function (SIDF) service offered by the network function UDM in the home network of the subscriber responsible for de-concealing the SUPI from the SUCI.

transformed authentication vector: an authentication vector where CK and IK have been replaced with CK' and IK'.

UE 5G security capability: The UE security capabilities for 5G AS and 5G NAS.

UE security capabilities: The set of identifiers corresponding to the ciphering and integrity algorithms implemented in the UE.

NOTE 9:	This includes capabilities for NG-RAN and 5G NAS, and includes capabilities for EPS, UTRAN and GERAN if these access types are supported by the UE.

**** NEXT CHANGE ****

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5G-AN	5G Access Network

NG-RAN	5G Radio Access Network

5G AV	5G Authentication Vector

5G HE AV	5G Home Environment Authentication Vector

5G SE AV	5G Serving Environment Authentication Vector

ABBA	Anti-Bidding down Between Architectures

AEAD	Authenticated Encryption with Associated Data

AES	Advanced Encryption Standard

AKA	Authentication and Key Agreement

AMF	Access and Mobility Management Function

AMF	Authentication Management Field

NOTE:	If necessary, the full word is spelled out to disambiguate the abbreviation.

ARPF	Authentication credential Repository and Processing Function

AUSF	Authentication Server Function

AUTN	AUthentication TokeN

AV	Authentication Vector

AV'	transformed Authentication Vector

BAP	Backhaul Adaptation Protocol

BH	Backhaul

Cell-ID	Cell Identity as used in TS 38.331 [22]

cIPX	consumer's IPX

CP	Control Plane

cSEPP	consumer's SEPP

CTR	Counter (mode)

CU	Central Unit

DN	Data Network

DNN	Data Network Name

DU	Distributed Unit

EAP	Extensible Authentication Protocol

EMSK	Extended Master Session Key

EPS	Evolved Packet System

gNB	NR Node B

GUTI	Globally Unique Temporary UE Identity

HRES	Hash RESponse

HXRES	Hash eXpected RESponse

IAB	Integrated Access and Backhaul

IKE	Internet Key Exchange

IPX	IP exchange service

KSI	Key Set Identifier

LI	Lawful Intercept

MN	Master Node

MR-DC	Multi-Radio Dual Connectivity

MSK	Master Session Key

N3IWF	Non-3GPP access InterWorking Function

NAI	Network Access Identifier

NAS	Non Access Stratum

NDS	Network Domain Security

NEA	Encryption Algorithm for 5G

NF	Network Function

NG	Next Generation

ng-eNB	Next Generation Evolved Node-B

ngKSI	Key Set Identifier in 5G

NIA	Integrity Algorithm for 5G

NR	New Radio

NR-DC	NR-NR Dual Connectivity

NSSAI	Network Slice Selection Assistance Information

PDN	Packet Data Network

PEI	Permanent Equipment Identifier

pIPX	producer's IPX

PRINS	PRotocol for N32 INterconnect Security

pSEPP	producer's SEPP

QoS	Quality of Service

RES	RESponse

SCG	Secondary Cell Group

SEAF	SEcurity Anchor Function

SEG	Security Gateway

SEPP	Security Edge Protection Proxy

SIDF	Subscription Identifier De-concealing Function

SMC	Security Mode Command

SMF	Session Management Function

SN	Secondary Node

SN Id	Serving Network Identifier

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

TLS	Transport Layer Security

UE	User Equipment

UEA	UMTS Encryption Algorithm

UDM	Unified Data Management

UIA	UMTS Integrity Algorithm

ULR	Update Location Request

UP	User Plane

UPF	User Plane Function

USIM	Universal Subscriber Identity Module

XRES	eXpected RESponse

**** NEXT CHANGE ****

### 5.3.9	Requirements for the gNB and IAB F1 interfaces

Requirements given below apply to gNBs with split DU-CU implementations and also for the IAB using F1 interface defined in TS 38.470 [31]. Signalling traffic (i.e. both F1-C interface management traffic defined in TS 38.470 [31] and F1-C signalling bearer defined in TS 38.472 [32]) and user plane data can be sent on the F1 interface between a given DU and its CU.

-	F1-C interface shall support confidentiality, integrity and replay protection.

-	All management traffic carried over the CU-DU link shall be integrity, confidentiality and replay protected.

-	The gNB shall support confidentiality, integrity and replay protection on the gNB DU-CU F1-U interface [33] for user plane.

-	F1-C and management traffic carried over the CU-DU link shall be protected independently from F1-U traffic.

NOTE:	The above requirements allow to have F1-U protected differently (including turning integrity and/or encryption off or on for F1-U) from all other traffic on the CU-DU (e.g. the traffic over F1-C).

**** NEXT CHANGE ****

# Annex X (normative): Security for Integrated Access and Backhaul

# X.1	General

This Annex provides the security procedures applied to NR IAB architecture and functional entities for supporting wireless backhauling of NR base stations. Security requirements applied to IAB architecture are defined in the main body of the present specification.

The overall stage 2 description for IAB architecture and functional entities are described in 3GPP TS 23.501 [2] and 3GPP TS 38.401 [aa].

The security requirements and security procedures applied to IAB in EN-DC architecture are defined in TS 33.401 [10].

# X.2	Security requirements and features

## X.2.1	Requirements on the IAB-node (IAB-UE)

The IAB-node (IAB-UE) shall support ciphering, integrity protection and replay protection of NAS-signalling between the IAB-node (IAB-UE) and the 5GC supporting IAB architecture.

The IAB-node (IAB-UE) shall support ciphering, integrity protection and replay protection of RRC-signalling between the IAB-node (IAB-UE) and the IAB donor.

Mutual authentication between the IAB-node (IAB-UE) and the 5GC supporting IAB architecture shall be supported.

## X.2.2	Requirements on the IAB donor

The IAB donor shall support ciphering, integrity protection and replay protection of RRC-signalling between the IAB donor and the IAB-node (IAB-UE).

## X.2.3	Requirements on the 5GC supporting IAB architecture

The 5GC supporting IAB architecture shall support ciphering, integrity protection and replay protection of NAS-signalling between the 5GC supporting IAB architecture and the IAB-node (IAB-UE).

Mutual authentication between the 5GC supporting IAB architecture and the IAB-node (IAB-UE) shall be supported.

The 5GC shall decide whether the IAB-node is authorized to operate as IAB-node.

## X.2.4	Requirements for secure environment

The security requirements for secure environment of the IAB-node (gNB-DU) and the IAB-donor are described in clause 5.3.8.

## X.2.5	Requirements on the F1 interface

The security requirements on the F1 interface between the IAB-node (gNB-DU) and the IAB-donor-CU are described in clause 5.3.9.

# X.3	IAB-node Integration Procedure

## X.3.1	General

IAB-node, consists of a UE function (referred to as IAB-UE) and gNB-DU function [2]. IAB integration procedure consists of 3 phases detailed in TS 38.401 [aa].

Phase-1: IAB-UE part setup:

The IAB-UE performs registration procedure to the network as a UE as described in TS 23.501 [2] and TS 23.502 [8] in order to register to the 5GC and consequently, the NAS and AS security are established between the IAB-node and 5GC.

Phase-2: BH RLC channel establishment and routing update:

The BH RLC channels and the BAP layer are established and configured in the IAB-node by the IAB-donor using the secured RRC signalling to support routing between the IAB-node and the IAB-donor.

Phase-3: IAB-DU part setup:

F1 security establishment for IAB is performed over the RLC channel.

The Phase-1 results in IAB-UE registration and consequently, AS security establishment between the IAB donor and IAB node, Phase-2 results in configuration of the IAB-node securely using the established AS security and Phase-3 results in the establishment of secure F1 interface between the IAB-donor and IAB-node.

## X.3.2	Authentication and Authorization of IAB-node (Phase-1)

The IAB-UE function shall behave as a UE, and shall reuse the UE procedures specified in this document for the primary authentication (see clause 6), key derivation and distribution scheme, subscription credential(s) storage requirements, NAS security and AS security.

NOTE 1: For isolated deployment scenarios, Annex B describes how additional EAP methods can be used.

NOTE 2: Storage of subscription credentials for EAP AKA’ and 5G AKA is described in clause 6 of the present document.

Authorization of IAB-nodes shall be performed by the 5G core network supporting IAB architecture as described in TS 23.501 [2].

## X.3.3 	Security mechanisms for F1 interface between the IAB-node (gNB-DU) and the IAB-donor-CU  (Phase-3)

**** END OF CHANGES ****