3GPP TSG-SA3 Meeting # 97	S3-194633

# Reno,US, 18 - 22 November 2019													revision of S3-194377

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.401 | CR | 0687 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security aspects of RLOS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | PARLOS |  |  |  |  |  | Date: |  |  | 2019-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As per the conclusions in TR 33.815, specify security aspects of RLOS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new annex to include the security aspects of RLOS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Security aspects of RLOS not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the changes ****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[3]	3GPP TS 23.003: "Numbering, addressing and identification".

[4]	3GPP TS 33.102: "3G security; Security architecture".

[5]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[6]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[7]	IETF RFC 4303: "IP Encapsulating Security Payload (ESP)".

[8]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic bootstrapping architecture".

[9]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[10] – [11]	Void.

[12]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification"

[13]	3GPP TS 31.102: "Characteristics of the Universal Subscriber Identity Module (USIM) application".

[14]	3GPP TS 35.215: "Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 1: UEA2 and UIA2 specifications"

[15]	NIST: "Advanced Encryption Standard (AES) (FIPS PUB 197) "

[16]	NIST Special Publication 800-38A (2001): "Recommendation for Block Cipher Modes of Operation".

[17]	NIST Special Publication 800-38B (2001): "Recommendation for Block Cipher Modes of Operation: The CMAC Mode for Authentication".

[18] – [20]	Void.

[21]	3GPP TS 36.331:"Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC); Protocol specification".

[22]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[23]	3GPP TS 22.101: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Service aspects; Service principles".

[24]	3GPP TS 25.331: "3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Radio Resource Control (RRC); Protocol Specification ".

[25]	3GPP TS 44.060: "3rd Generation Partnership Project; Technical Specification Group GSM/EDGE Radio Access Network; General Packet Radio Service (GPRS); Mobile Station (MS) - Base Station System (BSS) interface; Radio Link Control/Medium Access Control (RLC/MAC) protocol.

[26]	3GPP TS 23.122: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[27]	3GPP TS 33.320: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Security of Home Node B (HNB) / Home evolved Node B (HeNB)".

[28]	(void)

[29]	ETSI TS  102 484 V10.0.0: "Smart Cards; Secure channel between a UICC and an end-point terminal".

[30]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[31]	3GPP TS 31.116 "Remote APDU Structure for (Universal) Subscriber Identity Module (U)SIM Toolkit applications".

[32]	ETSI TS 102 221 V9.2.0: "Smart Cards; UICC-Terminal interface; Physical and logical characteristics".

[33]	3GPP TS 35.221: "Confidentiality and Integrity Algorithms EEA3 & EIA3; Document 1: EEA3 and EIA3 specifications".

[34]	RFC 4301: "Security Architecture for the Internet Protocol".

[35]	3GPP TS 22.346: "Isolated Evolved Universal Terrestrial Radio Access Network (E-UTRAN) operation for public safety; Stage 1".

[36]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[37]	3GPP TS.33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[38]	IETF RFC 7296: " Internet Key Exchange Protocol Version 2 (IKEv2)".

[39]	IEEE 802.11, Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications, IEEE Std.".

[40]	3GPP TS 36.463: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN) and Wireless LAN (WLAN); Xw application protocol (XwAP)".

[41]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE); Security aspects of non-3GPP accesses".

[42]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[43]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[xx]	3GPP TS 23.122: “Architectural requirements”

*************** Next change ****************

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AES	Advanced Encryption Standard

AK	Anonymity Key

AKA	Authentication and Key Agreement

AMF	Authentication Management Field

AN	Access Network

AS	Access Stratum

AUTN	Authentication token

AV	Authentication Vector

ASME	Access Security Management Entity

Cell-ID	Cell Identity as used in TS 36.331 [21]

CK	Cipher Key

CKSN	Cipher Key Sequence Number

C-RNTI	Cell RNTI as used in TS 36.331 [21]

CRL	Certificate Revocation List

DeNB	Donor eNB

DoS	Denial of Service

DSCP	Differentiated Services Code Point

EARFCN-DL	E-UTRA Absolute Radio Frequency Channel Number-Down Link

ECM	EPS Connection Management

EDT	Early Data Transmission

EEA	EPS Encryption Algorithm

EIA	EPS Integrity Algorithm

eKSI	Key Set Identifier in E-UTRAN

EMM	EPS Mobility Management

eNB	Evolved Node-B

EPC	Evolved Packet Core

EPS	Evolved Packet System

EPS-AV	EPS authentication vector

E-UTRAN	Evolved UTRAN

gNB	Next Generation Node-B

GERAN	GSM EDGE Radio Access Network

GUTI	Globally Unique Temporary Identity

HE	Home Environment

HFN	Hyper Frame Number

HO	Hand Over

HSS	Home Subscriber Server

IK	Integrity Key

IKE	Internet Key Exchange

IMEI	International Mobile Station Equipment Identity

IMEISV	International Mobile Station Equipment Identity and Software Version number

IMSI	International Mobile Subscriber Identity

IOPS	Isolated E-UTRAN Operation for Public Safety

IRAT	Inter-Radio Access Technology

ISR	Idle Mode Signaling Reduction

KDF	Key Derivation Function

KSI	Key Set Identifier

LWIP	LTE WLAN RAN Level Integration using IPSec

LSB	Least Significant Bit

LSM	Limited Service Mode

LWA	LTE-WLAN Aggregation

MAC-I	Message Authentication Code for Integrity (terminology of TS36.323 [12])

MACT	Message Authentication Code T used in AES CMAC calculation

MeNB	Master eNB

ME	Mobile Equipment

MME	Mobility Management Entity

MME-RN	MME serving the RN

MS	Mobile Station

MSC	Mobile Switching Center

MSIN	Mobile Station Identification Number

NAS	Non Access Stratum

NAS-MAC	Message Authentication Code for NAS for Integrity (called MAC in TS24.301 [9])

NASDVM	Non Access Stratum - Data via MME

NCC	Next hop Chaining Counter

NH	Next Hop

OCSP	Online Certificate Status Protocol

OTA	Over-The-Air (update of UICCs)

PCI	Physical Cell Identity as used in TS 36.331 [21]

PDCP	Packet Data Convergence Protocol

PLMN	Public Land Mobile Network

PRNG	Pseudo Random Number Generator

PSK	Pre-shared Key

P-TMSI	Packet- Temporary Mobile Subscriber Identity

RAND	RANDom number

RAU	Routing Area Update

RLOS	Restricted Local Operator Services

RN	Relay Node

RRC	Radio Resource Control

SCG	Secondary Cell Group

SEG	Security Gateway

SGSN	Serving GPRS Support Node

SIM	Subscriber Identity Module

SMC	Security Mode Command

SeNB	Secondary eNB

SgNB	Secondary gNB

SN	Serving Network

SN id	Serving Network identity

SQN	Sequence Number

SRB	Source Route Bridge

SRVCC	Single Radio Voice Call Continuity

S-TMSI	S-Temporary Mobile Subscriber Identity

TAI	Tracking Area Identity

TAU	Tracking Area Update

UE	User Equipment

UEA	UMTS Encryption Algorithm

UIA	UMTS Integrity Algorithm

UICC	Universal Integrated Circuit Card

UMTS	Universal Mobile Telecommunication System

UP	User Plane

USIM	Universal Subscriber Identity Module

UTRAN	Universal Terrestrial Radio Access Network

WT	WLAN Termination as used in TS 36.300 [30]

XRES	Expected Response

*************** Next change ****************

Annex X (normative):
Restricted Local Operator Services

# X.1 	Restricted Local Operator Services

### X.1.1	General

Restricted Local Operator Services (RLOS) is an optional feature supported in certain countries and is specified in TS 23.401 [2]. RLOS is always initiated by the UE based on explicit request from the user. Access to RLOS may be allowed for UEs in limited service state by the serving network depending on local regulation and operator policy. Allowing access to RLOS for UEs in limited service state means that there is no network authentication and in such a case further security checks need to be performed by the UE to address the potential security threats due to this lack of network authentication (i.e., EPS AKA).

This Annex is not applicable for UEs accessing RLOS after performing successful EPS AKA authentication as they follow the security procedures specified in the main body of this specification.

### X.1.2	Algorithm negotiation for unauthenticated UEs in LSM

UEs that are in limited service mode (LSM) and that cannot be authenticated by the MME (for whatever reason) may still be allowed to establish RLOS calls by sending the RLOS attach request message as specified in TS 23.401 [2].

It shall be possible to configure the MME to allow unauthenticated UEs in LSM to establish bearers for RLOS calls or not. If an MME is configured to allow unauthenticated UEs in LSM to establish bearers for an RLOS call, the MME shall for the NAS protocol use EIA0 and EEA0 as the integrity and ciphering algorithm respectively.

If the MME allows an unauthenticated UE in LSM to establish bearers for RLOS calls after it has received the RLOS attach request message from the UE, the MME shall:

-	Select EIA0 and EEA0, regardless of the supported algorithms announced previously by the UE as the NAS algorithms and signal this to the UE via the NAS security mode command procedure when activating the EPS NAS security context.

-	Set the UE EPS security capabilities to only contain EIA0 and EEA0 when sending these to the eNB in the following messages:

-	S1 UE INITIAL CONTEXT SETUP

-	S1 UE CONTEXT MODIFICATION REQUEST

-	S1 HANDOVER REQUEST

NOTE 1:	The result of the MME only sending a UE EPS security capability containing EIA0 and EEA0 to the eNB is that the eNB is only capable of selecting EIA0 for AS integrity protection and EEA0 for AS confidentiality protection. That is, if EIA0 is used for NAS integrity protection, then EIA0 will always be used for AS integrity protection.

If the UE has initiated an RLOS connection, the UE may accept the use of EIA0 for NAS and RRC signalling protection. The UE shall also support the mitigations given in clause X.1.3 to reduce the impact of a lack of network authentication for RLOS connections.

### X.1.3	Additional UE behaviour for RLOS connections

If the UE is in LSM and wishes to initiate an RLOS connection, the ME shall perform the following checks before initiating the RLOS attach procedure with the network:

- The ME shall enforce access control on applications that are authorized to trigger establishment of RLOS connection. Applications on the ME that are not explicitly authorized shall not be allowed to trigger the initiation of RLOS connection.

- A user confirmation shall be requested before the ME initiates RLOS connection. As part of the user confirmation, the user shall be notified of the security risk due to the lack of network authentication.

- The ME shall maintain a white list of MCCs where RLOS is supported (i.e., by preconfiguring the white list either at the time of ME manufacturing or hardcoding). The ME shall check that the MCC of the network name that advertises RLOS service is present in the white list before initiating the RLOS connection.

- If a USIM is present, the ME shall check that the MCC part of the IMSI configured in the USIM is present in the white list of MCCs on the ME before initiating the RLOS connection.

If the above checks are successful, the ME shall initiate the RLOS attach procedure. If any one of the above checks fail, then the ME shall not initiate the RLOS attach procedure.

# X.2	Recommendations for RLOS

Allowing RLOS connections to the network for UEs that are in LSM means that there is no network authentication. This means that it is not possible to protect the NAS and AS traffic. It is strongly recommended that RLOS services are protected by implementing application layer security mechanisms. Such application layer security mechanisms are dependent on the RLOS service and are outside the scope of this specification.

# X.3	MCC whitelist for RLOS

The whitelist for RLOS shall be: MCC = 310, MCC = 311, MCC= 312, MCC=313, MCC=314, MCC=315 and MCC = 316.

The version of the whitelist given in the latest available release shall supersede the whitelist defined in the present document.

*************** End of the changes ****************