3GPP TSG SA3 Meeting #97                  	                      S3-194653

# Reno, US, 18 – 22 November 2019	                                          revision of S3-194041

# Source:	Huawei, HiSilicon

# Title:	Solution on secure ID conversion in groupcast communication.

# Document for:	Approval

Agenda Item:	8.13 (FS_eV2X_Sec)

# 1	Decision/action requested

This pCR proposes a new solution for key issue #9 in TR 33.836

# 2	References

[1]	TR 33.836 Study on Security Aspects of 3GPP support for Advanced V2X Services

[2]	TS 23.287 Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services.

# 3	Rationale

This contribution proposes a solution that addresses the security requirements of the key issue #4 and part of key issue #3 and key issue #9.

# 4	Detailed proposal

*** START OF CHANGE ***

## 6.Y	Solution #Y: Solution on securely creating destination Layer-2 ID in groupcast communication

### 6.Y.1	Introduction

This solution addresses the KI #4 "Security of identifier conversion in groupcast communication", and part of KI #3 “Privacy protection for groupcast messages over PC5” and KI #9 “Minimizing the impact of privacy protection mechanism in the application layer communication”.

### 6.Y.2	Solution details

The detailed solution is illustrated in Figure 6.Y.2-1

Figure 6.Y.2-1: Procedure for secure conversion of application layer group ID to destination Layer-2 ID

- Group Setup: Once the group is created, the group management server will send the group ID to the associated UEs and a timer T. It will also send a set of random numbers and a specific sequence in which these random number are to be used.	 It is assumed that the application layer signalling is protected.

1.	ID Conversion: All the UEs use the group ID and the first random number according to the sequence as an input to a hash function to generate the destination Layer-2 ID.

2.	ID Update: When the timer T expires, a new destination Layer-2 ID is calculated using the next random number according to the sequence. The UEs can listen to both old a new destination Layer-2 IDs, to avoid any time synchronization issues, for a certain period of time or until receives a message with the new ID.

The destination Layer-2 ID is updated until the application layer changes the group ID.

The group management server can also send the corresponding materials to generate random numbers rather than sending the random numbers itself.

Editor’s Note 1: Whether the solution is compatible with SA2 architecture is FFS.

Editor’s Note 2: How to handle out of coverage and out of RAND when out of coverage is FFS.

### Editor’s Note 3: Complexity of storing set of random numbers is FFS.

### 6.Y.3	Solution Evaluation

TBD

*** END OF CHANGE ***