3GPP TSG-SA3Meeting #97	S3-194691

# Reno, Nevada, United States, 18th Nov 2019 - 22nd Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0672 | rev | 1 | Current version: | 15.6.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security handling in registration with direct NAS reroute |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon , Apple,Vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  | Date: |  |  | 2019-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clause 4.2.2.23 in 23.502 specifies registration procedure with AMF realloation. In option (A) direct NAS reroute, i.e. the intial AMF directly sending the registration request message along with UE security conetxt to the target AMF, registration failure may occure. If the UE and the initial AMF have securely exchanged NAS message, and after direct NAS reroute the target AMF performs authentication based on local policy, then the UE will reject the authentication request. This is because it is not clear in current 33.501 whether the target AMF will send the authentication request protected or not. If the target AMF sends the authentication request unprotected, the UE, which already has NAS security activated, will reject the authentication request. To solve the prolem, the target AMF shall send the authentication request protected. If the UE and the initial AMF have not securely exchanged NAS message, the target AMF may send the registration request either protected or unprotected. If the UE and the initial AMF have securely exchanged NAS message, and before direct NAS rerotue the initial AMF performs horizontal KAMF derviation, then the target AMF receives derived security context, and hence has inconsistent security context with the UE. In this case, the target AMF shall not perform authentication until the UE and the target AMF have established security context. And after the UE and the target AMF have established security context, the target AMF shall send the authenticaiton request protected, if the taget AMF decides authentication is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If the target AMF has received the indicator of horizontal KAMF derviation, the target AMF shall not perform authentication. If the target AMF has not received the indicator, the target AMF shall send authentiation request protected with the received security conetxt, if the target AMF decides to perform authentication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Registration failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9.6. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************************************************start of change************************************************

### 6.9.6	Security handling in registration with AMF reallocation via direct NAS reroute

In registration with AMF reallocation via direct NAS reroute, the initial AMF shall use its local policy to determine whether to perform horizontal KAMF derivation on current KAMF.

If the target AMF receives the indication of horizontal KAMF derivation (i.e., keyAmfHDerivationInd) from the initial AMF, it shall initiate NAS SMC.  If the target AMF does not receive keyAmfHDerivationInd, the target AMF shall use the received security context and send protected NAS messages including protected authentication request message if the target AMF decides to perform authentication.

**************************************************end of change************************************************