3GPP TSG-SA3 Meeting #98bis-e 	S3-200726

- e-meeting, 14 – 17 April 2020													Revision of S3-20xxxx

- Source:	Huawei, Hisilicon

- Title:	AUSF function in AKMA

- Document for:	Approval

Agenda Item:	2.4

# 1	Decision/action requested

It is proposed to approve this contribution.

# 2	References

NA

# 3	Rationale

This contribution proposes to define the AUSF function in AKMA.

# 4	Detailed proposal

***	BEGIN CHANGES	***

## 4.2	Network elements

Editor’s Note: This clause will describe the functionalities and requirements of each entities in the reference model.

### 4.2.1	AAnF

AAnF enables the AKMA anchor key (KAKMA) derivation for AKMA service. Before invoking AKMA service, UE shall have successfully registered to the 5G core, which results in KAUSF being stored at the AUSF and the UE after a successful 5G primary authentication.

### 4.2.2	AF

AF is defined in TS 23.501 [3] with the additional functions:

- AF with the AKMA service enabling requests for KAF from the AAnF using AKMA key Identifier.

### 4.2.3	NEF

NEF is defined in TS 23.501[3] with the additional functions:

- NEF finds the AAnF.

### 4.2.4	AUSF

AUSF is defined in TS 23.501[3] with the additional functions:

- AUSF Provides the AKMA Anchor Key (KAKMA) to the AAnF.

**	END OF CHANGES	***