3GPP TSG-SA3 Meeting #98e 	S3-200392-r2

- e-meeting, 2 – 6 March 2020													Merger of S3-200131, S3-200256, S3-200392

- Source:	China Mobile, Nokia, Nokia Shanghai Bell, ZTE, Huawei, Hisilicon, Samsung

- Title:	Derivation of KAKMA

- Document for:	Approval

Agenda Item:	3.9

# 1	Decision/action requested

This proposal is aimed to address the ENs in clause 6.1 in TS 33.535 v0.2.0, namely to propose derivation of KAKMA. SA3 is kindly requested to approve this document.

# 2	References

[1]	3GPP TS 33.535, v0.2.0

# 3	Rationale

This document is proposed to address the following Editor’s Notes in clause 6.1 in TS 33.535 v0.2.0.

Editor’s Note: Derivation of KAKMA is FFS.

4	Detailed proposal

# *** START 1st CHANGE ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[X]                     3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

*** END of 1st CHANGE***

*** START of 2nd CHANGE***

## 6.1	Deriving AKMA key during UE registration

There is no separate authentication of the UE to support AKMA functionality. Instead, it reuses the 5G primary authentication procedure executed during the UE Registration to authenticate the UE. A successful 5G primary authentication results in KAUSF being stored at the AUSF and the UE.

Figure 6.1-1 Deriving AKMA root key during UE registration

The UE and the AUSF shall generate the AKMA Anchor Key (KAKMA) and the associated key Identifier from KAUSF as part of the UE Registration procedure.

The key derivation of KAKMA shall be performed using the key derivation function (KDF) specified in TS 33.220 [X]. KAKMA is computed as KAKMA=KDF (KAUSF, "AKMA", SUPI), where the key derivation parameters consist of a static string"AKMA", and SUPI.

The KAKMA key identifier identifies the KAKMA key of the UE from which other AKMA keys are derived.

Editor’s Note: Format and derivation of KAKMA key identifier and its association with UE identifier is FFS. Whether the key identifier is generated during the primary authentication or as needed, is FFS.

Since AKMA keys are based on KAUSF from primary authentication run, the AKMA keys can only be refreshed by running a fresh primary authentication.

*** END of 2nd CHANGE***

*** START of 3rd CHANGE***

Annex A (normative): Key derivation functions

# A.1	KDF interface and input parameter construction

## A.1.1	General

All key derivations for AKMA shall be performed using the key derivation function (KDF) specified in Annex B.2.0 of TS 33.220 [X].

This clause specifies how to construct the input string, S, and the input key, KEY, for each distinct use of the KDF. Note that "KEY" is denoted "Key" in TS 33.220 [X].

## A.1.2	FC value allocations

The FC number space used is controlled by TS 33.220 [X], FC values allocated for the present document are in range of 0x7A – 0x7F.

# A.2	KAKMA derivation function

When deriving a KAKMA from KAUSF, the following parameters shall be used to form the input S to the KDF:

-	FC =  0x7A;

-	P0 = "AKMA";

-	L0 = length of "AKMA";

-	P1 = SUPI;

-	L1 = length of SUPI.

The input key KEY shall be KAUSF.

*** END of 3rd CHANGE***