3GPP TSG-SA WG3 Meeting #99e	S3-201112

# e-meeting, 11 – 15 May 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0823 | rev | - | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on SUCI computation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  | Date: |  |  | 2020-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The ephemeral public key (SharedInfo1 in the profile) is derived according to section 5.1.3 of "SECG SEC 1: Recommended Elliptic Curve Cryptography, Version 2.0, 2009". In the reference, there should be a prefix in ephemeral public key octet string, e.g., "02", "03" and "04" which indicate the point compression type of the curve point. These prefixes are already applied in Profile B. However, for Profile A, the Curve25519 has already defined the point representation and the prefix is not needed. On the other hand, there is no implementation issues on Profile A without the prefix, the test data of which has already specified in clause C.4.3. Therefore, clarification that prefix of Profile A is not needed shall be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification that prefix for the ephemeral public key of profile A is not needed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There would be ambiguity of SUCI computation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.3.4.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

C.3.4.0	General

Unless otherwise stated, the ECIES profiles follow the terminology and processing specified in SECG version 2 [29] and [30]. The profiles shall use "named curves" over prime fields.

For generating successive counter blocks from the initial counter block (ICB) in CTR mode, the profiles shall use the standard incrementing function in section B.1 of NIST Special Publication 800-38A [16] with m = 32 bits. The ICB corresponds to T1 in section 6.5 of [16].

The value of the MAC tag in ECIES, shall be the L most significant octects of the output generated by the HMAC function, where L equals to the maclen.

Profile A shall use its own standardized processing for key generation (section 6 of RFC 7748 [46]) and shared secret calculation (section 5 of RFC 7748 [46]). The Diffie-Hellman primitive X25519 (section 5 of RFC 7748 [46]) takes two random octet strings as input, decodes them as scalar and coordinate, performs multiplication, and encodes the result as an octet string. The shared secret output octet string from X25519 shall be used as the input Z in the ECIES KDF (section 3.6.1 of [29]). As the point compression is not applied for profile A, the prefix rule for compression type defined in [29] section 5.1.3 shall not be used in profile A, i.e., there shall be no prefix for the ephemeral public key of Profile A.

Profile B shall use point compression to save overhead and shall use the Elliptic Curve Cofactor Diffie-Hellman Primitive (section 3.3.2 of [29]) to enable future addition of profiles with cofactor h ≠ 1. For curves with cofactor h = 1 the two primitives (section 3.3.1 and 3.3.2 of [29]) are equal.

The profiles shall not use backwards compatibility mode (therefore are not compatible with version 1 of SECG).

C.3.4.1	Profile A

The ME and SIDF shall implement this profile. The ECIES parameters for this profile shall be the following:

-	EC domain parameters							: Curve25519 [46]

-	EC Diffie-Hellman primitive					: X25519 [46]

-	point compression								: N/A

-	KDF												: ANSI-X9.63-KDF [29]

-	Hash												: SHA-256

-	SharedInfo1										:  (the ephemeral public key octet string – see [29] section 5.1.3)

-	MAC												: HMAC–SHA-256

-	mackeylen										: 32 octets (256 bits)

-	maclen											: 8 octets (64 bits)

-	SharedInfo2										: the empty string

-	ENC												: AES–128 in CTR mode

-	enckeylen											: 16 octets (128 bits)

-	icblen												: 16 octets (128 bits)

-	backwards compatibility mode					: false

* * * End of Changes * * * *