3GPP TSG-SA WG3-LI Meeting #34

- Berlin, Germany, 14-16 July 2009

Title:	External Dynamic Triggering Capability

Document for:	Discussion

Date:	14 July 2009

Source:	Nortel Networks

Abstract:

Discussion on IRI reporting capability to support External Dynamic LI Triggering between Operators.

Reference: SA3LI09_043 Dynamic Triggering

Background: As noted in Figure 1 below (taken from contribution SA3LI09_043 Dynamic Triggering presented in San Francisco) external dynamic triggering between Operators can be based on the ADMFs HI1 Interface, enhanced as necessary.

Discussion: The following IRI reporting capability depicts an IRI reporting capability for such an External Network ‘Handover’ capability based on existing IRI reporting capabilities. The concept:

- the ‘handing over’/‘from’ network sends the IRI to the LEA;

- the LEA populates the ‘to’ lawful intercept identifier field and send to the ‘receiving’/’to’ network;

- the ‘receiving’/’to’ network populates the ‘to’ correlation number and sends back to the LEA.

Alternatively, three IRI messages could be defined to eliminate the ‘C’ conditional fields and make the respective fields ‘M’ Mandatory. Three messages is removes the possibility of incorrectly populating the ‘C’ conditional fields.

Table x.yy: External Network Handover record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| event type | M | ‘External Network Handover’ event type |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| ‘from’ network identifier | M | Identifies the network handing over the communications. |
| ‘to’ network identifier | M | Identifies the network receiving the communications |
| ‘from’ correlation number | C | The ‘old’ correlation number in the handing over network. Provided by only the network handing over the communications. |
| ‘to’ correlation number | C | The ‘new’ correlation number in the receiving network. Provided by only the network receiving the communications. |
| ‘from’ lawful intercept identifier | C | Identifies the lawful authorization in the handing over network. Provided only by the network handing over the communications. |
| ‘to’ lawful intercept identifier | C | Identifies the lawful authorization in the receiving network. Provided by both the LEA to the network receiving the communications and the receiving network to the LEA. |

Note: Given ‘handover’ carries certain connotations, there may be a better term than ‘Handover’ in the name of this IRI reporting capability. The issue is open for suggestions.

Figure 1 – Dynamic LI Triggering Interfaces

Note: Interfaces as used herein encompasses only the application layer operations/signaling defined for LI purposes.